# Permit

Environmental Protection Act 1994

Environmental authority EPPG00972513

This environmental authority is issued by the administering authority under Chapter 5 of the Environmental Protection Act 1994.

Environmental authority number: EPPG00972513

### Environmental authority takes effect on 5 February 2025.

The anniversary date of this environmental authority is 17 December each year. An annual return will be due each year on 01 April.

Environmental authority holder(s)

| Name(s) | Registered address |
| --- | --- |
| ARROW ENERGY PTY LTD | Level 39 111 Eagle Street BRISBANE QLD 4001 |
| ARROW (TIPTON TWO) PTY LTD | Level 39 111 Eagle Street BRISBANE CITY QLD 4000 |
| ARROW (TIPTON) PTY. LTD. | Level 39 111 Eagle Street BRISBANE CITY QLD 4000 |
| ARROW CSG (AUSTRALIA) PTY LTD | Level 39 111 Eagle Street BRISBANE CITY QLD 4000 |
| ARROW (DAANDINE) PTY. LTD. | Level 39 111 Eagle St BRISBANE CITY QLD 4000 |

Environmentally relevant activity and location details

| Environmentally relevant activity/activities | Location(s) |
| --- | --- |
| Schedule 3 - 06 - A petroleum activity carried out on a site containing a high hazard dam or a significant hazard dam | PL238, PL258, PL252, PL198, PL230, PL260 |
| Schedule 3 - 07 - A petroleum activity involving injection of a wastefluid into a natural underground reservoir or aquifer | PL238, PL258, PL252, PL198, PL230, PL260 |

Page 1 of 71	ABN 46 640 294 485

| Environmentally relevant activity/activities | Location(s) |
| --- | --- |
| Schedule 3 - 08 - A petroleum or GHG storage activity, other than items 1 to 7, that includes an activity from Schedule 2 with an AES | PL238, PL258, PL252, PL198, PL230, PL260 |
| Ancillary 14 - Electricity generation - 1 - Generating electricity by using gas at a rated capacity of 10MW electrical or more | PL238, PL258, PL252, PL198, PL230, PL260 |
| Ancillary 15 - Fuel burning - Using fuel burning equipment that is capable of burning at least 500kg of fuel in an hour | PL238, PL258, PL252, PL198, PL230, PL260 |
| Ancillary 56 - Regulated Waste Storage - Receiving and storing regulated waste | PL230 |
| Ancillary 63 - Sewage Treatment - 1(a-i) - Operating sewage treatment works, other than no-release works, with a total daily peak design capacity of 21 to 100EP - if treated effluent is discharged from the works to an infiltration trench or through an irrigation scheme | PL238, PL258, PL252, PL198, PL230, PL260 |
| Ancillary 63 - Sewage Treatment - 1(b-i) - Operating sewage treatment works, other than no-release works, with a total daily peak design capacity of more than 100 but not more than 1500EP if treated effluent is discharged from the works to an infiltration trench or through an irrigation scheme | PL238, PL258, PL252, PL198, PL230, PL260 |
| Ancillary 64 - Water treatment - 2(b) - Desalinating, in a day, the following quantity of water, allowing the release of waste to waters other than seawater - more than 5ML | PL238, PL258, PL252, PL198, PL230, PL260 |

### Additional information for applicants

Environmentally relevant activities

The description of any environmentally relevant activity (ERA) for which an environmental authority (EA) is issued is a restatement of the ERA as defined by legislation at the time the EA is issued. Where there is any inconsistency between that description of an ERA and the conditions stated by an EA as to the scale, intensity or manner of carrying out an ERA, the conditions prevail to the extent of the inconsistency.

An EA authorises the carrying out of an ERA and does not authorise any environmental harm unless a condition stated by the EA specifically authorises environmental harm.

A person carrying out an ERA must also be a registered suitable operator under the Environmental Protection Act 1994 (EP Act).

Contaminated land

It is a requirement of the EP Act that an owner or occupier of land give written notice to the administering authority if they become aware of the following:

- the presence of, or happening of an event involving, a hazardous contaminant on the land that is causing, or is reasonably likely to cause, serious or material environmental harm (notice must be given within 24 hours); or

- if the land is contaminated land – a change in the condition of the land that is causing, or is reasonably likely to cause, serious or material environmental harm (notice must be given within 24 hours); or

- a notifiable activity (as defined in Schedule 3) having been carried out, or is being carried out, on the land (notice must be given within 20 business days).

For further information, including the form for giving written notice, refer to the Queensland Government website www.qld.gov.au, using the search term ‘duty to notify’.

Take effect

Please note that, in accordance with section 200 of the EP Act, an EA has effect:

- if the authority is for a prescribed ERA and it states that it takes effect on the day nominated by the holder of the authority in a written notice given to the administering authority – on the nominated day; or

- if the authority states a day or an event for it to take effect – on the stated day or when the stated event happens; or

- otherwise – on the day the authority is issued.

However, if the EA is authorising an activity that requires an additional authorisation (a relevant tenure for a resource activity, a development permit under the Planning Act 2016 or an SDA Approval under the State Development and Public Works Organisation Act 1971), this EA will not take effect until the additional authorisation has taken effect.

If this EA takes effect when the additional authorisation takes effect, you must provide the administering authority written notice within 5 business days of receiving notification of the related additional authorisation taking effect.

The anniversary day of this environmental authority is the same day each year as the effective date. The payment of the annual fee will be due each year on this day. An annual return will be due each year on 01 April.

If you have incorrectly claimed that an additional authorisation is not required, carrying out the ERA without the additional authorisation is not legal and could result in your prosecution for providing false or misleading information or operating without a valid environmental authority.

|  |  | 5 February 2025 |
| --- | --- | --- |
| Signature |  | Date |
| Helena Braye Department of Environment, Tourism, Science and Innovation Delegate of the administering authority Environmental Protection Act 1994 |  | Enquiries: Energy and Extractive Resources Business Centre GPO Box 2454, BRISBANE QLD 4001 Phone: (07) 3330 5715 Email: EnergyandExtractive@des.qld.gov.au |

Obligations under the Environmental Protection Act 1994

In addition to the requirements found in the conditions of this environmental authority, the holder must also meet their obligations under the EP Act, and the regulations made under the EP Act. For example, the holder must comply with the following provisions of the Act:

- general environmental duty (section 319)

- duty to notify environmental harm (section 320-320G)

- offence of causing serious or material environmental harm (sections 437-439)

- offence of causing environmental nuisance (section 440)

- offence of depositing prescribed water contaminants in waters and related matters (section 440ZG)

- offence to place contaminant where environmental harm or nuisance may be caused (section 443)

### Other permits required

This permit only provides an approval under the Environmental Protection Act 1994. In order to lawfully operate you may also require permits / approvals from your local government authority, other business units within the department and other State Government agencies prior to commencing any activity at the site. For example, this may include permits / approvals with your local Council (for planning approval), the Department of Transport and Main Roads (to access State controlled roads), the Department of Resources (to clear vegetation), and the Department of Agriculture and Fisheries (to clear marine plants or to obtain a quarry material allocation).

### Conditions of environmental authority

This environmental authority incorporates the following schedules: Schedule A – General

Schedule B – Water

Schedule BE – Coal Seam Gas Water Injection Trial Schedule C – Regulated Dams

Schedule D – Land Schedule E – Acoustic Schedule F – Air Schedule G – Waste

Schedule H – Rehabilitation Schedule I – Definitions

Words and phrases which are underlined are defined in Schedule I – Definitions.

### Schedule A – General

General 1	This environmental authority authorises the carrying out of the following resource activities:

- The petroleum activities listed in Schedule A, Table 1 – Authorised Petroleum Activities to the extent they are carried out in accordance with the activity’s corresponding total number and maximum disturbance;

- The following specified relevant activities:

  - Regulated waste storage – receiving and storing regulated waste other than tyres;

  - Sewage treatment – operating sewage treatment works, other than no-release works, with a total daily peak design capacity of 21 to 100 EP;

- For the specified relevant activities listed in (General 1 (b)) above, another activity where Schedule 2 of the Environmental Protection Regulation 2019 (the Regulation) provides exemption for the activity, but only to the extent of the circumstances stated in Schedule 2 of the Regulation; and

- Incidental activities that are not otherwise specified relevant activities.

General 2	This environmental authority does not authorise environmental harm unless a condition contained in this environmental authority explicitly authorises that harm. Where there is no condition, the lack of a condition shall not be construed as authorising harm.

General 7	All monitoring must be undertaken by a suitably qualified person.

General 8	If requested by the administering authority in relation to investigating a complaint, monitoring must be commenced within 10 business days.

General 9	All laboratory analyses and tests must be undertaken by a laboratory that has NATA accreditation for such analyses and tests.

General 10	Notwithstanding condition (General 9), where there are no NATA accredited laboratories for a specific analyte or substance, then duplicate samples must be sent to at least two separate laboratories for independent testing or evaluation.

Schedule A, Table 1 - Authorised Petroleum Activities

| Petroleum Activity(ies) | Total Number of Authorised Petroleum Activities | Maximum Disturbance Authorised |
| --- | --- | --- |
| Total coal seam gas wells, including: Core wells Exploration wells Development wells Production wells | 1,324 wells | 1,324 ha |
| Communication towers | 7 | 7 units, 7 ha |
| Injection well(s) and associated facilities | 1 | 4 ha |
| Compressor units | 40 | 40 units, 23 ha |
| Central gas processing facilities | 3 | 3 facilities, 23 ha |
| Regulated structures | 22 | 22 dams |
| Water treatment facilities | 2 | 12 ML/d (each), 2 ha |
| Sewage treatment plants | 2 | Less than 450 EP (each), 0.15 ha |
| Sewage treatment plants | 9 | Less than 100 EP (each), 0.15 ha |
| Power stations | 1 | 40 MW, 1.2 ha |

General 11	Monitoring and sampling must be carried out in accordance with the requirements of the following documents (as relevant to the sampling being undertaken), as amended from time to time:

- for waters and aquatic environments, the Queensland Government’s Monitoring and Sampling Manual 2009 – Environmental Protection (Water) Policy 2009

- for groundwater, Groundwater Sampling and Analysis – A Field Guide (2009:27 GeoCat #6890.1)

- for noise, the Environmental Protection Regulation 2019

- for air, the Queensland Air Quality Sampling Manual and/or Australian Standard 4323.1:1995 Stationary source emissions method 1: Selection of sampling positions, as appropriate for the relevant measurement

- for soil, the Guidelines for Surveying Soil and Land Resources, 2nd edition (McKenzie et al. 2008), and/or the Australian Soil and Land Survey Handbook, 3rd edition (National Committee on Soil and Terrain, 2009)

- for dust, Australian Standard AS3580.

General 12	In addition to the requirements under Chapter 7, Part 1, Division 2 of the Environmental Protection Act 1994, the administering authority must be notified through the Pollution Hotline and in writing, as soon as possible, but within 48 hours of becoming aware of any of the following events:

- any unauthorised significant disturbance to land

- potential or actual loss of structural or hydraulic integrity of a dam

- when the level of the contents of any regulated dam reaches the mandatory reporting level

- when a regulated dam will not have available storage to meet the design storage allowance on 1 November of any year

- potential or actual loss of well integrity

- when the seepage trigger action response procedure required under condition (Water 13(g)) is or should be implemented

- unauthorised releases of any volume of prescribed contaminants to waters

- unauthorised releases of volumes of contaminants, in any mixture, to land greater than:

  - 200 L of hydrocarbons; or

  - 1 000 L of brine; or

  - 5 000 L of untreated coal seam gas water; or

  - 5 000 L of raw sewage; or

  - 10 000 L of treated sewage effluent.

- groundwater monitoring results from a landholder’s active groundwater bore monitored under the stimulation impact monitoring program which is a 10% or greater increase from a previous baseline value for that bore and which renders the water unfit for its intended use

- monitoring results where two out of any five consecutive samples do not comply with the relevant limits in the environmental authority.

General 13	Petroleum activities that cause significant disturbance to land must not be carried out until financial assurance has been given to administering authority as security for compliance with the environmental authority and for any costs or expenses, or likely costs or expenses, mentioned in section 298 of the Environmental Protection Act 1994.

General 16  Petroleum activities involving significant disturbance to land cannot commence until the development of written contingency procedures for emergency environmental incidents which include, but are not necessarily limited to:

- a clear definition of what constitutes an environmental emergency incident or near miss for the petroleum activity.

- consideration of the risks caused by the petroleum activity including the impact of flooding and other natural events on the petroleum activity.

- response procedures to be implemented to prevent or minimise the risks of environmental harm occurring.

- the practices and procedures to be employed to restore the environment or mitigate any environmental harm caused.

- procedures to investigate causes and impacts including impact monitoring programs for releases to waters and/or land.

- training of staff to enable them to effectively respond.

- procedures to notify the administering authority, local government and any potentially impacted landholder.

General 17	All plant and equipment must be maintained and operated in their proper and effective condition.

General 18	The following infrastructure must be signed with a unique reference name or number in such a way that it is clearly observable:

- regulated dams and low consequence dams

- exploration, appraisal and development wells

- water treatment facilities

- brine encapsulation facilities

- landfill cells

- sewage treatment facilities

- specifically authorised discharge points to air and waters

- any chemical storage facility associated with the environmentally relevant activity of chemical storage

- field compressor stations

- central compressor stations

- gas processing facilities; and

- pipeline compressor stations.

General 19	Measures to prevent fauna being harmed from entrapment must be implemented during the construction and operation of well infrastructure, dams and pipeline trenches.

General 20	For activities involving significant disturbance to land, control measures that are commensurate to the site- specific risk of erosion, and risk of sediment release to waters must be implemented to:

- allow stormwater to pass through the site in a controlled manner and at non-erosive flow velocities

- minimise soil erosion resulting from wind, rain, and flowing water

- minimise the duration that disturbed soils are exposed to the erosive forces of wind, rain, and flowing water

- minimise work-related soil erosion and sediment runoff; and

- minimise negative impacts to land or properties adjacent to the activities (including roads).

General 21	Petroleum activities must not cause environmental nuisance at a sensitive place, other than where an alternative arrangement is in place.

General 22	A certification must be prepared by a suitably qualified person within 30 business days of completing every plan, procedure, program and report required to be developed under this environmental authority, which demonstrates that:

- relevant material, including current published guidelines (where available) have been considered in the written document

- the content of the written document is accurate and true; and

- the document meets the requirements of the relevant conditions of the environmental authority.

General 23	All plans, procedures, programs, reports and methodologies required under this environmental authority must be written and implemented.

General 24	All documents required to be developed under this environmental authority must be kept for five years.

General 25	All documents required to be prepared, held or kept under this environmental authority must be provided to the administering authority upon written request within the requested timeframe.

General 26	A record of all complaints must be kept including the date, complainant’s details, source, reason for the complaint, description of investigations and actions undertaken in resolving the complaint.

### Schedule B – Water

Water 1	Contaminants that will or may cause environmental harm must not be directly or indirectly released to any waters, except as authorised by condition (B15).

Water 2	The extraction of groundwater as part of the petroleum activity(ies) from underground aquifers must not directly or indirectly cause environmental harm to a wetland.

Water 3	Petroleum activities must not occur:

- Within 200m of wetland of high ecological significance, other than disturbances identified by Schedule B, Table 1 – Authorised Disturbance Areas (Water);

- in a wetland of high ecological significance, other than that authorised by Schedule D, Table 3 — Significant residual impacts to prescribed environmental matters;

- in or within 200m of a Great Artesian Basin Spring;

- in or within 200m of a subterranean cave GDE.

Schedule B, Table 1 – Authorised Disturbance Areas (Water)

| Tenure | Description of Infrastructure | ESA Type | Area of disturbance (ha) |
| --- | --- | --- | --- |
| PL198 | Access tracks and Right of Ways to Tipton 119 well pad | Within 200 metres of a wetland of high ecological significance | 1.031 |
| PL260 | Access Track and Right of Ways to Longswamp 395 well pad |  | 5 |
|  | Well pad (Longswamp 395) |  | 1 |
|  | Well pad (Longswamp 396) |  | 1 |
| PL238 | Right of Way (SPV_PAD_WP430) |  | 1.1 |

NOPA 1	Despite condition Water 3 the additional disturbance areas prescribed in Schedule B, Table 2 – Prescribed additional disturbance areas (Water) are permitted to be located within the areas shown on shapefiles referenced in Schedule B, Table 2 — Prescribed additional disturbance areas (Water).

Schedule B, Table 2 — Prescribed additional disturbance areas (Water)

| Tenure | Description of Infrastructure | ESA Type | Area of disturbance | Shapefile Reference |
| --- | --- | --- | --- | --- |
| PL260 | Well Pads | Within 200 metres of a wetland of high ecological significance | 5.65 ha | Additional Disturbance Areas June 2024 |
| PL198 | Well Pad | Within 200 metres of a wetland of high ecological significance | 0.14 ha | Additional Disturbance Areas June 2024 |
| PL260 | Access tracks and Right of Ways | Within 200 metres of a wetland of high ecological significance | 13.13 ha | Additional Disturbance Areas January 2025 |
| PL198 | Access tracks and Right of Ways | Within 200 metres of a wetland of high ecological significance | 2.23 ha | Additional Disturbance Areas January 2025 |

Water 4	Only construction or maintenance of linear infrastructure is permitted in or within any wetland of other environmental value or in a watercourse.

Water 5A	The construction or maintenance of linear infrastructure in a wetland of other environmental value must not result in the:

- clearing of riparian vegetation outside of the minimum area practicable to carry out the works; or

- ingress of saline water into freshwater aquifers; or

- draining or filling of the wetland beyond the minimum area practicable to carry out the works.

Water 5B	After the construction or maintenance works for linear infrastructure in a wetland of other environmental value are completed, the linear infrastructure must not:

- drain or fill the wetland

- prohibit the flow of surface water in or out of the wetland

- lower or raise the water table and hydrostatic pressure outside the bounds of natural variability that existed before the activities commenced

- result in ongoing negative impacts to water quality

- result in bank instability; or

- result in fauna ceasing to use adjacent areas for habitat, feeding, roosting or nesting.

Water 6	The construction or maintenance of linear infrastructure activities in a watercourse must be conducted in the following preferential order:

- firstly, in times where there is no water present

- secondly, in times of no flow

- thirdly, in times of flow, providing a bankfull situation is not expected and that flow is maintained.

Water 7	The construction or maintenance of linear infrastructure authorised under condition (Water 4) must comply with the water quality limits as specified in Schedule B, Table 3 – Release limits for construction or maintenance of linear infrastructure.

Schedule B, Table 3 – Release limits for construction or maintenance of linear infrastructure

| Water quality parameters | Units | Water quality limits |
| --- | --- | --- |
| Turbidity | Nephelometric Turbidity Units (NTU) | For a wetland of other environmental value, if background water turbidity is above 45 NTU, no greater than 25% above background water turbidity measured within a 50m radius of the construction or maintenance activity. For a watercourse, if background water turbidity is above 45 NTU, no greater than 25% above background water turbidity measured within 50m downstream of the construction or maintenance activity. |
|  |  | For a wetland of other environmental value, if background water turbidity is equal to, or below 45 NTU, a turbidity limit of no greater than 55 NTU applies, measured within a 50m radius of the construction or maintenance activity. |

| Water quality parameters | Units | Water quality limits |
| --- | --- | --- |
|  |  | For a watercourse, if background water turbidity is equal to, or below 45 NTU, a turbidity limit of no greater than 55 NTU applies, measured within 50m downstream of the construction or maintenance activity. |
| Hydrocarbons | - | For a wetland of other environmental value, or watercourse, no visible sheen or slick |

Water 8	Monitoring must be undertaken at a frequency that is appropriate to demonstrate compliance with condition (Water 7).

Water 9	A register must be kept of all linear infrastructure construction and maintenance activities in a wetland of other environmental value and watercourses, which must include:

- location of the activity (e.g. GPS coordinates (GDA2020) and watercourse name)

- estimated flow rate of surface water at the time of the activity

- duration of works, and

- results of impact monitoring carried out under condition (Water 8).

Water 10	Measures must be taken to minimise negative impacts to, or reversal of, any river improvement works carried out in River Improvement Areas by Queensland’s River Improvement Trusts.

Water 11	Petroleum activity(ies) on floodplains must be carried out in a way that does not:

- concentrate flood flows in a way that will or may cause or threaten a negative environmental impact; or

- divert flood flows from natural drainage paths and alter flow distribution; or

- increase the local duration of floods; or

- increase the risk of detaining flood flows.

Water 12	A seepage monitoring program must be developed by a suitably qualified person which is commensurate with the site-specific risks of contaminant seepage from containment facilities, and which requires and plans for detection of any seepage of contaminants to groundwater as a result of storing contaminants by 1 August 2019.

Water 13	The seepage monitoring program required by condition (Water 12) must include but not necessarily be limited to:

- identification of the containment facilities for which seepage will be monitored

- identification of trigger parameters that are associated with the potential or actual contaminants held in the containment facilities

- identification of trigger concentration levels that are suitable for early detection of contaminant releases at the containment facilities

- installation of background seepage monitoring bores where groundwater quality will not have been affected by the petroleum activities authorised under this environmental authority to use as reference sites for determining impacts

- installation of seepage monitoring bores that:

  - are within formations potentially affected by the containment facilities authorised under this environmental authority (i.e. within the potential area of impact)

  - provide for the early detection of negative impacts prior to reaching groundwater dependent ecosystems, landholder’s active groundwater bores, or water supply bores

  - provide for the early detection of negative impacts prior to reaching migration pathways to other formations (i.e. faults, areas of unconformities known to connect two or more formations)

- monitoring of groundwater at each background and seepage monitoring bore at least annually for the trigger parameters identified in condition (Water 13(b))

- seepage trigger action response procedures for when trigger parameters and trigger levels identified in conditions (Water 13(b)) and (Water 13(c)) trigger the early detection of seepage, or upon becoming aware of any monitoring results that indicate potential groundwater contamination

- a rationale detailing the program conceptualisation including assumptions, determinations, monitoring equipment, sampling methods and data analysis; and

- provides for annual updates to the program for new containment facilities constructed in each annual return period.

Water 14	A bore drill log must be completed for each seepage monitoring bore in condition (Water 13) which must include:

- bore identification reference and geographical coordinate location

- specific construction information including but not limited to depth of bore, depth and length of casing, depth and length of screening and bore sealing details

- standing groundwater level and water quality parameters including physical parameter and results of laboratory analysis for the possible trigger parameters

- lithological data, preferably a stratigraphic interpretation to identify the important features including the identification of any aquifers; and

- target formation of the bore.

B15	The release of treated CSG water is authorised to occur in accordance with:

- Schedule B, Table 4 – Treated CSG Water Release Point, Source and Receiving Waters;

- Schedule B Table 5 – Treated CSG Water Release Limits for Monitoring Point M1 –

Daily Monitoring; and

- Schedule B, Table 6 – Treated CSG Water Release Limits for Monitoring Point M1 – Quarterly Monitoring.

B16	The quality of the treated CSG water being released must be:

- monitored at the frequency specified, and

- comply with each quality characteristic release limit and limit type, specified in Schedule B Table 5 – Treated CSG Water Release Limits for Monitoring Point M1 – Daily Monitoring and Schedule B, Table 6 – Treated CSG Water Release Limits for Monitoring Point M1 – Quarterly Monitoring when measured at the monitoring point M1 specified in Schedule B, Table 4 – Treated CSG Water Release Point, Source and Receiving Waters.

Schedule B, Table 4 – Treated CSG Water Release Point, Source and Receiving Waters

| Water Source and Location | Release Point | Location (GDA94, MGA zone 56) | Monitoring Point | Receiving Waters Description |
| --- | --- | --- | --- | --- |
| Treated CSG Water from the reverse osmosis water treatment plant and treated water dam located on PL230 | R1, defined as the outlet of discharge pipe to Wilkie Creek | 6995465 mN 303004 mE | M1, defined as the sample point in the discharge pipeline point to release into Wilkie Creek | Wilkie Creek |

Schedule B, Table 5 – Treated CSG Water Release Limits for Monitoring Point (M1) – Daily Monitoring

| Quality Characteristic | Unit | Release Limit(s) | Limit Type | Monitoring Frequency |
| --- | --- | --- | --- | --- |
| Electrical conductivity | µS/cm | 580 | 80th percentile, based on at least 5 samples with not less than 60 minutes between samples | Daily during release |
| pH | pH Unit | 6.5 - 9 | Range | Daily during release |
| Suspended Solids | mg/L | 180 | Maximum | Daily during release |
| Boron | mg/L | 0.37 | Maximum | Daily during release |

Schedule B, Table 6 – Treated CSG Water Release Limits for Monitoring Point (M1) – Quarterly Monitoring

| Quality characteristic | Unit | Maximum Release limit | Monitoring frequency |
| --- | --- | --- | --- |
| Aluminium | mg/L | 0.2 | The first release day of each quarter The first release day of each quarter |
| Antimony | ml/L | 0.003 |  |
| Arsenic | mg/L | 0.007 |  |
| Barium | mg/L | 0.7 |  |
| Bisphenol A | mg/L | 0.2 |  |
| Boron | mg/L | 4 |  |
| Bromide | mg/L | 7 |  |
| BTEX |  |  |  |
| Benzene | mg/L | 0.001 |  |
| Toluene | mg/L | 0.8 |  |
| Ethylbenzene | mg/L | 0.3 |  |
| Xylene (m & p) | mg/L | 0.6 |  |
| Cadmium | mg/L | 0.002 |  |
| Chromium | mg/L | 0.05 |  |
| Copper | mg/L | 2 |  |
| Cyanide | mg/L | 0.08 |  |
| Fluoride | mg/L | 1.5 |  |

| Quality characteristic |  | Unit | Maximum Release limit | Monitoring frequency |
| --- | --- | --- | --- | --- |
| Iodide |  | mg/L | 0.5 | The first release day of each quarter The first release day of each quarter |
| Iron |  | mg/L | 0.3 |  |
| Lead |  | mg/L | 0.01 |  |
| Manganese |  | mg/L | 0.5 |  |
| Mercury |  | mg/L | 0.001 |  |
| Molybdenum |  | mg/L | 0.05 |  |
| Nickel |  | mg/L | 0.02 |  |
| Nonylphenol |  | mg/L | 0.5 |  |
| PAH (as B(a)P TEF) |  |  | 0.01 |  |
| Species: | TEF: |  |  |  |
| benz[a]anthracene | 0.1 |  |  |  |
| benzo[b+j]fluoranthene | 0.1 |  |  |  |
| benzo[k]fluoranthene | 0.1 | µg/L |  |  |
| benzo[a]pyrene | 1.0 |  |  |  |
| chrysene | 0.1 |  |  |  |
| dibenz[a,h]anthracene | 1.0 |  |  |  |
| indeno[1,2,3-cd]pyrene | 0.1 |  |  |  |
| Selenium |  | mg/L | 0.01 |  |
| Silver |  | mg/L | 0.1 |  |
| Strontium |  | mg/L | 4 |  |
| Total Petroleum Hydrocarbons (TPH) |  | mg/L | 0.2 |  |
| Vanadium |  | mg/L | 0.05 |  |
| Zinc |  | mg/L | 3 |  |
| Radium-226 |  | mSv/year | 0.5 |  |
| Lead-210 |  |  | The dose for each |  |
| Polonium-210 |  |  | parameter is |  |
| Radium-228 |  |  | summed to give the |  |
|  |  |  | total dose which |  |
|  |  |  | must be less than or |  |
|  |  |  | equal to 0.5 |  |
|  |  |  | mSv/year |  |

B17	If the monitoring required by condition (B16) indicates that any of the quality characteristic release limits specified in Schedule B, Table 6 – Treated CSG Water Release Limits for Monitoring Point M1 – Quarterly Monitoring have been exceeded at any time during any release of treated CSG water, the environmental authority holder must, within five (5) business days of notifying the administering authority of the exceedance, unless a longer time is agreed to by the administering authority:

- complete an investigation into the exceedance, which includes an analysis of the (potential and actual) causes for the exceedance; and

- provide a written report to the administering authority on completion of the investigation that includes:

  - details of the investigation carried out;

  - any actions taken to prevent impacts to waters that may be used for drinking water;

  - the cause for the exceedance;

  - all water quality monitoring results pertaining to the investigation;

  - any general observations;

  - methodology(ies) and any relevant calculations used; and

  - corrective actions to rectify the cause of the exceedance.

B18	Where an exceedance of a quality characteristic release limit specified in Schedule B, Table 6 – Treated CSG Water Release Limits for Monitoring Point M1 – Quarterly Monitoring is being investigated in accordance with condition (B17), the investigation and reporting required by condition (B17) is not required if a subsequent exceedance occurs, during investigation of the initial exceedance, which has resulted from the same cause that triggered the initial exceedance.

B19	Prior to commencing any release of treated CSG water authorised under this environmental authority, a stream flow gauging station (GP1) must be installed.

B20	The stream flow gauging station (GP1) must be installed in accordance with Schedule B, Table 7 – Contaminant Release During Flow Events.

Schedule B, Table 7 – Contaminant Release During Flow Events

| Receiving water description | Gauging station description | Latitude or northing (GDA94) | Longitude or easting (GDA94) | Minimum Flow in Receiving Water Required for a Release Event | Flow recording Frequency |
| --- | --- | --- | --- | --- | --- |
| Wilkie Creek | Gauging station 1 (GP1 ) | 50-100 metres upstream of Release Point R1 | 50-100 metres upstream of Release Point R1 | 0.14 m3/s | At 6 hour intervals during discharge (minimum twice daily) |

B21	The flow rate of treated CSG water released from the release point authorised in Schedule B, Table 4 – Treated CSG Water Release Point, Source and Receiving Waters must not exceed a ratio of 1 part of the flow rate of the treated CSG water being released to 4 parts of the measured upstream receiving water flow rate.

B22	The volume of treated CSG water released from the release point authorised in Schedule B, Table 4 – Treated CSG Water Release Point, Source and Receiving Waters must not exceed a maximum of 8ML/day.

B23	The following characteristics of the treated CSG water released must be measured and recorded daily during all release events:

- the volume of treated CSG water released through the release point R1;

- the date and time of release commencing and ceasing;

- the release rate;

- for any change in the release rate:

  - the date and time of the change;

  - the new release rate; and

  - water levels and flow rate during the discharge event.

- water levels and flow rate during the discharge event;

- water quality characteristics monitoring results; and

- details of any observed impacts/conditions.

B24	Prior to commencing any release of treated CSG water authorised under this environmental authority, each monitoring and release point specified in Schedule B, Table 4 – Treated CSG Water Release Point, Source and Receiving Waters must be marked and readily identifiable from the banks of Wilkie Creek.

B25	The quality of the receiving waters must be monitored daily during release of treated CSG water at locations representative of the receiving waters determined in accordance with condition (B31), for the following water quality characteristics:

- Electrical conductivity (µS/cm);

- pH (pH Unit);

- Turbidity (NTU);

- Suspended Solids (mg/L);

- Calcium (mg/L);

- Magnesium (mg/L);

- Fluoride (mg/L);

- Sulphate (mg/I); and

- Boron (mg/L).

B26	If water has been released from authorised release points listed in Schedule B, Table 4 – Contaminated Release Points, Sources and Receiving Waters, the holder of this environmental authority must undertake an annual assessment of the contaminants of treated CSG water to determine the risk of environmental harm from release of treated CSG water to surface waters. This should consider the contaminants mentioned in the ANZECC & ARMCANZ 2000 guidelines. This annual assessment must be included in the Annual Return.

B27	Prior to the release of treated CSG water to Wilkie Creek a Receiving Environment Monitoring Program (REMP) must be developed and implemented to monitor, identify and describe any adverse impacts to surface water environmental values, water quality and flows due to the authorised release of treated CSG water to Wilkie Creek.

B28	The REMP required by condition (B27) must include periodic monitoring for the effects of the release on the receiving environment as a result of treated CSG water releases to waters from the release location (R1) specified in Table B15 - Treated CSG Water Release Point, Source and Receiving Waters.

B29	The quality of the receiving waters must be monitored at the locations specified in Schedule B, Table 8 – Receiving Water Upstream Background Sties and Downstream Monitoring Points.

Schedule B, Table 8 – Receiving Water Upstream Background Sites and Downstream Monitoring Points

| Monitoring Points | Receiving Waters | Northing (GDA94) | Easting (GDA94) |
| --- | --- | --- | --- |
| Upstream background monitoring point |  |  |  |
| Monitoring Point (M2) | Wilkie Creek | 50 – 100 metres upstream of Release Point R1 | 50 – 100 metres upstream of Release Point R1 |
| Downstream monitoring point |  |  |  |
| Monitoring Point (M3) | Wilkie Creek | 150 – 200 metres downstream of Release Point R1 | 150 - 200 metres downstream of Release Point R1 |

B30	The REMP required by Condition (B27) must:

- assess the condition or state of receiving waters, including upstream conditions, spatially within the REMP area, considering background water quality characteristics based on accurate and reliable monitoring data that takes into consideration temporal variation (e.g. seasonality);

- be designed to facilitate assessment against water quality objectives for the relevant environmental values that need to be protected;

- include monitoring from background reference sites (e.g. upstream or background) and downstream sites from the release;

- specify the frequency and timing of sampling required in order to reliably assess ambient conditions and to provide sufficient data to derive site specific background reference values in accordance with the Queensland Water Quality Guidelines 2006. This should include monitoring during periods of natural flow irrespective of other discharges;

- include monitoring and assessment of dissolved oxygen saturation and temperature and all water quality parameters listed in Condition (B25);

- include, where appropriate, monitoring of metals/metalloids in sediments (in accordance with ANZECC & ARMCANZ 2000, BATLEY and/or the most recent version of AS5667.1 Guidance on Sampling of Bottom Sediments);

- include, where appropriate, monitoring of macroinvertebrates in accordance with the AusRivas methodology;

- apply procedures and/or guidelines from ANZECC and ARMCANZ 2000 and other relevant guideline documents;

- describe sampling and analysis methods and quality assurance and control; and

- incorporate stream flow and hydrological information in the interpretations of water quality and biological data.

B31	A report outlining the findings of the REMP, including all monitoring results and interpretations in accordance with conditions (B27) to (B30) must be prepared annually. This must include a determination of the risk of environmental harm from release of treated CSG water to the receiving environment waters.

# Schedule BE – Coal Seam Gas Water Injection Trial

BE1	The only fluids authorised to be injected into an aquifer(s) are those fluid types specified in

Schedule BE, Table 1 – Details of Authorised Fluid Injection.

## Schedule BE, Table 1 – Details of Authorised Fluid Injection

| Well Location (GDA94, MGA zone 56) | Injection Well Number / Reference | Target Formation and Perforated Zone | Fluid Type | Maximum Injection Rate ML/d | Hydraulic Impact Zone | Water Quality Impact Zone |
| --- | --- | --- | --- | --- | --- | --- |
| 313669 mE 6967949 mN | Tipton-193 | Precipice Formation, between 1040m to 1110m depth | Treated CSG water1 | 4 | 9,340 m radius from injection well | 148 m radius from injection well |

1 Treated CSG water means any fluid that contains any proportion of treated CSG water, blended or otherwise.

BE2	Fluid injection must be in accordance with the quantities and locations listed in Schedule BE, Table 1 – Details of Authorised Fluid Injection.

BE3	The holder of this environmental authority must notify the administering authority the commencement date of the trial at least seven (7) days prior to its commencement.

BE4	The injection trial is limited to a period of 180 days of injection, followed by a six-month recovery period.

BE5	Injection must cease immediately upon becoming aware that environmental harm is caused or threatened to be caused as a result of the injection activities.

BE6	The construction of the Tipton-193 injection well must be carried out in accordance with the well construction requirements described in the most recent version of the Department of Environment and Heritage Protection’s “CSG Water Injection Well Construction Requirements”, as amended from time to time.

BE7	The Tipton-193 injection well must be mechanically functional such that there is no significant fluid movement into a water resource aquifer through vertical channels adjacent to the well bore hole.

BE8	The construction, operation and maintenance of the injection well specified in Schedule BE, Table 1 – Details of Authorised Fluid Injection must be undertaken in a way that prevents and/or minimises impacts to the environmental values of the target formation and ensures the integrity of the bore.

BE9	The injection pressure must not exceed 90 percent of the formation fracture pressure.

BE10	The quality of the fluid being injected into the injection well specified in Schedule BE, Table 1 – Details of Authorised Fluid Injection must comply with the contaminant limits prescribed in Schedule BE, Table 2 – Specific Contaminant Limits for Injection Fluid.

Schedule BE, Table 2 – Specific Contaminant Limits for Injection Fluid

| Quality Characteristics | Release Limits | Limit Type |
| --- | --- | --- |
| pH (pH units) | Minimum 6.5, Maximum 10 | Range |
| Total Dissolved Solids (TDS)1 | Mean TDS of injection fluid must not exceed median TDS of formation water measured in background water quality monitoring well | Maximum |
| Dissolved Oxygen | 500 µg/L | Maximum |
| Total suspended solids | 25 mg/L | Maximum |

1 Electrical conductivity is an acceptable proxy measurement for measuring TDS of the injection fluid. If used, the conversion from EC to TDS must be stated and confirmed with laboratory monitoring results.

BE11 A	The quality of the fluid being injected into the injection well specified in Schedule BE, Table 1 – Details of Authorised Fluid Injection must be monitored at the frequency specified in Schedule BE, Table 3 – Monitoring Parameters and Frequencies for Injection Fluid.

BE11 B	The administering authority must be notified in writing as soon as reasonably practicable, but within 48 hours of becoming aware of:

- migration of injected fluid out of the target formation; or

- a loss of hydraulic isolation of the target formation; or

- the detection of groundwater contaminants that were not detected in background samples; or

- an injection fluid monitoring result that does not comply with any one of the parameters in Schedule BE, Table 2 – Specific Contaminant Limits for Injection Fluid.

Schedule BE, Table 3 – Monitoring Parameters and Frequencies for Injection Fluid

| Monitoring parameter | Unit | Monitoring frequency during injection |
| --- | --- | --- |
| Temperature of target formation | °C | N/A |
| Injection fluid temperature | °C | Continuous |
| Inlet pressure | KPa | Continuous |
| Fluid flow rate | L/sec | Continuous |
|  | ML/day | Daily |
| Dissolved Oxygen (DO) | µg/L | Daily |
| Electrical Conductivity | µS/cm | Daily |
| pH | pH units | Daily |
| Turbidity | NTU | Daily |

| Monitoring parameter | Unit | Monitoring frequency during injection |
| --- | --- | --- |
| Sodium | mg/L | Weekly |
| Potassium | mg/L | Weekly |
| Calcium | mg/L | Weekly |
| Magnesium | mg/L | Weekly |
| Chloride | mg/L | Weekly |
| Sulphate as SO4 | mg/L | Weekly |
| Carbonate / Bicarbonate | mg/L | Weekly |
| Bromine | mg/L | Monthly |
| Fluoride | mg/L | Monthly |
| Iodide | mg/L | Weekly |
| Silica | mg/L | Monthly |
| Iodine | mg/L | Monthly |
| Nitrate | mg/L | Monthly |
| Total organic carbon | mg/L | Weekly |
| Total Dissolved Solids | mg/L | Weekly |
| Redox Potential | mV | Weekly |
| Total Hardness as CaCO3 | mg/L | Weekly |
| Total Alkalinity as CaCO3 | mg/L | Weekly |
| Hydroxide alkalinity as CaCO3 | mg/L | Weekly |
| Carbonate Alkalinity as CaCO3 | mg/L | Weekly |
| Bicarbonate Alkalinity as CaCO3 | mg/L | Weekly |
| Total suspended solids | mg/L | Weekly |
| Bromide | mg/L | Weekly |
| Aluminium | mg/L | Monthly |
| Arsenic | mg/L | Monthly |
| Barium | mg/L | Monthly |
| Boron | mg/L | Monthly |
| Copper | mg/L | Monthly |
| Iron | mg/L | Monthly |
| Manganese | mg/L | Monthly |
| Mercury | mg/L | Monthly |

| Monitoring parameter | Unit | Monitoring frequency during injection |
| --- | --- | --- |
| Nickel | mg/L | Monthly |
| Strontium | mg/L | Monthly |
| Zinc | mg/L | Monthly |
| Total Petroleum Hydrocarbons | µg/L | Monthly |
| BTEX | µg/L | Monthly |

BE12	Notwithstanding conditions (BE9) and (BE10), the injection of treated CSG water into the injection well specified in Schedule BE, Table 1 – Details of Authorised Fluid Injection must have considered the recommendations for water blending described in the following documents, where appropriate:

- Surat Gas Project – Geochemical Blending and Compatibility Study: Tipton Injection Trials, March 2013; and

- Surat Gas Project – Geochemical Blending and Compatibility Study: Tipton Injection Trials – Phase 2, March 2013.

BE13	The holder of this environmental authority must provide for non-chemical disinfection of injection fluid if results of six-monthly testing of injected fluid show levels of coliform bacteria, sulphate reducing bacteria or iron fixing bacteria that has potential to cause adverse impacts on the groundwater within the target formation.

BE14	An Injection Management Plan which has been certified by a suitably qualified person must be developed and implemented prior to the carrying out of the trial fluid injection activity(ies).

BE15	The Injection Management Plan must include but not necessarily be limited to:

- estimated volumes and rates of water to be injected;

- a description of the physical, chemical and biological components and their concentrations of the water to be produced;

- details of how and where the fluid will be produced, aggregated, stored and kept separate from other waters until it is treated and injected;

- details of where the fluid is proposed to be treated including a description of the treatment process;

- a water quality compatibility assessment which demonstrates that the injection fluid has inconsequential reactivity with the target formation and native groundwater it will come into contact with;

- the regional characteristics of the receiving environment;

- identification of the water quality impact zone and the hydraulic impact zone;

- identification of any fluid injection well, all existing bores, springs, lakes, wetlands, environmental assets and watercourses connected to groundwater, faults and other

geologic features that occur within the water quality impact zone and the hydraulic impact zone;

- identification of the environmental values and water quality objectives of the potential water quality impact zone of the target formation in accordance with the Environmental Protection (Water and Wetland) Biodiversity Policy 2019 and the Queensland Water Quality Guidelines 2009;

- an assessment of the potential for migration of injection fluid or native groundwater out of the target formation through wells, bores, springs, connected watercourses, faults or other geologic features likely to impact on other aquifers;

- a risk assessment identifying potential hazards, their inherent risk, preventative measures for the management of potential hazards and details on sampling and analysis methods to verify preventative measures of potential hazards, including frequency and locations and quality assurance and control;

- control measures that will be implemented for fluid storage, treatment and injection to prevent or control the release of a contaminant or waste to the environment other than the release authorised in Schedule BE, Table 1 – Details of Authorised Fluid Injection and Schedule BE, Table 2 – Specific Contaminant Limits for Injection Fluid;

- verification methods to assess performance of the injection activities;

- the indicators or other criteria against which the performance of fluid injection will be assessed;

- procedures that will be adopted to regularly review the monitoring program;

- reporting procedures to management and the administering authority should unforeseen or non-compliant monitoring results be recorded; and

- procedures that will be implemented to prevent unauthorised environmental harm from unforeseen or non-compliant monitoring results.

BE16	A Receiving Environment Monitoring Program (REMP) for Injection Activities which has been certified by a suitably qualified person must be developed and implemented to monitor, identify and describe any adverse impacts to the following values of the target formation due to the injection of treated CSG water:

  - hydraulic response;

  - water quality response; and

  - any other groundwater environmental values identified.

BE17	The REMP for Injection Activities required by condition (BE16) must be developed and implemented prior to the carrying out of the trial fluid injection activity(ies).

BE18	The REMP for Injection Activities required by condition (BE16) must include, but not necessarily be limited to:

- methods to validate the assumptions, predicted impacts and the effectiveness of the proposed preventative measures associated with the modelled water quality impact zone and hydraulic impact zone;

- monitoring of the hydraulic response to the trial injection at the well(s) referred to in condition (BE18)(f)(ii);

- monitoring of the water quality of the injection fluid during injection for the parameters listed in Schedule BE, Table 3 – Monitoring Parameters and Frequencies for Injection Fluid;

- monitoring of the water quality response following the completion of the injection trial recovery period through sampling of the target aquifer, within the water quality impact zone, for the parameters listed in Schedule BE, Table 3 – Monitoring Parameters and Frequencies for Injection Fluid;

- a definition of the background water quality of the Precipice Sandstone to be used for monitoring compliance with Schedule BE, Table 2 – Specific Contaminant Limits for Injection Fluid and for reporting in the Injection Trial Report required by condition (BE19);

- the installation and use of a minimum of two wells:

  - one of which accesses the target aquifer within the water quality impact zone; and

  - the other of which is placed at an adequate distance to determine the extent of the hydraulic response to the injection;

- methods for the analysis and interpretation and a description of the statistical basis on which conclusions will be drawn to verify the assumptions of the injection proposal; and

- a demonstration of how the REMP will validate assumptions, predicted impacts and the effectiveness of quality assurance and control measures.

BE19	Upon completion and within two (2) months of the completion of the injection trial’s recovery period the holder of the authority must submit to the administering authority an Injection Trial Report prepared by a suitably qualified person.

BE20	The Injection Trial Report must include, but not necessarily be limited to:

- details of the injection well including but not limited to:

  - location details (GDA2020);

  - the inferred lithology *;

  - casing details including type, outer diameter (mm), wall thickness (mm) and locations (depth from and to in metres);

  - cementing details including type, hole diameter (mm), casing outer diameter (mm) and locations (depth from and to in metres);

  - calculated target formation fracture pressure; and

  - target formation pressure prior to injection;

- a detailed interpretation of the logs and other tests conducted during drilling and construction or refurbishment of the well against their specific objectives;

- a completed well schematic diagram;

- a temperature survey;

- a cement integrity log;

- outcomes of the injection trial including, but not limited to:

  - well head injection rates versus formation pressure;

  - target formation pressure within the hydraulic impact zone during and upon completion of the trial;

  - hydraulic gradient of the target formation within the hydraulic impact zone upon completion of the trial;

  - the effectiveness of aquitards (including the stability of the aquitard) and aquicludes of confining the injected fluid within the target formation; and

  - a detailed interpretation of the logs and other tests conducted during the injection trial against their specific objectives;

  - validation of conceptual framework for injection; and

  - additional hazards that were not identified earlier;

- the results of the REMP for Injection Activities;

- analysis of monitoring and operational data in terms of:

  - validation of conceptual framework for injection; and

  - additional hazards that were not identified earlier;

- a revised risk analysis that identifies all potential hazards, likelihood of various risk elements and associated consequences;

- a revised water quality compatibility assessment which demonstrates that the injection fluid has inconsequential reactivity with the target formation and native groundwater it will come into contact with;

- a re-evaluation of the hydraulic impact zone; and

- a re-evaluation of the water quality impact zone.

* Inferred lithology means the best available description of the lithology based upon historical drilling records, interpretation of logs and any other information that the suitably qualified person may have discovered.

BE21	Following completion of the injection trial, a fluid injection cessation report which has been certified by a suitably qualified person must be submitted to the administering authority within two (2) months of completion of the injection trial’s recovery period.

BE22	The fluid injection cessation report must include, but not necessarily be limited to:

- volumes of fluid injected at each well;

- a risk assessment statement providing details on identified hazards including their inherent risk, summary of the results from the verification monitoring, preventative measures and the residual risk; and

- a monitoring report outlining the methods and results of verification monitoring undertaken to assess the performance of the injection activities and preventative measures for identified hazards.

### Schedule C – Regulated Dams

Dams 1	The consequence category of any structure must be assessed by a suitably qualified and experienced person in accordance with the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933) at the following times:

- prior to the design and construction of the structure, if it is not an existing structure; or

- prior to any change in its purpose or the nature of its stored contents.

Dams 2	A consequence assessment report and certification must be prepared for each structure assessed and the report may include a consequence assessment for more than one structure.

Dams 3	Certification must be provided by the suitably qualified and experienced person who undertook the assessment, in the form set out in the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933).

Dams 4	Conditions (Dams 5) to (Dams 9) inclusive do not apply to existing structures.

Dams 5	All regulated structures must be designed by, and constructed under the supervision of, a suitably qualified and experienced person in accordance with the requirements of the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/19338).

Dams 6	Construction of a regulated structure is prohibited unless:

- the holder of this environmental authority has submitted a consequence category assessment report and certification to the administering authority; and

- certification for the design, design plan and the associated operating procedures has been certified by a suitably qualified and experienced person in compliance with the relevant condition of this authority.

Dams 7	Certification must be provided by the suitably qualified and experienced person who oversees the preparation of the design plan in the form set out in the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/19338), and must be recorded in the Register of Regulated Structures.

Dams 8	Regulated structures must:

- be designed and constructed in compliance with the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/19338);

- be designed and constructed with due consideration given to ensuring that the design integrity would not be compromised on account of:

  - floodwaters from entering the regulated dam from any watercourse or drainage line; and

  - wall failure due to erosion by floodwaters arising from any watercourse or drainage line.

- have the floor and sides of the dam designed and constructed to prevent or minimise the passage of the wetting front and any entrained contaminants through either the

floor or sides of the dam during the operational life of the dam and for any period of decommissioning and rehabilitation of the dam.

Dams 9	Certification by the suitably qualified and experienced person who supervises the construction must be submitted to the administering authority on the completion of construction of the regulated structure, and state that:

- the 'as constructed' drawings and specifications meet the original intent of the design plan for that regulated structure;

- construction of the regulated structure is in accordance with the design plan.

Dams 10	All affected persons must be provided with a copy of the emergency action plan in place for each regulated structure

- for existing structures that are regulated structures, within 10 business days of this condition taking effect;

- prior to the operation of the new regulated structure; and

- if the emergency action plan is amended, within 5 business days of it being amended.

Dams 11	Operation of a regulated structure, except for an existing structure, is prohibited.

Dams 12	For existing structures that are regulated structures:

- where the existing structure that is a regulated structure is to be managed as part of an integrated containment system for the purpose of sharing the DSA volume across the system, the holder of this environmental authority must submit to the administering authority within 12months of the commencement of this condition a copy of the certified system design plan including that structure; and

- there must be a current operational plan for the existing structures.

Dams 13	Each regulated structure must be maintained and operated, for the duration of its operational life until decommissioned and rehabilitated, in compliance with the current operational plan and, if applicable, the current design plan and associated certified ‘as constructed’ drawings.

Dams 14	Conditions Dams 15 to Dams 18 inclusive only apply to Regulated Structures which have not been certified as low consequence category for ‘failure to contain – overtopping’.

Dams 15	The Mandatory Reporting Level (the MRL) must be marked on a regulated dam in such a way that during routine inspections of that dam, it is clearly observable.

Dams 16	The holder of this environmental authority must, as soon as practical and within forty-eight

(48) hours of becoming aware, notify the administering authority when the level of the contents of a regulated dam reaches the MRL.

Dams 17	The holder of this environmental authority must, immediately on becoming aware that the MRL has been reached, act to prevent the occurrence of any unauthorised discharge from the regulated dam.

Dams 18	The holder of this environmental authority must record any changes to the MRL in the Register of Regulated Structures.

Dams 19	The holder of this environmental authority must assess the performance of each regulated dam or linked containment system over the preceding November to May period based on actual observations of the available storage in each regulated dam or linked containment system taken prior to 1 July of each year.

Dams 20	By 1 November of each year, storage capacity must be available in each regulated dam (or network of linked containment systems with a shared DSA volume), to meet the Design Storage Allowance (DSA) volume for the dam (or network of linked containment systems).

Dams 21	The holder of this environmental authority must, as soon as possible and within forty-eight

(48) hours of becoming aware that the regulated dam (or network of linked containment systems) will not have the available storage to meet the DSA volume on 1 November of any year, notify the administering authority.

Dams 22	The holder of this environmental authority must, immediately on becoming aware that a regulated dam (or network of linked containment systems) will not have the available storage to meet the DSA volume on 1 November of any year, act to prevent the occurrence of any unauthorised discharge from the regulated dam or linked containment systems.

Dams 23	Each regulated structure must be inspected each calendar year by a suitably qualified and experienced person.

Dams 24	At each annual inspection, the condition and adequacy of all components of the regulated structure must be assessed and a suitably qualified and experienced person must prepare an annual inspection report containing details of the assessment and include a recommendations section, with any recommended actions to ensure the integrity of the regulated structure or a positive statement that no recommendations are required.

Dams 25	The suitably qualified and experienced person who prepared the annual inspection report must certify the report in accordance with the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933).

Dams 26	The holder of this environmental authority must, within 20 business days of receipt of the annual inspection report, provide to the administering authority:

- The recommendations section of the annual inspection report; and

- If applicable, any actions being taken in response to those recommendations; and

- If, following receipt of the recommendations and (if applicable) recommended actions, the administering authority requests a copy of the annual inspection report from the holder of this environmental authority, provide this to the administering authority within 10 business days of receipt of the request.

Dams 27	The holder of this environmental authority must provide a copy of any reports, documentation and certifications prepared under this environmental authority, including but not limited to any Register of Regulated Structures, consequence assessment, design plan

and other supporting documentation, to a new holder on transfer of this environmental authority.

Dams 30	A Register of Regulated Dams must be established and maintained by the holder of this environmental authority for each regulated dam.

Dams 31	The holder of this environmental authority must provisionally enter the required information in the Register of Regulated Dams when a design plan for a regulated dam is submitted to the administering authority.

Dams 32   The holder of this environmental authority must make a final entry of the required information in the Register of Regulated Structures once compliance with conditions Dams 11 and Dams 12 has been achieved.

Dams 33	The holder of this environmental authority must ensure that the information contained in the Register of Regulated Dams is current and complete on any given day.

Dams 34	All entries in the Register of Regulated Dams must be approved by the chief executive officer for the holder of this environmental authority, or their delegate, as being accurate and correct.

Dams 35	The holder of this environmental authority must, at the same time as providing the annual return, supply to the administering authority a copy of the records contained in the Register of Regulated Dams, in the electronic format required by the administering authority.

Dams 36	All existing structures that have not been assessed in accordance with either the Manual or the former Manual for Assessing Hazard Categories and Hydraulic Performance of Dams must be assessed and certified in accordance with the Manual within 6 months of amendment of the authority adopting this schedule.

Dams 37	All existing structures must subsequently comply with the timetable for any further assessments in accordance with the Manual specified in Schedule C, Table 1 – Transitional requirements for existing structures, depending on the consequence category for each existing structure assessed in the most recent previous certification for that structure.

Dams 38  Schedule C, Table 1 – Transitional requirements for existing structures ceases to apply for a structure once any of the following events has occurred:

- it has been brought into compliance with the hydraulic performance criteria applicable to the structure under the Manual; or

- it has been decommissioned; or

- it has been certified as no longer being assessed as a regulated structure.

Dams 39  Certification of the transitional assessment required by conditions Dams 36 and Dams 37 (as applicable) must be provided to the administering authority within 6 months of amendment of the authority adopting this schedule.

Schedule C, Table 1 - Transitional hydraulic performance requirements for existing structures

| Transition period required for existing structures to achieve the requirements of the Manual for Assessing Consequence Categories and Hydraulic Performance of Dams |  |  |  |
| --- | --- | --- | --- |
| Compliance with criteria | High | Significant | Low |
| >90% and a history of good compliance performance in last 5 years | No transition required | No transition required | No transitional conditions apply. Review consequence assessment every 7 years. |
| >70% - ≤90% | Within 7 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases. | Within 10 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases. | No transitional conditions apply. Review consequence assessment every 7 years. |
| >50 -≤70 percent | Within 5 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases. | Within 7 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases. | Review consequence assessment every 7 years. |
| ≤50% | Within 5 years or as per compliance requirements (e.g. TEP timing) | Within 5 years or as per compliance requirements (e.g. TEP timing) | Review consequence assessment every 5 years. |
| Regulated levee designed to prevent the ingress of clean flood water <100% compliant1 | Within 5 years unless otherwise agreed with the administering authority. |  |  |

1 Levees designed for the diversion of contaminated waters or protection of the structural integrity of a dam are not to be considered as part of this provision. These levees are considered a key design element of the relevant dam and transitional periods should as such align to that relevant compliance criteria and consequence category.

# Schedule D – Land

Land 1	Contaminants must not be directly or indirectly released to land except for those releases authorised by this environmental authority.

Land 2	Top soil must be managed in a manner that preserves its biological and chemical properties.

Land 3	Land that has been significantly disturbed by the petroleum activities must be managed to ensure that mass movement, gully erosion, rill erosion, sheet erosion and tunnel erosion do not occur on that land.

Land 4	Acid sulfate soils must be treated and managed in accordance with the latest edition of the

Queensland Acid Sulfate Soil Technical Manual.

Land 5	Chemicals and fuels stored, must be effectively contained and where relevant, meet Australian Standards, where such a standard is applicable.

Land 6	Pipeline operation and maintenance must be in accordance, to the greatest practicable extent, with the relevant section of the APGA Code of Environmental Practice: Onshore Pipelines (2017) (or more recent editions).

Land 7	Pipeline trenches must be backfilled and topsoils reinstated within three months after pipe laying.

Land 8	Reinstatement and revegetation of the pipeline right of way must commence within 6 months after cessation of petroleum activities for the purpose of pipeline construction.

Land 9	Backfilled, reinstated and revegetated pipeline trenches and right of ways must be:

- a stable landform;

- re-profiled to a level consistent with surrounding soils;

- re-profiled to original contours and established drainage lines; and

- vegetated with groundcover which is not a pest species, and which is established and growing.

| Land 10 | Stormwater Management The size of any sedimentation dam must be sufficient to contain the run-off expected from a 24-hour storm with an average recurrence interval of 1 in 5 years. |
| --- | --- |
| Land 11 | Contaminants other than settled/treated stormwater runoff waters must not be released from the site to surface waters or the bed or banks of surface waters. |
| Land 12 | There must be no release of stormwater runoff that has been in contact with any contaminants at the site to any waters, roadside gutter or stormwater drain other than as authorised under condition Land 11 |
| Land 13 | Land 12 does not prohibit the release of stormwater provided that the stormwater has been treated in an oil/water interceptor, sedimentation basin or other process to effectively remove contaminants before the stormwater is released to waters. |

Biodiversity 1	Prior to undertaking activities that result in significant disturbance to land in areas of native vegetation, confirmation of on-the-ground biodiversity values of the native vegetation communities at that location must be undertaken by a suitably qualified person.

Biodiversity 2	A suitably qualified person must develop and certify a methodology so that condition (Biodiversity 1) can be complied with and which is appropriate to confirm on-the-ground biodiversity values.

Biodiversity 3	For conditions (Biodiversity 4) to (Biodiversity 9), where mapped biodiversity values differ from those confirmed under conditions (Biodiversity 1) and (Biodiversity 2), petroleum activities may proceed in accordance with the conditions of the environmental authority based on the confirmed on-the-ground biodiversity value.

Biodiversity 4	The location of the petroleum activity(ies) must be selected in accordance with the following site planning principles:

- maximise the use of areas of pre-existing disturbance;

- in order of preference, avoid, minimise or mitigate any impacts, including cumulative impacts, on areas of native vegetation or other areas of ecological value;

- minimise disturbance to land that may result in land degradation ;

- in order of preference, avoid then minimise isolation, fragmentation, edge effects or dissection of tracts of native vegetation; and

- in order of preference, avoid then minimise clearing of native mature trees.

Biodiversity 5	Linear infrastructure construction corridors must:

- maximise co-location;

- be minimised in width to the greatest practicable extent; and

- for linear infrastructure that is an essential petroleum activity authorised in an environmentally sensitive area or its protection zone, be no greater than 40m in total width, except for those authorised in Schedule D, Table 1 - Authorised Disturbance Widths (Linear Infrastructure)

Schedule D, Table 1— Authorised Disturbance Widths (Linear Infrastructure)

| Infrastructure | ESA Type | Location | Maximum Disturbance Area (ha) |
| --- | --- | --- | --- |
| Road and Right of Way (Lynwood to Tipton) | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ | DXP 4 EA Disturbance Spatial Data | 4.5 |
| RoW (STP_PAD_WP622) | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ |  | 1.4 |
| Right of Way Boundary Road | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ |  | 13.1 |

Biodiversity 6	Despite Biodiversity 5 (c), Should the quality of protection zone land be deemed historically disturbed (subject to assessment by a suitably qualified person), or of low environmental value, then Biodiversity 5 (c) is silent.

Biodiversity 7	Despite condition (Biodiversity 8A), activities are authorised in areas field verified as cleared, cropped, or grazing,, including primary protection zone (PPZ) and secondary protection zone (SPZ).

Biodiversity 8	Produced water gathering and/or flow pipelines are authorised in all areas essential petroleum activities are authorised in Schedule D, Table 2— Authorised petroleum activities in environmentally sensitive areas and their protection zones.

Biodiversity 8A	Where petroleum activities are to be carried out in environmentally sensitive areas or their protection zones, the petroleum activities must be carried out in accordance with Schedule D, Table 2— Authorised petroleum activities in environmentally sensitive areas and their protection zones.

Schedule D, Table 2— Authorised petroleum activities in environmentally sensitive areas and their protection zones

| Environmentally sensitive area | Within the environmentally sensitive area | Primary protection zone of the environmentally sensitive area | Secondary protection zone of the environmentally sensitive area |
| --- | --- | --- | --- |
| Category A environmentally sensitive areas | No petroleum activities permitted. | Only low impact petroleum activities permitted. | Only essential petroleum activities permitted. |
| Category B environmentally sensitive areas that are other than ‘endangered’ regional ecosystems | Only low impact petroleum activities permitted. | Only low impact petroleum activities permitted. | Only essential petroleum activities permitted. |
| Category B environmentally sensitive areas that are ‘endangered’ regional ecosystems | Only low impact petroleum activities permitted. | Only essential petroleum activities permitted. | Only essential petroleum activities permitted. |
| Category C ESAs that are on the ‘essential habitat map’, or are ‘of concern’ regional ecosystems | Only low impact petroleum activities permitted. | Only essential petroleum activities permitted. |  |
| Category C ESAs that are ‘protected wildlife habitat’ | Only essential petroleum activities permitted. | Only essential petroleum activities permitted. |  |
| Category C environmentally sensitive areas that are ‘regional parks’ (previously known as ‘resources reserves’) | Only essential petroleum activities permitted. | Only essential petroleum activities permitted. |  |
| Category C environmentally sensitive areas that are ‘state forests’ or ‘timber reserves’ | Only essential petroleum activities permitted. | Petroleum activities permitted. |  |
| Areas of vegetation that are ‘critically limited’ | Only low impact petroleum activities permitted. | Only essential petroleum activities permitted. |  |

NOPA 2	Despite Biodiversity 8A the additional disturbance areas prescribed in Schedule D, Table 3 – Prescribed additional disturbance areas (Land) are permitted to be located within the areas shown on shape file Additional Disturbance Areas June 2024.

Schedule D, Table 3 – Prescribed additional disturbance areas (Land)

| Tenure | Description of Infrastructure | ESA Type | Disturbance Area | Shapefile Reference |
| --- | --- | --- | --- | --- |
| PL260 | Road upgrade [Broadwater Road] and access tracks | Primary protection zone of Category A ESA | 1.74 ha | Additional Disturbance Areas June 2024 |
|  | Right of way and access track | Category C Environmentally sensitive areas that are ‘essential habitat’, ‘essential regrowth habitat’, or ‘of concern’ regional ecosystems | 0.65 ha | Additional Disturbance Areas June 2024 |
| PL252 | Right of way, road upgrade [Ducklo School Road] and access tracks | Category B environmentally sensitive areas that are ‘endangered’ regional ecosystems | 0.48 ha | Additional Disturbance Areas June 2024 |
| PL230 | Right of way and road upgrade [Dalby Kogan Road] | Category C Environmentally sensitive areas that are ‘essential habitat’, ‘essential regrowth habitat’, or ‘of concern’ regional ecosystems | 9.93 ha | Additional Disturbance Areas June 2024 |
| PL198 | Well Pad and road upgrade [Hennings Road | Category C Environmentally sensitive areas that are ‘essential habitat’, ‘essential regrowth habitat’, or ‘of concern’ regional ecosystems | 0.22 ha | Additional Disturbance Areas June 2024 |

Biodiversity 8B	Despite (Biodiversity 8A), activities and associated activities necessary for construction, operation, monitoring and maintenance are permitted within the limits prescribed in Schedule D, Table 4 – Coordinates and disturbance areas for authorised disturbances to environmentally sensitive areas and protection zones.

Schedule D, Table 4 – Coordinates and disturbance areas for authorised disturbances to environmentally sensitive areas and protection zones

| Infrastructure | ESA Type | Latitude |  | Longitude | Maximum Disturbance Area |
| --- | --- | --- | --- | --- | --- |
| Longswamp 31 shallow monitoring bore | Category A ESA | -27.343471 |  | 151.095733 | 9 m2 |
| Tipton 253 gas well | Primary protection zone of Category C ESA | -27.36818 |  | 151.13539 | 1.96 ha |
| PL238 SPV_PAD_WP110 Multi Well Pad | Primary Protection zone of Category C ESA OCRE and Essential Habitat. | -27.321875 |  | 151.228109 | 1.60 ha |
| PL238 SPV_PAD_WP120 Multi Well Pad | Primary Protection zone of Category C ESA OCRE and Essential Habitat | -27.344911 |  | 151.243697 | 1.75 ha |
| Right of Way (SLS_PAD_WP369 | Category B ESAs that are ‘endangered’ regional ecosystems | DXP 4 EA Disturbance Spatial Data |  |  | 0.07 |
| Right of Way (SLS_PAD_WP382) | Category B ESAs that are ‘endangered’ regional ecosystems |  |  |  | 0.05 |
| Access Track (SPV_PAD_WP045) | Category C ESAs that are on the ‘essential habitat map’ |  |  |  | 1.1 |
| Laydown (SLS_PAD_WP378) | Secondary Protection zone of Category B ESA that are ‘endangered’ regional ecosystems |  |  |  | 0.81 |
| Well Pad (STP_PAD_WP642) | Category C ESAs that are ‘of concern’ regional ecosystems |  |  |  | 1 |
| Right of Way (STP_PAD_WP642, STP_PAD_WP643, STP_PAD_WP639) | Category C ESAs that are ‘of concern’ regional ecosystems |  |  |  | 3.6 |
| EWA (Wanka Rd Crossing) | Category C ESAs that are on the ‘essential habitat map’ |  |  |  | 0.05 |
| EWA (Greenbank Access Rd Crossing) | Primary Protection Zone of Category B ESAs that are ‘endangered’ regional ecosystems |  |  |  | 0.05 |
| Lynwood FCS and Ancillary Infrastructure | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 32.77 |
| Well Pad (Tipton 608) and Communication Tower | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat |  |  |  | 1.75 |
| Well Pad (Tipton 660) | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 2.05 |
| EWA (Crossing near STP_PAD_WP630) | Category C ESAs that are ‘protected wildlife habitat’ and Primary Protection Zone of Category C ESAs that are ‘of concern’ regional ecosystems |  |  |  | 0.2 |
| EWA (Crossing near STP_PAD_WP620) | Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 0.2 |
| EWA (Crossing near STP_PAD_WP623) | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 0.16 |
| EWA (Wilkin Rd Crossing) | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 0.05 |
| EWA (Boundary Road Crossing) | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ Primary Protection Zone of Category C ESAs that are ‘of concern’ regional ecosystems |  |  |  | 0.11 |
| Segregation Valve Area | Primary Protection Zone of Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 0.24 |
| EWA Boundary Road RoW | Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 0.13 |
| EWA (STP_PAD_WP601) Gathering | Category C ESAs that are ‘protected wildlife habitat’ |  |  |  | 0.06 |
| Road (Broadwater Road) | Primary Protection zone of Category A ESA |  |  |  | 9.3 |
| Road (Wilkin Rd) | Category C ESAs that are on the ‘essential habitat map’ |  |  |  | 5.9 |

| Infrastructure | Reference Point | Easting (GDA94, Zone 56) | Northing (GDA94, Zone 56) | Maximum disturbance Area |
| --- | --- | --- | --- | --- |
| Daandine Brine Dam 2 | 1 | 297524 | 7001708 | 53.3ha |
|  | 2 | 297384 | 7001153 |  |
|  | 3 | 298345 | 7001051 |  |
|  | 4 | 298444 | 7001418 |  |
|  | 5 | 298406 | 7001601 |  |
|  | 6 | 298190 | 7001620 |  |
| Water Release Outlet and Pipeline | Valve Pit | 302897 | 6995424 | 0.18 ha |
|  | High Point on Bank | 302991 | 6995460 |  |
|  | Outlet at Creek | 303004 | 6995465 |  |
| Tipton Treated Water Pipeline | Section 1 | 310653 | 6969687 | 16.4 ha |
|  |  | 310656 | 6969747 |  |
|  |  | 310696 | 6969745 |  |
|  |  | 310661 | 6970448 |  |
|  |  | 310621 | 6970453 |  |
|  |  | 310651 | 6970652 |  |
|  |  | 310611 | 6970657 |  |
|  |  | 310642 | 6970846 |  |
|  |  | 310602 | 6970846 |  |
|  |  | 310707 | 6971627 |  |
|  |  | 310667 | 6971630 |  |
|  |  | 310659 | 6972161 |  |
|  |  | 310618 | 6972167 |  |
|  |  | 310621 | 6973392 |  |
|  |  | 310581 | 6973399 |  |
|  |  | 310613 | 6973550 |  |
|  |  | 310573 | 6973550 |  |
|  |  | 310639 | 6973650 |  |
|  |  | 310539 | 6973550 |  |
|  |  | 310608 | 6973650 |  |
|  |  | 310568 | 6973650 |  |
|  |  | 310639 | 6973550 |  |
|  |  | 310539 | 6973650 |  |
|  | Section 2 | 310617 | 6973719 | 5.6ha |
|  |  | 310577 | 6973724 |  |
|  |  | 310722 | 6974060 |  |
|  |  | 310682 | 6974065 |  |
|  |  | 310717 | 6974172 |  |
|  |  | 310677 | 6974177 |  |
|  |  | 310714 | 6974221 |  |
|  |  | 310675 | 6974217 |  |
|  |  | 310692 | 6974378 |  |
|  |  | 310654 | 6974365 |  |
|  |  | 310692 | 6974378 |  |
|  |  | 310567 | 6974521 |  |
|  |  | 310617 | 6974514 |  |
|  |  | 310563 | 6974528 |  |
|  |  | 310613 | 6974522 |  |
|  |  | 310499 | 6974536 |  |
|  |  | 310594 | 6974610 |  |
|  |  | 310497 | 6974636 |  |
|  | Section 3 | 310534 | 6974847 | 2.0ha |
|  |  | 310494 | 6974835 |  |
|  |  | 310534 | 6974857 |  |
|  |  | 310494 | 6974862 |  |
|  |  | 310528 | 6975192 |  |
|  |  | 310488 | 6975197 |  |
|  |  | 310526 | 6975351 |  |
|  |  | 310486 | 6975347 |  |

Biodiversity 9	A report must be prepared for each annual return period for all petroleum activities that involved clearing of any environmentally sensitive area or protection zone which includes:

- records able to demonstrate compliance with conditions (Biodiversity 4), (Biodiversity 5) and (Biodiversity 8A).

- a description of the works;

- a description of the area and its pre-disturbance values (which may include maps or photographs, but must include GPS coordinates for the works); and

- based on the extent of environmentally sensitive areas and primary protection zones on the relevant resource authority(ies), the proportion of native vegetation cleared per environmentally sensitive area and primary protection zone, including regional ecosystem type, over the annual return period.

Biodiversity 10 Significant residual impacts to prescribed environmental matters (other than if the impacts were authorised by an existing authority issued before the commencement of the Environmental Offsets Act 2014) are not authorised under this environmental authority or the Environmental Offsets Act 2014 unless the impact(s) is specified in Schedule D, Table 5 — Significant residual impacts to prescribed environmental matters.

Biodiversity 11 Records demonstrating that each impact to a prescribed environmental matter not listed in Schedule D, Table 5 — Significant residual impacts to prescribed environmental matters did not, or is not likely to, result in a significant residual impact to that matter must be:

- completed by an appropriately qualified person; and

- kept for the life of the environmental authority.

Schedule D, Table 5 — Significant residual impacts to prescribed environmental matters

| Prescribed environmental matter |  | Maximum extent of impact (ha) | Stage 1f (DXP 4) |
| --- | --- | --- | --- |
| REGULATED VEGETATION |  |  |  |
| Endangered regional ecosystem |  |  |  |
| RE 11.3.21 |  | 3 | 0 |
| RE 11.4.3 |  | 0.44 | 0.44 (MNES) |
| Of concern regional ecosystem (not within an urban area) |  |  |  |
| RE 11.3.2 |  | 16 | 2.42 |
| RE 11.3.4 |  | 15 | 0 |
| RE 11.3.17 |  | 15 | 0 |
| Regional ecosystems (not within an urban area) that intersect a wetland on the vegetation management wetlands map |  |  |  |
| RE 11.3.4 |  | 1 | 0 |
| RE 11.3.27 |  | 2 | 0 |
| Regional ecosystems (not within an urban area) within the defined distance from the defining banks of a relevant watercourse on the vegetation management watercourse map |  |  |  |
| RE 11.3.2 |  | 0.86 | 0.11 |
| 11.3.14 |  | 0.20 | 0.20 |
| 11.3.4 (BVG 16c) |  | 4.28 | 0 |
| 11.3.18 |  | 2.57 | 0.30 |
| 11.3.25 |  | 12.86 | 0.56 |
| 11.4.12 (BVG 17a) |  | 0.86 | 0 |
| 11.5.1 (BVG 17a; 18b) |  | 4.28 | 0 |
| 11.7.4 |  | 0.43 | 0.01 |
| 11.7.7 (BVG 12a) |  | 0.86 | 0 |
| Essential habitat (not in an urban area) for endangered wildlife |  |  |  |
| Hemiaspis damelii |  | 6 | 0 |
| Essential habitat (not in an urban area) for vulnerable wildlife |  |  |  |
| Calyptorhynchus lathami |  | 1 | 0 |
| Jalmenus eubulus |  | 0.5 | 0 |
| Petauroides volans |  | 0.003 | 0.003 |
| CONNECTIVITY AREAS |  |  |  |
| Connectivity area that is a Regional Ecosystem (not in urban area) |  | 29.48 | 29.48 |
| WETLANDS AND WATERCOURSES |  |  |  |
| A wetland of high ecological significance shown on the Map of referable wetlands |  | 1.81 | 0.34 (no SRI) |
| PROTECTED WILDLIFE HABITAT |  |  |  |
| An area shown as a high risk area on the flora survey trigger map that contains plants that are endangered or vulnerable wildlife |  |  |  |
| Picris barbarorum |  | 2.5 | 0 |
| Solanum papaverifolium |  | 3 | 0 |
| Habitat for an animal that is vulnerable wildlife |  |  |  |
| Acanthophis antarcticus |  | 317 | 31.67 |
| Calyptorhynchus lathami |  | 94 | 12.04 |
| Furina dunmalli |  | 27.25 | 27.25 (MNES) |
| Grantiella picta |  | 0.94 | 0.94 |
| Nyctophilus corbeni |  | 29.11 | 29.11 (MNES) |
| Petaurus australis |  | 21.79 | 21.79 |
| Petauroides volans |  | 24.40 | 24.50 |
| Stagonopleura guttata |  | 32.38 | 32.38 |
| Jalmenus eubulus |  | 17 | 0 |
| Tachyglossus aculeatus |  | 57 | 0 |
| Habitat for an animal that is endangered wildlife |  |  |  |
| Hemiaspis damelii |  | 219 | 88.60 |
| Phascolarctos cinereus |  | 72.81 | 72.81 |
| Adclarkia cameroni |  | 8.27 | 8.27 |
| FISH HABITAT AREAS |  |  |  |
| Waterway providing for fish passage Fish passage (not in an urban area) |  | 6 | 0.20 (no SRI) |

Biodiversity 12 An environmental offset made in accordance with the Environmental Offsets Act 2014 and Queensland Environmental Offsets Policy, as amended from time to time, must be undertaken for the maximum extent of impact to each prescribed environmental matter authorised in Schedule D, Table 3— Significant residual impacts to prescribed environmental matters, unless a lesser extent of the impact has been approved in accordance with condition (Biodiversity 14).

Biodiversity 13 The significant residual impacts to a prescribed environmental matter authorised in condition (Biodiversity 10) for which an environmental offset is required by condition (Biodiversity 12) may be carried out in stages. An environmental offset can be delivered for each stage of the impacts to prescribed environmental matters.

Biodiversity 14 Prior to the commencement of each stage, a report completed by an appropriately qualified person, that includes an analysis of the following must be provided to the administering authority:

- for the forthcoming stage—the estimated significant residual impacts to each prescribed environmental matter; and

- for the previous stage, if applicable—the actual significant residual impacts to each prescribed environmental matter, to date.

Biodiversity 15 The report required by condition (Biodiversity 14) must be approved by the administering authority before a notice of election for the forthcoming stage, if applicable, is given to the administering authority.

Biodiversity 16 A notice of election for the staged environmental offset referred to in condition (Biodiversity 14), if applicable, must be provided to the administering authority no less than three months before the proposed commencement of that stage, unless a lesser timeframe has been agreed to by the administering authority.

Biodiversity 17 Within six months from the completion of the final stage of the project, a report completed by an appropriately qualified person, that includes the following matters must be provided to the administering authority:

- an analysis of the actual impacts on prescribed environmental matters resulting from the final stage; and

- if applicable, a notice of election to address any outstanding offset debits for the authorised impacts.

# Schedule E – Acoustic

Noise 1	Notwithstanding condition (General 21), emission of noise from the petroleum activity(ies) at levels less than those specified in Schedule E, Table 1—Noise nuisance limits are not considered to be environmental nuisance.

Noise 2	If the noise subject to a valid complaint is tonal or impulsive, the adjustments detailed in Schedule E, Table 2—Adjustments to be added to noise levels at sensitive receptors are to be added to the measured noise level(s) to derive LAeq, adj, 15 min.

Schedule E, Table 1—Noise nuisance limits1

| Time period | Metric | Short term noise event | Medium term noise event | Long term noise event |
| --- | --- | --- | --- | --- |
| 7:00am—6:00pm | LAeq,adj,15 min | 45 dBA | 43 dBA | 40 dBA |
| 6:00pm—10:00pm | LAeq,adj,15 min | 40 dBA | 38 dBA | 35 dBA |
| 10:00pm—6:00am | LAeq,adj,15 min | 28 dBA | 28 dBA | 28 dBA |
|  | Max LpA, 15mins | 55 dBA | 55 dBA | 55 dBA |
| 6:00am—7:00am | LAeq,adj,15 min | 40 dBA | 38 dBA | 35 dBA |
| Drilling activities undertaken from 10:00pm – 7:00am2 | LAeq, adj, 15min | 28 dBA (measured indoors) 33 dBA (measured outdoors) |  |  |

1 The noise limits in Schedule E, Table 1 – Noise nuisance limits have been set based on the following deemed background noise levels (LABG):

7:00am—6:00 pm: 35 dBA

6:00pm—10:00 pm: 30 dBA

10:00pm—6:00 am: 25 dBA

6:00am—7:00 am: 30 dBA

2 Drilling activities (e.g. drilling, workover, completion activities) undertaken from 10:00 pm – 7:00 am must be temporary and mobile in nature, and must not contribute to long-term background noise creep.

Schedule E, Table 2—Adjustments to be added to noise levels at sensitive receptors

| Noise characteristic | Adjustment to noise |
| --- | --- |
| Tonal characteristic is just audible | + 2 dBA |
| Tonal characteristic is clearly audible | + 5 dBA |
| Impulsive characteristic is detectable | + 2 to + 5 dBA |

Noise 3  Notwithstanding condition (Noise 1), emission of any low frequency noise must not exceed either (Noise 3(a)) and (Noise 3(b)), or (Noise 3(c)) and (Noise 3(d)) in the event of a valid complaint about low frequency noise being made to the administering authority:

- 60 dB(C) measured outside the sensitive receptor; and

- the difference between the external A-weighted and C-weighted noise levels is no greater than 20 dB; or

- 50 dB(Z) measured inside the sensitive receptor; and

- the difference between the internal A-weighted and Z-weighted (Max LpZ, 15 min) noise levels is no greater than 15 dB.

E10	Within 12 months of commissioning the units listed in Schedule E, Table 3 – Tipton Expansion Project units, the EA holder must, conduct noise monitoring under worst case noise propagation conditions to validate the pre-commissioning noise predictions at sensitive receptors.

E11	The holder of this environmental authority must provide the administering authority with a report of the monitoring results required under condition (E10) that evaluates the accuracy of the pre- commissioning model predictions at sensitive receptors.

Schedule E, Table 3 – Tipton Expansion Project units

| Resource Authority | Field | Facility | Unit Description |
| --- | --- | --- | --- |
| PL198 | Tipton | Tipton Central Gas Processing Facility | K-0007 Compressor 7 |
|  |  |  | K-0007 Compressor 8 |
|  |  |  | K-0007 Compressor 9 |
|  |  |  | K-0007 Compressor 10 |
|  |  |  | K-0015 Inlet Fuel Gas Compressor Engine |
|  |  | Tipton Water Treatment Facility | Generator 1 |
|  |  |  | Generator 2 |
|  |  |  | Generator 3 |
|  |  |  | Generator 4 |
|  |  |  | Generator 5 |

Noise 4	A Blast Management Plan must be developed for each blasting activity in accordance with Australian Standard 2187.

Noise 5	Blasting operations must be designed to not exceed an airblast overpressure level of 120 dB (linear peak) at any time, when measured at or extrapolated to any sensitive place.

Noise 6	Blasting operations must be designed to not exceed a ground-borne vibration peak particle velocity of 10mm/s at any time, when measured at or extrapolated to any sensitive place.

# Schedule F – Air

Air 1	Unless venting is authorised under the Petroleum and Gas (Production and Safety) Act 2004 or the Petroleum Act 1923, waste gas must be flared in a manner that complies with all of (Air 1(a)) and (Air 1(b)) and (Air 1(c)), or with (Air 1(d)):

- an automatic ignition system is used, and

- a flame is visible at all times while the waste gas is being flared, and

- there are no visible smoke emissions other than for a total period of no more than 5 minutes in any 2 hours, or

- it uses an enclosed flare.

Air 2A	A fuel burning or combustion facility must not be operated unless it is listed in Schedule F, Table 1– Authorised point sources.

Air 2B	If a fuel burning or combustion facility is listed in Schedule F, Table 1—Authorised point sources, the fuel burning or combustion facility must be operated so that the releases to air do not exceed the limits specified in Schedule F, Table 1—Authorised point sources at the specified release point reference.

Air 3	Point source air monitoring for each fuel burning or combustion facility listed in Schedule F, Table 1– Authorised point sources must:

- be undertaken:

  - once in the first three months after each facility is first commissioned, and then

  - annually or biennially thereafter at the frequency specified in Schedule F,

Table 2 – Annual Air Quality Monitoring

- be carried out when the facility the subject of the sampling is operating under maximum operating conditions for the annual period; and

- demonstrate compliance with the limits listed in Schedule F, Table 1– Authorised point sources at each release point reference.

Schedule F, Table 1 — Authorised point sources

| Tenure | Facility | Release Point Reference | Unit Description | Minimum Release Height (m) | Minimum Efflux Velocity (m/sec)1 | NOX as Nitrogen Dioxide | Carbon Monoxide |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Maximum Mass Emission Rate (g/sec)1 | Maximum Mass Emission Rate (g/sec)1 |
|  | Daandine | A1 | K-0001 Compressor 1 |  |  |  |  |
| PL 230 | Central Gas Processing |  |  | 10 | 30 | 3.0 | 5.5 |
|  |  | A2 | K-0002 Compressor 2 |  |  |  |  |
|  | Facility |  |  |  |  |  |  |
|  |  | A3 | K-0003 Compressor 3 |  |  |  |  |

| Tenure | Facility | Release Point Reference | Unit Description | Minimum Release Height (m) | Minimum Efflux Velocity (m/sec)1 | NOX as Nitrogen Dioxide | Carbon Monoxide |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Maximum Mass Emission Rate (g/sec)1 | Maximum Mass Emission Rate (g/sec)1 |
|  |  | A4 | K-0004 Compressor 4 |  |  |  |  |
|  |  | A5 | K-0005 Compressor 5 |  |  |  |  |
|  |  | A6 | K-0006 Compressor 6 |  |  |  |  |
|  |  | A7 | K-0007 Compressor 7 |  |  |  |  |
|  |  | A8 | K-9008 Compressor 8 | 17 | 17 | 1.4 | 4.8 |
|  |  | A9 | K-9009 Compressor 9 |  |  |  |  |
|  |  | A10 | K-9010 Compressor 10 |  |  |  |  |
|  |  | A11 | K-9011 Inlet Fuel Screw Compressor Engine 11 | 8.5 | 30 | 1.5 | 1.0 |
| PL 198 | Tipton West Central Gas Processing Facility | A14 | K-0001 Compressor 1 | 7.6 | 30 | 6.8 | 5.5 |
|  |  | A15 | K-0002 Compressor 2 |  |  |  |  |
|  |  | A16 | K-0003 Compressor 3 |  |  |  |  |
|  |  | A17 | K-0004 Compressor 4 |  |  |  |  |
|  |  | A18 | K-0005 Compressor 5 |  |  |  |  |
|  |  | A19 | K-0006 Compressor 6 |  |  |  |  |
|  |  | A20 | K-0007 Compressor 7 | 17 | 17 | 1.4 | 4.8 |
|  |  | A21 | K-0008 Compressor 8 |  |  |  |  |
|  |  | A22 | K-0009 Compressor 9 |  |  |  |  |
|  |  | A23 | K-0010 Compressor 10 |  |  |  |  |
|  | Tipton Water | A24 | K-0015 Inlet Fuel Gas | 8.5 | 38 | 1.5 | 1.0 |

| Tenure | Facility | Release Point Reference | Unit Description | Minimum Release Height (m) | Minimum Efflux Velocity (m/sec)1 | NOX as Nitrogen Dioxide | Carbon Monoxide |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Maximum Mass Emission Rate (g/sec)1 | Maximum Mass Emission Rate (g/sec)1 |
|  | Treatment Facility2 |  | Compressor Engine |  |  |  |  |
|  |  | A25 | Generator 1 | 7.5 | 27 | 1.5 | 1.5 |
|  |  | A26 | Generator 2 |  |  |  |  |
|  |  | A27 | Generator 3 |  |  |  |  |
|  |  | A28 | Generator 4 |  |  |  |  |
|  |  | A29 | Generator 5 |  |  |  |  |
|  | Lynwood Field Compression Station | A30 | Generator Stack 1 | 5.5 | 12 | 0.8 | NA |
|  |  | A31 | Generator Stack 2 |  |  |  |  |
|  |  | A32 | Generator Stack 3 |  |  |  |  |
|  |  | A33 | Generator Stack 4 |  |  |  |  |
|  |  | A34 | Generator Stack 5 |  |  |  |  |
|  |  | A35 | Generator Stack 6 |  |  |  |  |
|  |  | A36 | Generator Stack 7 |  |  |  |  |
|  |  | A37 | Generator Stack 8 |  |  |  |  |
|  |  | A38 | Generator Stack 9 |  |  |  |  |
|  |  | A39 | Generator Stack 10 |  |  |  |  |
|  |  | A40 | Generator Stack 11 |  |  |  |  |
|  |  | A41 | Generator Stack 12 |  |  |  |  |
|  |  | A42 | Generator Stack 13 |  |  |  |  |
|  |  | A43 | Generator Stack 14 |  |  |  |  |
|  |  | A44 | Generator Stack 15 |  |  |  |  |
|  |  | A45 | Generator Stack 16 |  |  |  |  |
|  |  | A46 | Generator Stack 17 |  |  |  |  |
|  |  | A47 | Generator Stack 18 |  |  |  |  |

1 Minimum efflux velocity, maximum mass emission and maximum concentration limits relate to plant maximum continuous ratings.

2 The Water Treatment Facility’s aggregated fuel consumption exceeds the 500 kg per hour threshold when all five generators are in simultaneous operation.

## Schedule F, Table 2 – Annual Air Quality Monitoring

| Release Point | Parameter1 |  | Minimum Monitoring Frequency |
| --- | --- | --- | --- |
|  | Mass emission rate (g/s) | Concentration (mg/Nm3) |  |
| A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17, A18, A19, A20, A21, A22, A23, A24, A252, A262, A272, A282, A292 A303, A313, A323, A333, A343, A353, A363, A373, A383, A393, A403, A413, A423, A433, A443, A453. A463, A473 | oxides of nitrogen (measured as NO2) carbon monoxide | oxides of nitrogen (measured as NO2) carbon monoxide | Biennial from commission |

1 Measured in flue gas at the 5% oxygen reference level

2 Biennial monitoring is not required until at least four of the five generators are commissioned.

3 Monitoring of carbon monoxide from A30 – A47 is not required as per Schedule F, Table1 – Authorised point sources.

# Schedule G – Waste

Waste 1	Measures must be implemented so that waste is managed in accordance with the waste and resource management hierarchy and the waste and resource management principles.

Waste 2	Waste, including waste fluids, but excluding waste used in closed-loop systems, must be transported off-site for lawful re-use, remediation, recycling or disposal, unless the waste is specifically authorised by conditions of this environmental authority to be disposed of or used on site.

Waste 3	Waste fluids, other than flare precipitant stored in flare pits, or residual drilling material or drilling fluids stored in sumps, must be contained in either:

- an above ground container; or

- a structure which contains the wetting front.

Waste 4	Green waste may be used on-site for either rehabilitation or sediment and erosion control, or both.

Waste 5	Vegetation waste may be burned if it relates to a state forest, timber reserve or forest entitlement area administered by the Forestry Act 1959 and a permit has been obtained under the Fire and Rescue Service Act 1990.

Waste 6	Pipeline waste water may be released to land provided that it:

- can be demonstrated it meets the acceptable standards for release to land; and

- is released in a way that does not result in visible scouring or erosion or pooling or run-off or vegetation die-off.

Waste 7	Produced water may be re-used in drilling and well hole activities.

Waste 8	Produced water may be used for dust suppression provided the following criteria are met:

- The amount applied does not exceed the amount required to effectively suppress dust; and

- The application:

  - Does not cause on-site ponding or runoff;

  - Is directly applied to the area being dust suppressed;

  - Does not harm vegetation surrounding the area being dust suppressed; and

  - Does not cause visible salting.

Waste 9	Produced water may be used for construction and operation purposes provided the use:

- Does not result in negative impacts on the composition and structure of soil or subsoils;

- Is not directly or indirectly released to waters;

- Does not result in runoff from the construction site; and

- Does not harm vegetation surrounding the construction site.

Waste 10	If there is any indication that any of the circumstances in condition (Waste 8)(b)(i) to (Waste 8(b)(iv)) or (Waste 9)(a) to (Waste 9(d)) is occurring, the use must cease immediately and the affected area must be remediated without delay.

Waste 11	Treated sewage effluent or greywater can be released to land provided it:

- meets or exceeds secondary treated class B standards for a treatment system with a daily peak design capacity of between 150 EP and 1500 EP; or

- meets or exceeds secondary treated class C standards for a treatment system with a daily peak design capacity of less than 150 EP.

Waste 12	The release of treated sewage effluent or greywater authorised in condition (Waste 11) must:

- be to a fenced and signed contaminant release area(s);

- not result in pooling or run-off or aerosols or spray drift or vegetation die-off;

- be to a contaminant release area(s) that is kept vegetated with groundcover, that is:

  - not a pest species;

  - kept in a viable state for transpiration and nutrient uptake; and

  - grazed or harvested and removed from the contaminant release area as needed, but not less than every three months.

Waste 13	Notwithstanding condition (Waste 11), treated sewage effluent that meets or exceeds secondary treated class A standards may be used for dust suppression or construction activities, provided the use meets the criteria in condition (Waste 8) or (Waste 9), as relevant to the use.

Waste 14	Sewage pump stations must be fitted with a:

- stand-by pump; and

- high level alarm to warn of imminent pump station overflow, that operates without mains power or with a back-up power source that starts automatically in the event of a power failure.

Waste 15	If sumps are used to store residual drilling material or drilling fluids, they must only be used for the duration of drilling activities.

Waste 16	Residual drilling material can only be disposed of on-site:

- by mix-bury-cover method if the residual drilling material meets the approved quality criteria; or

- if it is certified by a suitably qualified third party as being of acceptable quality for disposal to land by the proposed method and that environmental harm will not result from the proposed disposal.

Waste 17	Records must be kept to demonstrate compliance with condition (Waste 15) and (Waste 16).

G12	Coal seam gas water may be transferred to a third party to be used for the following purposes subject to compliance with conditions (G13) and (G14):

- dust suppression if the coal seam gas water quality complies with the limits specified in Schedule G, Table 1 – Water Contaminant Release Limits;

- construction and operational purposes if the coal seam gas water quality complies with the limits specified in Schedule G, Table 1 – Water Contaminant Release Limits;

- irrigation and livestock watering purposes;

- the following industrial purposes:

  - coal washing;

  - power stations; and

  - water treatment facilities.

G13	Any coal seam gas water supplied to a third party for irrigation and/or livestock watering purposes in accordance with Condition (G12)(c) must comply with the relevant trigger values contained in ANZECC and ARMCANZ Water Quality Guidelines 2000, or subsequent versions thereof.

Schedule G, Table 1 – Water Contaminant Release Limits

| Water Quality Characteristics | Unit | Limit | Limit Type |
| --- | --- | --- | --- |
| pH | pH units | 6.0 to 9.0 | Range |
| Sodium Adsorption Ratio | ratio | 6 | 80th Percentile |
|  |  | 12 | Maximum |
| Total Dissolved Solids | mg/L | 1500 | Maximum |
| Total Petroleum Hydrocarbons | mg/L | 10 | Maximum |

G14	If the responsibility of coal seam gas water is given or transferred to a third party in accordance with Condition (G12), the holder of environmental authority must ensure that:

- the responsibility of the coal seam gas water is given or transferred in accordance with a written agreement (the third party agreement); and

- the third party is made aware of the General Environmental Duty under section 319 of the Environmental Protection Act 1994.

G15	The only off tenure waste to be accepted is brine waste from Braemar 2 Power Station demineralisation plant located on Lot 101 Plan SP221421.

G16	The waste referred to in condition (G15) must only be accepted at the Daandine Feedwater Dam, and Daandine Aggregation Dam.

# Schedule H – Rehabilitation

Rehabilitation 1	A Rehabilitation Plan must be developed by a suitably qualified person and must include the:

  - rehabilitation goals; and

  - procedures to be undertaken for rehabilitation that will:

    - achieve the requirements of conditions (Rehabilitation 2) to (Rehabilitation 8), inclusive; and

    - provide for appropriate monitoring and maintenance.

Rehabilitation 2	Significantly disturbed areas that are no longer required for the on-going petroleum activities, must be rehabilitated within 12 months (unless an exceptional circumstance in the area to be rehabilitated (e.g. a flood event) prevents this timeframe being met) and be maintained to meet the following acceptance criteria:

- contaminated land resulting from petroleum activities is remediated and rehabilitated;

- the areas are:

  - non-polluting;

  - a stable landform;

  - re-profiled to contours consistent with the surrounding landform;

- surface drainage lines are re-established;

- top soil is reinstated; and

- either:

  - groundcover, that is not a pest species, is growing; or

  - an alternative soil stabilisation methodology that achieves effective stabilisation is implemented and maintained.

Rehabilitation 3	All significantly disturbed areas caused by petroleum activities which are not being or intended to be utilised by the landholder or overlapping tenure holder, must be rehabilitated to meet the following final acceptance criteria measured either against the highest ecological value adjacent land use or the pre-disturbed land use:

- greater than or equal to 70% of native ground cover species richness

- greater than or equal to the total per cent of ground cover

- less than or equal to the per cent species richness of plant pest species; and

- where the adjacent land use contains, or the pre-clearing land use contained, one or more regional ecosystem(s), then at least one regional ecosystem(s) from the same broad vegetation group, and with the equivalent biodiversity

status or a biodiversity status with a higher conservation value as any of the regional ecosystem(s) in either the adjacent land or pre-disturbed land, must be present.

Rehabilitation 4	Where significant disturbance to land has occurred in an environmentally sensitive area, the following final rehabilitation criteria as measured against the pre-disturbance biodiversity values assessment (required by conditions (Biodiversity 1) and (Biodiversity 2)) must be met:

- greater than or equal to 70% of native ground cover species richness;

- greater than or equal to the total per cent ground cover;

- less than or equal to the per cent species richness of plant pest species;

- greater than or equal to 50% of organic litter cover;

- greater than or equal to 50% of total density of coarse woody material; and

- all predominant species in the ecologically dominant layer, that define the pre- disturbance regional ecosystem(s) are present.

Rehabilitation 5	Conditions (Rehabilitation 2), (Rehabilitation 3) and (Rehabilitation 4) continue to apply after this environmental authority has ended or ceased to have effect.

Rehabilitation 8	Where there is a dam (including a low consequence dam) that is being or intended to be utilised by the landholder or overlapping tenure holder, the dam must be decommissioned to no longer accept inflow from the petroleum activity(ies) and the contained water must be of a quality suitable for the intended on-going uses(s) by the landholder or overlapping tenure holder.

### Definitions

Key terms and/or phrases used in this document are defined in this section. Where a term is not defined, the definition in the Environmental Protection Act 1994, its regulations or environmental protection policies must be used. If a word remains undefined it has its ordinary meaning.

| Word or Phrase | Definition |
| --- | --- |
| acceptable standards for release to land | means wastewater of the following quality as determined by monitoring results or by characterisation: - electrical conductivity (EC) not exceeding 3000μS/cm; - sodium adsorption ratio (SAR) not exceeding 8; - pH between 6.0 and 9.0; - heavy metals (measured as total) meets the respective short term trigger value in section 4.2.6, Table 4.2.10—Heavy metals and metalloids in Australian and New Zealand Guidelines for Fresh and Marine Water Quality; - does not contain biocides. |
| acid sulfate soil(s) | means a soil or soil horizon which contains sulfides or an acid soil horizon affected by oxidation of sulfides. |
| additional disturbance areas | means the areas that were disturbed prior to authorisation by the administering authority for the activity/ies under Chapter 5 of the EP Act: - prescribed in Schedule B, Table B — Prescribed additional disturbance areas (Water), and Schedule D, Table A – Prescribed additional disturbance areas (Land) and mapped in shapefile Additional Disturbance Areas June 2024 and in reference to the Notice of Proposed Action to Amend the Environmental Authority (ss215(2)(a) and 217 of the EP Act) issued to Arrow Energy Pty Ltd, dated 9 July 2024. or - prescribed in Schedule B, Table B - Prescribed additional disturbance areas (Water) and mapped in shapefile Disturbance Areas January 2025 |
| adjacent land use(s) | means the ecosystem function adjacent to an area of significant disturbance, or where there is no ecosystem function, the use of the land. An adjacent land use does not include an adjacent area that shows evidence of edge effect. |
| administering authority | means: - for a matter, the administration and enforcement of which has been devolved to a local government under section 514 of the Environmental Protection Act 1994—the local government; or - for all other matters—the Chief Executive of the Department of Environment, Tourism, Science and Innovation; or - another State Government Department, Authority, Storage Operator, Board or Trust, whose role is to administer provisions under other enacted legislation. |
| alternative arrangement | means a written agreement about the way in which a particular environmental nuisance impact will be dealt with at a sensitive place, and may include an agreed period of time for which the arrangement is in place. An alternative arrangement may include, but is not limited to, a range of nuisance abatement |

| Word or Phrase | Definition |  |  |  |
| --- | --- | --- | --- | --- |
|  | measures to be installed at the sensitive place, or provision of alternative accommodation for the duration of the relevant nuisance impact. |  |  |  |
| analogue site(s) | means an area of land which contains values and characteristics representative of an area to be rehabilitated prior to disturbance. Such values must encompass land use, topographic, soil, vegetation, vegetation community attributes and other ecological characteristics. Analogue sites can be the pre- disturbed site of interest where significant surveying effort has been undertaken to establish benchmark parameters. |  |  |  |
| annual return period | means the most current 12-month period between two anniversary dates. |  |  |  |
| appraisal well | means a petroleum well to test the potential of one (1) or more natural underground reservoirs for producing or storing petroleum. For clarity, an appraisal well does not include an exploration well. |  |  |  |
| appropriately qualified person / suitably qualified person | means a person who has professional qualifications, training or skills or experience relevant to the nominated subject matters and can give authoritative assessment, advice and analysis about performance relevant to the subject matters using relevant protocols, standards, methods or literature. |  |  |  |
| Approved quality criteria | for the purposes of residual drilling materials, means the residual drilling material meet the following quality standards: Part A In all cases: Parameter Maximum concentration pH 6 to 10.5 (range) Electrical 20 dS/m Conductivity (20,000 μS/cm) Chloride* 8000 mg/L *Chloride analysis is only required if an additive containing chloride was used in the drilling process The limits in Part A must be measured in the clarified filtrate of oversaturated solids prior to mixing. Part B If any of the following metals are a component of the drilling fluids, then for that metal: |  |  |  |
|  |  | Parameter | Maximum concentration |  |
|  |  | Arsenic | 20 mg/kg |  |
|  |  | Selenium | 5 mg/kg |  |
|  |  | Boron | 100 mg/kg |  |
|  |  | Cadmium | 3 mg/kg |  |
|  |  | Chromium (total) | 400 mg/kg |  |
|  |  | Copper | 100 mg/kg |  |

| Word or Phrase | Definition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Lead | 600 mg/kg |  |  |  |
|  | The limits in Part B and Part C refer to the post soil/by-product mix. Part C If a hydrocarbon sheen is visible, the following hydrocarbon fractions: |  |  |  |  |  |
|  |  | TPH |  | Maximum concentration |  |  |
|  |  | C6-C10 |  | 170 mg/kg |  |  |
|  |  | C10-C16 |  | 150 mg/kg |  |  |
|  |  | C16-C34 |  | 1300 mg/kg |  |  |
|  |  | C34-C40 |  | 5600 mg/kg |  |  |
|  |  | Total polycyclic aromatic hydrocarbons (PAH) |  | 20 mg/kg |  |  |
|  |  | Phenols (halogenated) |  | 1 mg/kg |  |  |
|  |  | Phenols (non- halogenated) |  | 60 mg/kg |  |  |
|  |  | Monocyclic aromatic hydrocarbons (total sum of benzen, toluene, ethyl benzene, xylenes (includes ortho, para, and meta xylenes) and styrene) |  | 7 mg/kg |  |  |
|  |  | Benzene |  | 1 mg/kg |  |  |
| areas of pre-existing disturbance | means areas where environmental values have been negatively impacted as a result of anthropogenic activity and these impacts are still evident. Areas of pre-disturbance may include areas where legal clearing, logging, timber harvesting, or grazing activities have previously occurred, where high densities of weed or pest species are present which have inhibited re-colonisation of native regrowth, or where there is existing infrastructure (regardless of whether the infrastructure is associated with the authorised petroleum activities). The term ‘areas of pre-disturbance’ does not include areas that have been impacted by wildfire/s, controlled burning, flood or natural vegetation die-back. |  |  |  |  |  |
| associated water | means underground water taken or interfered with, if the taking or interference happens during the course of, or results from, the carrying out of another authorised activity under a petroleum authority, such as a petroleum well, and includes waters also known as produced formation water. The term includes all contaminants suspended or dissolved within the water. |  |  |  |  |  |
| associated works | in relation to a dam, means: - operations of any kind and all things constructed, erected or installed for that dam; and - any land used for those operations. |  |  |  |  |  |

| Word or Phrase | Definition |
| --- | --- |
| Australian Standard 3580 | means any of the following publications: - AS3580.10.1 Methods for sampling and analysis of ambient air— Determination of particulate matter—Deposited matter—Gravimetric method. - AS3580.9.6 Methods for sampling and analysis of ambient air— Determination of suspended particulate matter—PM10 high volume sampler with size-selective inlet— Gravimetric method - AS3580.9.9 Methods for sampling and analysis of ambient air— Determination of suspended particulate matter— PM10 low volume sampler—Gravimetric sampler. |
| background noise level | means the sound pressure level, measured in the absence of the noise under investigation, as the L A90,T being the A-weighted sound pressure level exceeded for 90% of the measurement time period T of not less than 15 minutes (or LA 90, adj, 15 mins), using Fast response. |
| bankfull | means the channel flow rate that exists when the water is at the elevation of the channel bank above which water begins to spill out onto the floodplain. The term describes the condition of the channel relative to its banks (e.g. overbank, in-bank, bankfull, low banks, high bank). |
| bed | of any waters, has the meaning in Schedule 19 of the Environmental Protection Regulation 2019 and— - includes an area covered, permanently or intermittently, by tidal or non-tidal waters; but - does not include land adjoining or adjacent to the bed that is from time to time covered by floodwater. |
| being or intended to be utilised by the landholder or overlapping tenure holder | for significantly disturbed land, means there is a written agreement (e.g. land and compensation agreement) between the landholder or the overlapping tenure holder and the holder of the environmental authority identifying that the landholder or the overlapping tenure holder has a preferred use of the land such that rehabilitation standards for revegetation by the holder of the environmental authority are not required. For dams, means there is a written agreement (e.g. land and compensation agreement) between the landholder or the overlapping tenure holder and the holder of the environmental authority identifying that the landholder or the overlapping tenure holder has a preferred use for the dam such that rehabilitation standards for revegetation by the holder of the environmental authority are not required. |
| biodiversity values | for the purposes of this environmental authority, means environmentally sensitive areas, prescribed environmental matters and wetlands. |
| BTEX | means benzene, toluene, ethylbenzene, ortho-xylene, para-xylene, meta- xylene and total xylene. |
| Category A Environmentally Sensitive Area | means any area listed in Schedule 19, Section 3 of the Environmental Protection Regulation 2019. |
| Category B Environmentally Sensitive Area | means any area listed in Schedule 19, Section 3 of the Environmental Protection Regulation 2019. |

| Word or Phrase | Definition |
| --- | --- |
| Category C Environmentally Sensitive Area | means any of the following areas: - nature refuges as defined in the conservation agreement for that refuge under the Nature Conservation Act 1992 - koala habitat areas as defined under the Nature Conservation (Koala) Conservation Plan 2017 - state forests or timber reserves as defined under the Forestry Act 1959 - regional parks (previously known as resource reserves) under the Nature Conservation Act 1992 - an area validated as ‘essential habitat’ from ground-truthing surveys in accordance with the Vegetation Management Act 1999 for a species of wildlife listed as endangered or vulnerable under the Nature Conservation Act 1992 - ‘of concern regional ecosystems’ that are remnant vegetation and identified in the database called ‘RE description database’ containing regional ecosystem numbers and descriptions. |
| certified or certification | in relation to any matter other than a design plan, ‘as constructed’ drawings or an annual report regarding dams means, a Statutory Declaration by a suitably qualified person or suitably qualified third party accompanying the written document stating: - the person’s qualifications and experience relevant to the function - that the person has not knowingly included false, misleading or incomplete information in the document - that the person has not knowingly failed to reveal any relevant information or document to the administering authority - that the document addresses the relevant matters for the function and is factually correct; and - that the opinions expressed in the document are honestly and reasonably held. |
| clearing | has the meaning in the dictionary of the Vegetation Management Act 2000 and for vegetation— - means remove, cut down, ringbark, push over, poison or destroy in any way including by burning, flooding or draining; but - does not include destroying standing vegetation by stock, or lopping a tree. |
| closed-loop systems | means using waste on site in a way that does not release waste or contaminants in the waste to the environment. |
| coal seam gas water | means underground water brought to the surface of the earth, or moved underground in connection with exploring for, or producing coal seam gas. |
| control measure | has the meaning in section 47 of the Environmental Protection Regulation 2019 and means a device, equipment, structure, or management strategy used to prevent or control the release of a contaminant or waste to the environment. |
| critically limited regional ecosystem | means the regional ecosystems defined and listed in Appendix 5 of the Queensland Biodiversity Offset Policy. |
| daily peak design capacity | for sewage treatment works, has the meaning in Schedule 2, section 63(4) of the Environmental Protection Regulation 2019 as the higher equivalent person |

| Word or Phrase | Definition |
| --- | --- |
|  | (EP) for the works calculated using each of the formulae found in the definition for EP. |
| dam(s) | means a land-based structure or a void that contains, diverts or controls flowable substances, and includes any substances that are thereby contained, diverted or controlled by that land-based structure or void and associated works. |
| design storage allowance or DSA | means an available volume, estimated in accordance with the Manual for Assessing Consequence Categories and Hydraulic Performance of Structures (ESR/2016/19337), published by the administering authority, as amended from time to time, that must be provided in a dam to an annual exceedance probability specified in that Manual. |
| designated precinct | has the meaning in Part 5 section 15(3) of the Regional Planning Interests Regulation 2014 and means: - for a strategic environmental area mentioned in section 4(1) – the area identified as a designated precinct on the strategic environmental area map for the strategic environmental are; or - if a strategic environmental area is shown on a map in a regional plan – the area identified on the map as a designated precinct for the strategic environmental area |
| development wells | means a petroleum well which produces or stores petroleum. For clarity, a development well does not include an appraisal well. |
| document | has the meaning in the Acts Interpretation Act 1954 and means: - any paper or other material on which there is writing; and - any paper or other material on which there are marks; and - figures, symbols or perforations having a meaning for a person qualified to interpret them; and - any disc, tape or other article or any material from which sounds, images, writings or messages are capable of being produced or reproduced (with or without the aid of another article or device). |
| ecologically dominant layer | has the meaning in the Methodology for Surveying and Mapping of Regional Ecosystems and Vegetation Communities in Queensland (Version 5.1 March 2020) and means the layer making the greatest contribution to the overall biomass of the site and the vegetation community (National Land and Water Resources Audit 2001). This is also referred to as the ecologically dominant stratum or the predominant canopy in woody ecosystems. |
| ecosystem function | means the interactions between and within living and nonliving components of an ecosystem and generally correlates with the size, shape and location of the vegetation community. |
| enclosed flare | means a device where the residual gas is burned in a cylindrical or rectilinear enclosure that includes a burning system and a damper where air for the combustion reaction is admitted. |
| environmental harm | has the meaning in section 14 of the Environmental Protection Act 1994 and means any adverse effect, or potential adverse effect (whether temporary or permanent and of whatever magnitude, duration or frequency) on an |

| Word or Phrase | Definition |
| --- | --- |
|  | environmental value, and includes environmental nuisance. Environmental harm may be caused by an activity— - whether the harm is a direct or indirect result of the activity; or - whether the harm results from the activity alone or from the combined effects of the activity and other activities or factors. |
| environmental nuisance | has the meaning in section 15 of the Environmental Protection Act 1994 and means unreasonable interference or likely interference with an environmental value caused by— - aerosols, fumes, light, noise, odour, particles or smoke; or - an unhealthy, offensive or unsightly condition because of contamination; or - another way prescribed by regulation. |
| environmental offset | has the meaning in section 7 of the Environmental Offsets Act 2014. |
| environmentally sensitive area | means Category A, B or C environmentally sensitive areas (ESAs) |
| equivalent person or EP | has the meaning under section 3 of the Planning Guidelines For Water Supply and Sewerage, 2010, published by the Queensland Government. It is calculated in accordance with Schedule 2, Section 63(4) of the Environmental Protection Regulation 2019where: - EP = V/200 where V is the volume, in litres, of the average dry weather flow of sewage that can be treated at the works in a day; or - EP = M/2.5 where M is the mass, in grams, of phosphorus in the influent that the works are designed to treat as the inlet load in a day. |
| essential petroleum activities | means activities that are essential to bringing the resource to the surface and are only the following: - low impact petroleum activities - geophysical, geotechnical, geological, topographic and cadastral surveys (including seismic, sample / test / geotechnical pits, core holes) - single well sites not exceeding 1 hectare disturbance and multi-well sites not exceeding 1.5 hectare disturbance - well sites with monitoring equipment (including monitoring bores): - for single well sites, not exceeding 1.25 hectares disturbance - for multi-well sites, not exceeding 1.75 hectares disturbance - well sites with monitoring equipment (including monitoring bores) and tanks (minimum 1 ML) for above ground fluid storage: - for single well sites, not exceeding 1.5 hectares disturbance - for multi-well sites, not exceeding 2.0 hectares disturbance - well sites with slope considerations (>2% slope) for cut and fill earthworks and drainage: - for single well sites, not exceeding 1.5 hectares disturbance - for multi-well sites, not exceeding 2.5 hectares disturbance - swell sites including a Communications Tower: - for single well sites, not exceeding 1.5 hectare disturbance - for multi-well sites, not exceeding 3.0 hectare disturbance - associated infrastructure located on a well site necessary for the construction and operations of wells: - water pumps and generators |

| Word or Phrase | Definition |
| --- | --- |
|  | - flare pits - chemical / fuel storages - sumps for residual drilling material and drilling fluids - tanks, or dams which are not significant or high consequence dams to contain wastewater (e.g. stimulation flow back waters, produced water) - pipe laydown areas - soil and vegetation stockpile areas - a temporary camp associated with a drilling rig that may involve sewage treatment works that are no release works - temporary administration sites and warehouses - dust suppression activities using water that meets the quality and operational standards approved under the environmental authority - communication and power lines that are necessary for the undertaking of petroleum activities and that are located within well sites, well pads and pipeline right of ways without increasing the disturbance area of petroleum activities - on site disposal of residual drilling material as per condition (Waste 16) - communications towers, not exceeding 1.0 hectares disturbance - supporting access tracks - gathering / flow pipelines from a well head to the initial compression facility - activities necessary to achieve compliance with the conditions of the environmental authority in relation to another essential petroleum activity (e.g. sediment and erosion control measures, rehabilitation). |
| existing authority | has the meaning in section 94 of the Environmental Offsets Act 2014. |
| exploration well | means a petroleum well that is drilled to: - explore for the presence of petroleum or natural underground reservoirs suitable for storing petroleum; or - obtain stratigraphic information for the purpose of exploring for petroleum. For clarity, an exploration well does not include an appraisal or development well. |
| extra work areas (EWA) | means temporary workspace which are necessary to facilitate construction of linear infrastructure. Their uses include bell holes for Horizontal Directional Drilling or pipe thrusting, storage of spoil and vegetation stockpiles, laydown for construction equipment, and providing additional space for manoeuvring of plant. |
| flare pit | has the meaning in the Manual for Assessing Consequence Categories and Hydraulic Performance of Structures (ESR/2016/19338), and means containment area where any hydrocarbon that is discovered in an over- pressured reservoir during a drilling operation is diverted to, and combusted, The flare pit is only used during the drilling and work over process on a petroleum well. |
| flare precipitant | means waste fluids which result from the operation of a flare. |

| Word or Phrase | Definition |
| --- | --- |
| floodplains | has the meaning in the Water Act 2000 and means an area of reasonably flat land adjacent to a watercourse that— - is covered from time to time by floodwater overflowing from the watercourse; and - does not, other than in an upper valley reach, confine floodwater to generally follow the path of the watercourse; and - has finer sediment deposits than the sediment deposits of any bench, bar or in-stream island of the watercourse. |
| flowable substance | means matter or a mixture of materials which can flow under any conditions potentially affecting that substance. Constituents of a flowable substance can include water, other liquids fluids or solids, or a mixture that includes water and any other liquids fluids or solids either in solution or suspension. |
| fuel burning or combustion facility | means a permanent fuel burning or combustion equipment which in isolation, or combined in operation, or which are interconnected, is, or are capable of burning more than 500 kg of fuel in an hour. |
| GDA | means Geocentric Datum of Australia. |
| Great Artesian Basin (GAB) spring | means an area protected under the Environment Protection and Biodiversity Conservation Act 1999 because it is considered to be a Matter of National Environmental Significance and identified as a: - community of native species dependent on natural discharge of groundwater from the Great Artesian Basin; or - Great Artesian Basin spring; or - Great Artesian Basin discharge spring wetland. A GAB spring includes a spring vent, spring complex or watercourse spring and includes the land to which water rises naturally from below the ground and the land over which the water then flows. Note: The Australian Government’s Protected Matters Search Tool should be used to get an indication of whether the area of interest may contain an MNES spring. Note: The GAB springs dataset can be requested from the Queensland Government Herbarium |
| green waste | means waste that is grass cuttings, trees, bushes, shrubs, material lopped from trees, untreated timber or other waste that is similar in nature but does not include pest species. |
| greywater | means wastewater generated from domestic activities such as laundry, dishwashing, and bathing. Greywater does not include sewage. |
| groundwater dependent ecosystem (GDE) | means ecosystems which require access to groundwater on a permanent or intermittent basis to meet all or some of their water requirements so as to maintain their communities of plants and animals, ecological processes and ecosystem services. For the purposes of the environmental authority, groundwater dependent ecosystems do not include those mapped as “unknown”. |

| Word or Phrase | Definition |
| --- | --- |
| growing | means to increase by natural development, as any living organism or part thereof by assimilation of nutriment; increase in size or substance. |
| hydraulic integrity | refers to the capacity of a dam to contain or safely pass flowable substances based on its design. |
| impulsive (for noise) | means sound characterised by brief excursions of sound pressure (acoustic impulses) that significantly exceed the background sound pressure. The duration of a single impulsive sound is usually less than one second. |
| LA 90, adj, 15 mins | means the A-weighted sound pressure level, adjusted for tonal character that is equal to or exceeded for 90% of any 15 minutes sample period equal, using Fast response. |
| LAeq, adj, 15 mins | means an A-weighted sound pressure level of a continuous steady sound, adjusted for tonal character, that within a 15 minute period has the same square sound pressure as a sound level that varies with time. |
| land degradation | has the meaning in the Vegetation Management Act 1999 and means the following: - soil erosion - rising water tables - the expression of salinity - mass movement by gravity of soil or rock - stream bank instability - a process that results in declining water quality. |
| landholder’s active groundwater bore | means bores that are able to continue to provide a reasonable yield of water in terms of quantity for the bores authorised purpose or use. This term does not include monitoring bores owned by the administering authority of the Water Act 2000. |
| linear infrastructure | means powerlines, pipelines, roads and access tracks. |
| liquid | means a substance which is flowing and offers no permanent resistance to changes of shape. |
| long term noise event | means a noise exposure, when perceived at a sensitive receptor, persists for a period of greater than five (5) days, even when there are respite periods when the noise is inaudible within those five (5) days. |
| low consequence dam | means any dam that is not classified as high or significant as assessed using the Manual for Assessing Consequence Categories and Hydraulic Performance of Structures, published by the administering authority, as amended from time to time. |
| low impact petroleum activities | means petroleum activities which do not result in the clearing of native vegetation, cause disruption to soil profiles through earthworks or excavation or result in significant disturbance to land which cannot be rehabilitated immediately using hand tools after the activity is completed. Examples of such activities include but are not necessarily limited to soil surveys (excluding test pits), topographic surveys, cadastral surveys and ecological surveys, may include installation of monitoring equipment provided that it is within the meaning of low impact and traversing land by car or foot via existing access |

| Word or Phrase | Definition |
| --- | --- |
|  | tracks or routes or in such a way that does not result in permanent damage to vegetation. |
| Map of Queensland Wetland Environmental Values | has the meaning defined in the Environmental Protection (Water and Wetland Biodiversity) Policy 2019 Environmental Protection Regulation 2019and means the ‘Map of Queensland Wetland Environmental Values, a document approved by the chief executive on and published by the department, as amended from time to time by the chief executive under Section 19. |
| Max LpA, 15 min | means the absolute maximum instantaneous A-weighted sound pressure level, measured over 15 minutes. |
| Max LpZ, 15 min | means the maximum value of the Z-weighted sound pressure level measured over 15 minutes. |
| maximum extent of impact | means the total, cumulative, residual extent and duration of impact to a prescribed environmental matter that will occur over a project’s life after all reasonable avoidance and reasonable on-site mitigation measures have been, or will be, undertaken. |
| medium term noise event | is a noise exposure, when perceived at a sensitive receptor, persists for an aggregate period not greater than five days and does not re-occur for a period of at least four weeks. Re-occurrence is deemed to apply where a noise of comparable level is observed at the same receptor location for a period of one hour or more, even if it originates from a difference source or source location. |
| methodology | means the science of method, especially dealing with the logical principles underlying the organisation of the various special sciences, and the conduct of scientific inquiry. |
| mix-bury-cover method | means the stabilisation of residual drilling solids in the bottom of a sump by mixing with subsoil and which occurs in accordance with the following methodology: - the base of the subsoil and residual solid mixture must be separated from the groundwater table by at least one metre of a continuous layer of impermeable subsoil material (kw=10-8m/s) or subsoil with a clay content of greater than 20 percent; and - the residual solids is mixed with subsoil in the sump and cover; and - the subsoil and residual solids is mixed at least three parts subsoil to one part waste (v/v); and - a minimum of one metre of clean subsoil must be placed over the subsoil and residual solids mixture; and - topsoil is replaced. |
| month | has the meaning in the Acts Interpretation Act 1954 and means a calendar month and is a period starting at the beginning of any day of one (1) of the 12 named months and ending— - immediately before the beginning of the corresponding day of the next named month; or - if there is no such corresponding day—at the end of the next named month. |
| NATA accreditation | means accreditation by the National Association of Testing Authorities Australia. |

| Word or Phrase | Definition |
| --- | --- |
| notice of election | has the meaning in section 18(2) Environmental Offsets Act 2014. |
| pest | Means a plant or animal, other than a native species of plant or animal, that is – - an Invasive biosecurity matter under the Biosecurity Act 2014* - a Controlled biosecurity matter or regulated biosecurity matter under the Biosecurity Act 2014 or - a Locally significant invasive species declared under Local Government Act 2009 as local law. *See Biosecurity Act 2014, schedule 1, part 3 or 4 or schedule 2, part 2. See also the notes to the Biosecurity Act 2014, schedules 1 and 2. Invasive biosecurity matter is defined to include invasive plants and animals as listed as prohibited and restricted matter in schedules 1 and 2 of the Biosecurity Act 2014. |
| pipeline waste water | means hydrostatic testing water, flush water or water from low point drains. |
| pre-disturbed land use | means the function or use of the land as documented prior to significant disturbance occurring at that location. |
| predominant species | has the meaning in the Methodology for Surveying and Mapping of Regional Ecosystems and Vegetation Communities in Queensland (Version 5.1 March 2020) and means a species that contributes most to the overall above-ground biomass of a particular stratum |
| prescribed contaminants | has the meaning in section 440ZD of the Environmental Protection Act 1994. |
| prescribed environmental matters | has the meaning in section 10 of the Environmental Offsets Act 2014, limited to the matters of State environmental significant listed in schedule 2 of the Environmental Offsets Regulation 2014. |
| primary protection zone | means an area within 200m from the boundary of any Category A, B or C ESA. |
| produced water | has the meaning in Section 15A of the Petroleum and Gas (Production and Safety) Act 2004 and means CSG water or associated water for a petroleum tenure. |
| protection zone | means the primary protection zone of any Category A, B or C ESA or the secondary protection zone of any Category A or B ESA. |
| regional ecosystem | has the meaning in the Methodology for Surveying and Mapping of Regional Ecosystems and Vegetation Communities in Queensland (Version 5.1 March 2020) and means a vegetation community in a bioregion that is consistently associated with a particular combination of geology, landform and soil. Regional ecosystems of Queensland were originally described in Sattler and Williams (1999). The Regional Ecosystem Description Database (Queensland Herbarium 2013) is maintained by Queensland Herbarium and contains the current descriptions of regional ecosystems. |
| regulated dam | means any dam in the significant or high consequence category as assessed using the Manual for Assessing Consequence Categories and Hydraulic |

| Word or Phrase | Definition |
| --- | --- |
|  | Performance of Structures (ESR/2016/19339), published by the administering authority, as amended from time to time. |
| rehabilitation or rehabilitated | means the process of reshaping and revegetating land to restore it to a stable landform and in accordance with acceptance criteria and, where relevant, includes remediation of contaminated land. For the purposes of pipeline rehabilitation, rehabilitation includes reinstatement, revegetation and restoration |
| reinstate or reinstatement | for pipelines, means the process of bulk earth works and structural replacement of pre-existing conditions of a site (i.e. soil surface typography, watercourses, culverts, fences and gates and other landscape(d) features) and is detailed in the Australian Pipeline Industry Association (APIA) Code of Environmental Practice: Onshore Pipelines (2013). |
| reporting limit | means the lowest concentration that can be reliably measured within specified limits of precision and accuracy during routine laboratory operating conditions. For many analytes, the reporting limit is selected as the lowest non-zero standard in the calibration curve. Results that fall below the reporting limit will be reported as “less than” the value of the reporting limit. The reporting limit is also referred to as the practical quantitation limit or the limit of quantitation. For polycyclic aromatic hydrocarbons, the reporting limit must be based on super- ultra trace methods and, depending on the specific polycyclic aromatic hydrocarbon, will range between 0.005 µg/L–0.020 µg/L. |
| residual drilling material | means waste drilling materials including muds and cuttings or cement returns from well holes and which have been left behind after the drilling fluids are pumped out. |
| restoration | means the replacement of structural habitat complexity, ecosystem processes, services and function from a disturbed or degraded site to that of a pre- determined or analogue site. For the purposes of pipelines, restoration applies to final rehabilitation after pipeline decommissioning. |
| restricted stimulation fluids | has the meaning in section 206 of the Environmental Protection Act 1994 and means fluids used for the purpose of stimulation, including fracturing, that contain the following chemicals in more than the maximum amount prescribed under a regulation— - petroleum hydrocarbons containing benzene, ethylbenzene, toluene or xylene - chemicals that produce, or are likely to produce, benzene, ethylbenzene, toluene or xylene as the chemical breaks down in the environment. |
| revegetation or revegetating or revegetate | means to actively re-establish vegetation through seeding or planting techniques in accordance with site specific management plans. |
| secondary protection zone | in relation to a Category A or Category B ESA means an area within 100 metres from the boundary of the primary protection zone. |
| secondary treated class A standards | means treated sewage effluent or greywater which meets the following standards: - total phosphorous as P, maximum 20mg/L - total nitrogen as N, maximum 30mg/L |

| Word or Phrase | Definition |
| --- | --- |
|  | - 5-day biochemical oxygen demand (inhibited) (e.g. release pipe from sewage treatment plant), maximum 20mg/L - suspended solids, maximum 30mg/L - pH, range 6.0 to 8.5 - e-coli, 80th percentile based on at least 5 samples with not less than 30 minutes between samples, 100cfu per 100mL, maximum 1000cfu per 100mL. |
| secondary treated class B standards | means treated sewage effluent or greywater which meets the following standards: - total phosphorous as P, maximum 20mg/L - total nitrogen as N, maximum 30mg/L - 5-day biochemical oxygen demand (inhibited) (e.g. release pipe from sewage treatment plant), maximum 20mg/L - suspended solids, maximum 30mg/L - pH, range 6.0 to 8.5 - e-coli, 80th percentile based on at least 5 samples with not less than 30 minutes between samples, 1000cfu per 100mL, maximum 10000cfu per 100mL. |
| secondary treated class C standards | means treated sewage effluent or greywater which meets the following standards: - total phosphorous as P, maximum 20mg/L - total nitrogen as N, maximum 30mg/L - 5-day biochemical oxygen demand (inhibited) (e.g. Release pipe from sewage treatment plant), maximum 20mg/L - suspended solids, maximum 30mg/L - pH, range 6.0 to 8.5 - e-coli, 80th percentile based on at least 5 samples with not less than 30 minutes between samples, 10 000cfu per 100mL, maximum 100000cfu per 100mL. |
| sensitive place | means: - a dwelling (including residential allotment, mobile home or caravan park, residential marina or other residential premises, motel, hotel or hostel) - a library, childcare centre, kindergarten, school, university or other educational institution - a medical centre, surgery or hospital - a protected area - a public park or garden that is open to the public (whether or not on payment of money) for use other than for sport or organised entertainment - a work place used as an office or for business or commercial purposes, which is not part of the petroleum activity(ies) and does not include employees accommodation or public roads - for noise, a place defined as a sensitive receptor for the purposes of the Environmental Protection (Noise) Policy 2008. |
| sensitive receptor | is defined in Schedule 2 of the Environmental Protection (Noise) Policy 2008, and means an area or place where noise is measured. |

| Word or Phrase | Definition |
| --- | --- |
| short term noise event | is a noise exposure, when perceived at a sensitive receptor, persists for an aggregate period not greater than eight hours and does not re-occur for a period of at least seven (7) days. Re-occurrence is deemed to apply where a noise of comparable level is observed at the same receptor location for a period of one hour or more, even if it originates from a different source or source location. |
| significant residual impact | has the meaning in section 8 Environmental Offsets Act 2014. |
| significantly disturbed or significant disturbance or significant disturbance to land or areas | has the meaning in Schedule 12, section 4 of the Environmental Protection Regulation 2008. Land is significantly disturbed if— - it is contaminated land; or - it has been disturbed and human intervention is needed to rehabilitate it— - to a condition required under the relevant environmental authority; or - if the environmental authority does not require the land to be rehabilitated to a particular condition—to the condition it was in immediately before the disturbance. |
| species richness | means the number of different species in a given area. |
| Stable | has the meaning in Schedule 8, Part 1 of the Environmental Protection Regulation 2019 and, for a site, means the rehabilitation and restoration of the site is enduring or permanent so that the site is unlikely to collapse, erode or subside. |
| statement of compliance | for a condition in an environmental authority has the meaning in section 208 of the Environmental Protection Act 1994 and is a condition that requires the holder to give the administering authority a statement of compliance about a document or work relating to a relevant activity. The condition must also state— - the criteria (the compliance criteria) the document or work must comply with; and - that the statement of compliance must state whether the document or work complies with the compliance criteria; and - the information (the supporting information) that must be provided to the administering authority to demonstrate compliance with the compliance criteria; and - when the statement of compliance and supporting information must be given to the administering authority. |
| stimulation | means a technique used to increase the permeability of natural underground reservoir that is undertaken above the formation pressure and involves the addition of chemicals. It includes hydraulic fracturing / hydrofraccing, fracture acidizing and the use of proppant treatments. |
| stimulation fluid | means the fluid injected underground to increase permeability. For clarity, the term stimulation fluid only applies to fluid injected down well post-perforation. |
| stimulation impact zone | means a 100m maximum radial distance from the stimulation target location within a gas producing formation. |
| strategic environmental area | has the meaning in section 11(1) of the Regional Planning Interest Act 2014. |

| Word or Phrase | Definition |
| --- | --- |
| structure | means dam or levee. |
| subterranean cave GDE | - means an area identified as a subterranean cave in the mapping produced by the Queensland Government and identified in the Queensland Government Information System, as amended from time to time; and - means a cave ecosystem which requires access to groundwater on a permanent or intermittent basis to meet all or some of their water requirements so as to maintain its communities of plants and animals, ecological processes and ecosystem services. Subterranean cave GDEs are caves dependent on the subterranean presence of groundwater. Subterranean cave GDEs have some degree of groundwater connectivity and are indicated by either high moisture levels or the presence of stygofauna, or both, referred to in the Queensland Government WetlandsInfo mapping program, as amended from time to time. Note: the Subterranean GDE (caves) dataset can be displayed through the Queensland Government WetlandInfo mapping program. Note: the Subterranean GDE (caves) dataset can be obtained from the Queensland Government Information System. |
| suitably qualified third party | means a person who: - has qualifications and experience relevant to performing the function including but not limited to: - a bachelor’s degree in science or engineering; and - 3 years’ experience in undertaking soil contamination assessments; and - is a member of at least one organisation prescribed in Schedule 14 of the Environmental Protection Regulation 2019; and not be an employee of, nor have a financial interest or any involvement which would lead to a conflict of interest with the holder(s) of the environmental authority. |
| sump | means a pit in which waste residual drilling material or drilling fluids are stored only for the duration of drilling activities. |
| synthetic based drilling mud | means a mud where the base fluid is a synthetic oil, consisting of chemical compounds which are artificially made or synthesised by chemically modifying petroleum components or other raw materials rather than the whole crude oil. |
| top soil | means the surface (top) layer of a soil profile, which is more fertile, darker in colour, better structured and supports greater biological activity than underlying layers. The surface layer may vary in depth depending on soil forming factors, including parent material, location and slope, but generally is not greater than about 300mm in depth from the natural surface. |
| total density of coarse woody material | means the total length of logs on the ground greater than or equal to 10cm diameter per hectare and number of logs on the ground greater than or equal to 10cm diameter per hectare. |

| Word or Phrase | Definition |
| --- | --- |
| valid complaint | means all complaints unless considered by the administering authority to be frivolous, vexatious or based on mistaken belief. |
| void | means any constructed, open excavation in the ground. |
| waste and resource management hierarchy | has the meaning provided in section 9 of the Waste Reduction and Recycling Act 2011 and is the following precepts, listed in the preferred order in which waste and resource management options should be considered— - AVOID unnecessary resource consumption - REDUCE waste generation and disposal - RE-USE waste resources without further manufacturing - RECYCLE waste resources to make the same or different products - RECOVER waste resources, including the recovery of energy - TREAT waste before disposal, including reducing the hazardous nature of waste - DISPOSE of waste only if there is no viable alternative. |
| waste and resource management principles | has the meaning provided in section 4(2)(b) of the Waste Reduction and Recycling Act 2011 and means the: - polluter pays principle - user pays principle - proximity principle - product stewardship principle. |
| waste fluids | has the meaning in section 13 of the Environmental Protection Act 1994 in conjunction with the common meaning of “fluid” which is “a substance which is capable of flowing and offers no permanent resistance to changes of shape”. Accordingly, to be a waste fluid, the waste must be a substance which is capable of flowing and offers no permanent resistance to changes of shape. |
| watercourse | has the meaning in Schedule 4 of the Environmental Protection Act 1994 and means: - a river, creek or stream in which water flows permanently or intermittently— - in a natural channel, whether artificially improved or not; or - in an artificial channel that has changed the course of the watercourse. - Watercourse includes the bed and banks and any other element of a river, creek or stream confining or containing water. |
| waters | includes all or any part of a creek, river, stream, lake, lagoon, swamp, wetland, spring, unconfined surface water, unconfined water in natural or artificial watercourses, bed and bank of any waters, non-tidal or tidal waters (including the sea), stormwater channel, stormwater drain, roadside gutter, stormwater run-off, and underground water. |
| well integrity | the ability of a well to contain the substances flowing through it. |
| wetland | for the purpose of this environmental authority, wetland means: - areas shown on the Map of Queensland environmental Values which is a document approved by the chief executive and published by the department, as amended from time to time by the chief executive under section 19 of the Environmental Protection (Water and Wetland Biodiversity) Policy 2019; and |

| Word or Phrase | Definition |
| --- | --- |
|  | - areas defined under the Queensland Wetlands Program as permanent or periodic / intermittent inundation, with water that is static or flowing fresh, brackish or salt, including areas of marine water, the depth of which at low tide does not exceed six (6) metres, and possess one or more of the following attributes: - at least periodically, the land supports plants or animals that are adapted to and dependent on living in wet conditions for at least part of their life cycle, or - the substratum is predominantly undrained soils that are saturated, flooded or ponded long enough to develop anaerobic conditions in the upper layers, or - the substratum is not soil and is saturated with water, or covered by water at some time. The term wetland includes riverine, lacustrine, estuarine, marine and palustrine wetlands; and it does not include a Great Artesian Basin Spring or a subterranean wetland that is a cave or aquifer. |
| wetland of high ecological significance | means a wetland that meets the definition of a wetland and that is shown as a wetland of ‘high ecological significance’ or wetland of ‘high ecological value’ on the Map of Queensland wetland environmental values. |
| wetland of other environmental value | means a wetland that meets the definition of a wetland and that is shown as a wetland of ‘general environmental significance’ or wetland of ‘other environmental value’ on the Map of Queensland wetland environmental values.. |

END OF ENVIRONMENTAL AUTHORITY