Environmental Protection Act 1994

# Environmental authority EA0001401

This environmental authority is issued by the administering authority under Chapter 5 of the Environmental Protection Act 1994.

Environmental authority number: EA0001401

Environmental authority takes effect on xx month 202X. This is the take effect date.

The anniversary date of this environmental authority is the same day each year as the take effect date. The payment of the annual fee will be due each year on this day.

An annual return will be due each year on 01 April.

#### Environmental authority holder(s)

| Name(s) | Registered address |
| --- | --- |
| ARROW CSG (AUSTRALIA) PTY LTD | Level 39 111 Eagle Street BRISBANE CITY QLD 4000 |

Environmentally relevant activity and location details

| Environmentally relevant activity/activities | Location(s) |
| --- | --- |
| Schedule 3 - 06 – A petroleum activity carried out on a site containing a high hazard dam or a significant hazard dam | PL253 |
| Schedule 3 – 03 – A petroleum activity that is likely to have a significant impact on a category A or B environmentally sensitive area. | PL253 |
| Schedule 2 – Ancillary 63 – Sewage Treatment 1: Operating sewage treatment works, other than no release works, with a total daily peak design capacity of (a-i) 21 – 100EP if treated effluent is discharged from the works to an infiltration trench or through an irrigation scheme. | PL253 |

Additional information for applicants

# Environmentally relevant activities

The description of any environmentally relevant activity (ERA) for which an environmental authority (EA) is issued is a restatement of the ERA as defined by legislation at the time the EA is issued. Where there is any inconsistency between that description of an ERA and the conditions stated by an EA as to the scale, intensity or manner of carrying out an ERA, the conditions prevail to the extent of the inconsistency.

An EA authorises the carrying out of an ERA and does not authorise any environmental harm unless a condition stated by the EA specifically authorises environmental harm.

A person carrying out an ERA must also be a registered suitable operator under the Environmental Protection Act 1994 (EP Act).

# Mobile and temporary activities

If you operate a mobile and temporary environmentally relevant activity (ERA), other than regulated waste transport, you are required to maintain a work diary. You must:

- use the approved form for a work diary (ESR/2015/1696);

- keep the work diary records for 2 years after the last entry;

- inform the administering authority within 7 days of the work diary being lost or stolen;

- record the information required in the work diary for each location within 1 day of leaving the location.

# Contaminated land

It is a requirement of the EP Act that an owner or occupier of land give written notice to the administering authority if they become aware of the following:

- the presence of, or happening of an event involving, a hazardous contaminant on the land that is causing, or is reasonably likely to cause, serious or material environmental harm (notice must be given within 24 hours); or

- if the land is contaminated land – a change in the condition of the land that is causing, or is reasonably likely to cause, serious or material environmental harm (notice must be given within 24 hours); or

- a notifiable activity (as defined in Schedule 3) having been carried out, or is being carried out, on the land (notice must be given within 20 business days).

For further information, including the form for giving written notice, refer to the Queensland Government website www.qld.gov.au, using the search term ‘duty to notify’.

# Take effect

# Please note that, in accordance with section 200 of the EP Act, an EA has effect:

- if the authority is for a prescribed ERA and it states that it takes effect on the day nominated by the holder of the authority in a written notice given to the administering authority – on the nominated day; or

- if the authority states a day or an event for it to take effect – on the stated day or when the stated event happens; or

- otherwise – on the day the authority is issued.

However, if the EA is authorising an activity that requires an additional authorisation (a relevant tenure for a resource activity, a development permit under the Planning Act 2016 or an SDA Approval under the State Development and Public Works Organisation Act 1971), this EA will not take effect until the additional authorisation has taken effect.

If this EA takes effect when the additional authorisation takes effect, you must provide the administering authority written notice within 5 business days of receiving notification of the related additional authorisation taking effect.

The anniversary day of this environmental authority is the same day each year as the effective date. The payment of the annual fee will be due each year on this day. An annual return will be due each year on 01 April.

# If you have incorrectly claimed that an additional authorisation is not required, carrying out the ERA without the additional authorisation is not legal and could result in your prosecution for providing false or misleading information or operating without a valid environmental authority.

| Signature |  | Date |
| --- | --- | --- |
| Amelia Sellars Department of the Environment, Tourism, Science and Innovation Delegate of the administering authority Environmental Protection Act 1994 |  | Enquiries: Energy and Extractive Resources Business Centre GPO Box 2454, Brisbane QLD 4001 (07) 3330 5715 EnergyandExtractive@des.qld.gov.au |

Obligations under the Environmental Protection Act 1994

In addition to the requirements found in the conditions of this environmental authority, the holder must also meet their obligations under the EP Act, and the regulations made under the EP Act. For example, the holder must comply with the following provisions of the Act:

- general environmental duty (section 319)

- duty to notify environmental harm (section 320-320G)

- offence of causing serious or material environmental harm (sections 437-439)

- offence of causing environmental nuisance (section 440)

- offence of depositing prescribed water contaminants in waters and related matters (section 440ZG)

- offence to place contaminant where environmental harm or nuisance may be caused (section 443)

Other permits required

This permit only provides an approval under the Environmental Protection Act 1994. In order to lawfully operate you may also require permits / approvals from your local government authority, other business units within the department and other State Government agencies prior to commencing any activity at the site. For example, this may include permits / approvals with your local Council (for planning approval), the Department of Transport and Main Roads (to access State controlled roads), the Department of Resources (to clear vegetation), and the Department of Agriculture and Fisheries (to clear marine plants or to obtain a quarry material allocation).

Conditions of environmental authority

| General Conditions |  |
| --- | --- |
| Condition Number | Condition |
| General 1 | This environmental authority authorises the carrying out of the following resource activities: - The petroleum activities and specified relevant activities listed in General, Table 1 - Authorised Petroleum Activities to the extent they are carried out in accordance with the activity’s corresponding intensity or maximum number or both (where applicable); and - The following specified relevant activities: Resource Activity, Schedule 3 – 06: A petroleum activity carried out on a site containing a high hazard dam or a significant hazard dam; Resource Activity, Schedule 3 – 03: A petroleum activity that is likely to have a significant impact on a category A or B environmentally sensitive area. - The following specified environmentally relevant activities (ERA/s): - Schedule 2, Ancillary ERA 63 - Sewage treatment – operating sewage treatment works, other than no release works, with a total daily peak design capacity of (a-i) 21 to 100 EP if treated effluent is discharged from the works to an infiltration trench or through an irrigation scheme; - Incidental activities that are not otherwise specified relevant activities. General, Table 1 – Authorised Petroleum Activities Petroleum Activities and Infrastructure; Scale. Intensity (Extent); Maximum. Existing Wells; 6 ha See definition of Essential Petroleum Activities for well pad disturbance size (in hectares); 6 wells. Stage 1 Wells; 55 ha See definition of Essential Petroleum Activities for well pad disturbance size (in hectares); 55 wells. Stage 2 Wells; 55 wells. Existing and additional Water Monitoring Bores; 0.03 ha for shallow monitoring bores 1 ha for deep monitoring bores; 30. Borrow Pits; 3 ha; 6 borrow pits. Sediment Ponds; 0.76 ha; 2 sediment ponds. Hopeland Water Dam (regulated structure); 21 ha; 1 dam. |
| General 2 | This environmental authority does not authorise environmental harm unless a condition contained in this environmental authority explicitly authorises that harm. Where there is no condition, the lack of a condition shall not be construed as authorising harm. |
| General 7 | All monitoring must be undertaken by a suitably qualified person. |
| General 8 | If requested by the administering authority in relation to investigating a complaint, monitoring must be commenced within 10 business days. |
| General 9 | All laboratory analyses and tests must be undertaken by a laboratory that has NATA accreditation for such analyses and tests. |
| General 10 | Notwithstanding condition (General 9), where there are no NATA accredited laboratories for a specific analyte or substance, then duplicate samples must be sent to at least two separate laboratories for independent testing or evaluation. |
| General 11 | Monitoring and sampling must be carried out in accordance with the requirements of the following documents (as relevant to the sampling being undertaken), as amended from time to time: - for waters and aquatic environments, the Queensland Government’s Monitoring and Sampling Manual 2018 – Environmental Protection (Water and Wetland Biodiversity) Policy 2019; - for groundwater, Groundwater Sampling and Analysis – A Field Guide (2009:27 GeoCat #6890.1); - for noise, the Environmental Protection Regulation 2019; and - for dust, the relevant Australian standard. |
| General 12 | In addition to the requirements under Chapter 7, Part 1, Division 2 of the Environmental Protection Act 1994, the administering authority must be notified through the Pollution Hotline and in writing, as soon as possible, but within 48 hours of becoming aware of any of the following events: - any unauthorised significant disturbance to land - potential or actual loss of structural or hydraulic integrity of a dam - when the level of the contents of any regulated dam reaches the mandatory reporting level - when a regulated dam will not have available storage to meet the design storage allowance on 1 November of any year - likely or actual loss of well integrity - when the seepage trigger action response procedure required under condition (Water 29(g)) is or should be implemented - unauthorised releases of any volume of prescribed contaminants to waters - unauthorised releases of volumes of contaminants, in any mixture, to land greater than: - 200 L of hydrocarbons; or - 5 000 L of untreated coal seam gas water; or - 1 000 L of brine; or - 5 000 L of raw sewage; or - 10 000 L of treated sewage effluent. - monitoring results where two out of any five consecutive samples do not comply with the relevant limits in the environmental authority. |
| General 16 | Petroleum activities involving significant disturbance to land cannot commence until the development of written contingency procedures for emergency environmental incidents which include, but are not necessarily limited to: - a clear definition of what constitutes an environmental emergency incident or near miss for the petroleum activity. - consideration of the risks caused by the petroleum activity including the impact of flooding and other natural events on the petroleum activity. - response procedures to be implemented to prevent or minimise the risks of environmental harm occurring. - the practices and procedures to be employed to restore the environment or mitigate any environmental harm caused. - procedures to investigate causes and impacts including impact monitoring programs for releases to waters and/or land. - training of staff to enable them to effectively respond. - procedures to notify the administering authority, local government and any potentially impacted landholder. |
| General 17 | All plant and equipment must be maintained and operated in their proper and effective condition. |
| General 18 | The following infrastructure must be signed with a unique reference name or number in such a way that it is clearly observable: - regulated dams and low consequence dams - exploration, appraisal and development wells - sewage treatment facilities; and - any chemical storage facility associated with the environmentally relevant activity of chemical storage. |
| General 19 | Measures to prevent fauna being harmed from entrapment must be implemented during the construction and operation of well infrastructure, dams and pipeline trenches. |
| General 20 | For activities involving significant disturbance to land, control measures that are commensurate to the site-specific risk of erosion, and risk of sediment release to waters must be implemented to: - allow stormwater to pass through the site in a controlled manner and at non-erosive flow velocities; - minimise soil erosion resulting from wind, rain, and flowing water; - minimise the duration that disturbed soils are exposed to the erosive forces of wind, rain, and flowing water; - minimise work-related soil erosion and sediment runoff; and - minimise negative impacts to land or properties adjacent to the activities (including roads). |
| General 21 | Petroleum activities must not cause environmental nuisance at a sensitive place, other than where an alternative arrangement is in place. |
| General 22 | A certification must be prepared by a suitably qualified person within 30 business days of completing every plan, procedure, program and report required to be developed under this environmental authority, which demonstrates that: - relevant material, including current published guidelines (where available) have been considered in the written document; - the content of the written document is accurate and true; and - the document meets the requirements of the relevant conditions of the environmental authority. |
| General 23 | All plans, procedures, programs, reports and methodologies required under this environmental authority must be written and implemented. |
| General 24 | All documents required to be developed under this environmental authority must be kept for five years. |
| General 25 | All documents required to be prepared, held or kept under this environmental authority must be provided to the administering authority upon written request within the requested timeframe. |
| General 26 | A record of all complaints must be kept including the date, complainant’s details, source, reason for the complaint, description of investigations and actions undertaken in resolving the complaint. |
| Waste Conditions |  |
| Condition Number | Condition |
| Waste 1 | Measures must be implemented so that waste is managed in accordance with the waste and resource management hierarchy and the waste and resource management principles. |
| Waste 2 | Waste, including waste fluids, but excluding waste used in closed-loop systems, must be transported off-site for lawful re-use, remediation, recycling or disposal, unless the waste is specifically authorised by condition Waste 3, Waste 4, Waste 8 and Waste 16 to be disposed of or used on site. |
| Waste 3 | Waste fluids other than residual drilling material or drilling fluids stored in sumps, must be contained in either: - an above ground container; or - a structure which contains the wetting front. |
| Waste 4 | Green waste may be used on-site for either rehabilitation or sediment and erosion control, or both. |
| Waste 7 | Produced water may be re-used in: - drilling and well hole activities. |
| Waste 8 | Produced water may be used for dust suppression provided the following criteria are met: - the amount applied does not exceed the amount required to effectively suppress dust; and - the application: - does not cause on-site ponding or runoff; - is directly applied to the area being dust suppressed; - does not harm vegetation surrounding the area being dust suppressed; and - does not cause visible salting. |
| Waste 9 | Produced water may be used for construction purposes provided the use: - does not result in negative impacts on the composition and structure of soil or subsoils - is not directly or indirectly released to waters; - does not result in runoff from the construction site; and - does not harm vegetation surrounding the construction site. |
| Waste 10 | If there is any indication that any of the circumstances in condition (Waste 8)(b)(i) to (Waste 8(b)(iv)) or (Waste 9)(a) to (Waste 9)(d) is occurring the use must cease immediately and the affected area must be remediated without delay. |
| Waste 11 | Treated sewage effluent or greywater from a treatment system with a daily peak design capacity of less than 50EP may be released to land provided it: - is to a fenced and signed contaminant release area(s); - does not result in pooling or run-off or aerosols or spray drift or vegetation die-off; - minimises deep drainage below the root zone any vegetation; - does not adversely affect the quality of shallow aquifers; - does not adversely impact soil quality; and - is to a contaminant release area(s) that is kept vegetated with groundcover, that is: - not a declared pest species; - kept in a viable state for transpiration and nutrient uptake; and - grazed or harvested and removed from the contaminant release area as needed, but not less than every three months. |
| Waste 12 | If sumps are used to store residual drilling material or drilling fluids, they must only be used for the duration of drilling activities. |
| Waste 13 | Residual drilling material can only be disposed of on-site: - by mix-bury-cover method if the residual drilling material meets the approved quality criteria; or - if it is certified by a suitably qualified third party as being of acceptable quality for disposal to land by the proposed method and that environmental harm will not result from the proposed disposal. |
| Waste 14 | Records must be kept to demonstrate compliance with condition (Waste 12) and (Waste 13). |
| Noise Conditions |  |
| Condition Number | Condition |
| Noise 1 | Notwithstanding condition (General 21), emission of noise from the petroleum activity(ies) at levels less than those specified in Protecting acoustic values, Table 1—Noise nuisance limits are not considered to be environmental nuisance. Protecting acoustic values, Table 1—Noise nuisance limits Time Period; Metric; Short term noise event; Medium term noise event; Long term noise event. 7:00am-6:00pm; LAeq,adj,15 min; 45 dBA; 43 dBA; 40 dBA. 6:00pm-10:00pm; LAeq,adj,15 min; 40 dBA; 38 dBA; 35 dBA. 10:00pm-6:00am; LAeq,adj,15 min; 28 dBA; 28 dBA; 28 dBA. Max LpA, 15mins; 55 dBA; 55 dBA; 55 dBA. 6:00am-7:00am; LAeq,adj,15 min; 40 dBA; 38 dBA; 35 dBA. Drilling activities undertaken from 10:00pm-7:00am2; LAeq,adj,15 min; 28 dBA (measured indoors) 33 dBA (measured outdoors). 1 The noise limits in Protecting acoustic values, Table 1—Noise nuisance limits have been set based on the following deemed background noise levels (LABG): 7:00am—6:00pm: 35 dBA 6:00pm—10:00pm: 30 dBA 10:00pm—6:00am: 25 dBA 6:00am—7:00am: 30 dBA 2 Drilling activities undertaken from 10:00pm – 7:00am must be temporary and mobile in nature, and must not contribute to long-term background noise creep. |
| Noise 2 | If the noise subject to a valid complaint is tonal or impulsive, the adjustments detailed in Protecting acoustic values, Table 2—Adjustments to be added to noise levels at sensitive receptors are to be added to the measured noise level(s) to derive LAeq, adj, 15 min. Protecting acoustic values, Table 2—Adjustments to be added to noise levels at sensitive receptors Noise characteristic; Adjustment to noise. Tonal characteristic is just audible; +2 dBA. Tonal characteristic is clearly audible; +5 dBA. Impulsive characteristic is detectable; +2 to +5 dBA. |
| Noise 3 | Notwithstanding condition (Noise 1), emission of any low frequency noise must not exceed either (Noise 3(a)) and (Noise 3(b)), or (Noise 3(c)) and (Noise 3(d)) in the event of a valid complaint about low frequency noise being made to the administering authority: - 60 dB(C) measured outside the sensitive receptor; and - the difference between the external A-weighted and C-weighted noise levels is no greater than 20 dB; or - 50 dB(Z) measured inside the sensitive receptor; and - the difference between the internal A-weighted and Z-weighted (Max LpZ, 15 min) noise levels is no greater than 15 dB. |
| Air Conditions |  |
| Condition Number | Condition |
| Air 1 | Unless venting is authorised under the Petroleum and Gas (Production and Safety) Act 2004 or the Petroleum Act 1923, waste gas must be flared in a manner that complies with all of (Air 1(a)) and (Air 1(b)) and (Air 1(c)), or with (Air 1(d)): - an automatic ignition system is used, and - a flame is visible at all times while the waste gas is being flared, and - there are no visible smoke emissions other than for a total period of no more than 5 minutes in any 2 hours, or - it uses an enclosed flare. |
| Air 2 | Other than as permitted within this environmental authority, odours or airborne contaminants must not cause environmental harm to any sensitive place. |
| Air 3 | Petroleum activities must not cause an exceedance of the following dust and particulate matter concentrations at any sensitive place: - dust deposition of 120 milligrams per square metre per day, when monitored in accordance with Australian Standard AS 3580.10.1 (or more recent editions), or - a concentration of particulate matter with an aerodynamic diameter of less than 10 micrometre (μm) (PM10) suspended in the atmosphere of 50 micrograms per cubic metre over a 24-hour averaging time, when monitored in accordance with Australian Standard AS 3580.9.6 (or more recent editions) or any other method approved by the administering authority. |
| Land Conditions |  |
| Condition Number | Condition |
| Land 1 | Contaminants must not be directly or indirectly released to land except for those releases authorised by condition Waste 3, Waste 4, Waste 8, Waste 11 and Waste 16. |
| Land 2 | Top soil must be managed in a manner that preserves its biological and chemical properties. |
| Land 3 | Land that has been significantly disturbed by the petroleum activities must be managed to ensure that mass movement, gully erosion, rill erosion, sheet erosion and tunnel erosion do not occur on that land. |
| Land 5 | Chemicals and fuels stored, must be effectively contained and where relevant, meet Australian Standards, where such a standard is applicable. |
| Land 6 | Pipeline operation and maintenance must be in accordance, to the greatest practicable extent, with the relevant section of the APGA Code of Practice for Upstream Polyethylene Gathering Networks in the Coal Seam Gas Industry (2019 or latest version). |
| Land 7 | Pipeline trenches must be backfilled and topsoils reinstated within three months after pipe laying. |
| Land 8 | Reinstatement and revegetation of the pipeline right of way must commence within 6 months after cessation of petroleum activities for the purpose of pipeline construction. |
| Land 9 | Backfilled, reinstated and revegetated pipeline trenches and right of ways must be: - a stable landform; - re-profiled to a level consistent with surrounding soils; - re-profiled to original contours and established drainage lines; and - vegetated with groundcover which is not a prohibited matter, restricted matter species or weed of national significance, and which is established and growing. |
| Biodiversity Conditions |  |
| Condition Number | Condition |
| Biodiversity 1 | Prior to undertaking activities that result in significant disturbance to land in areas of native vegetation, confirmation of on-the-ground biodiversity values of the native vegetation communities at that location must be undertaken by a suitably qualified person. |
| Biodiversity 2 | A suitably qualified person must develop and certify a methodology so that condition (Biodiversity 1) can be complied with and which is appropriate to confirm on-the-ground biodiversity values. |
| Biodiversity 3 | For conditions (Biodiversity 4) to (Biodiversity 9), where mapped biodiversity values differ from those confirmed under conditions (Biodiversity 1) and (Biodiversity 2), petroleum activities may proceed in accordance with the conditions of the environmental authority based on the confirmed on-the-ground biodiversity value. |
| Biodiversity 4 | The location of the petroleum activity(ies) must be selected in accordance with the following site planning principles: - maximise the use of areas of pre-existing disturbance; - in order of preference, avoid, minimise or mitigate any impacts, including cumulative; impacts, on areas of native vegetation or other areas of ecological value; - minimise disturbance to land that may result in land degradation; - in order of preference, avoid then minimise isolation, fragmentation, edge effects or dissection of tracts of native vegetation; and - in order of preference, avoid then minimise clearing of native mature trees. |
| Biodiversity 5 | Linear infrastructure corridors must: - maximise co-location; - be minimised in width to the greatest practicable extent; and - for linear infrastructure that is an essential petroleum activity authorised in an environmentally sensitive area or its protection zone, be no greater than 40m in total width. |
| Biodiversity 7 | Despite condition (Biodiversity 8A), Extra Work Areas (EWAs) are authorised in areas clear of vegetation, including primary protection zone (PPZ) and secondary protection zone (SPZ). |
| Biodiversity 8A | Where petroleum activities are to be carried out in environmentally sensitive areas or their protection zones, the petroleum activities must be carried out in accordance with Protecting Biodiversity Values, Table 1— Authorised petroleum activities in environmentally sensitive areas and their protection zones. Protecting Biodiversity Values, Table 1—Authorised petroleum activities in environmentally sensitive areas and their protection zones Environmentally Sensitive Area; Within the Environmentally Sensitive Area; Primary protection zone of the Environmentally Sensitive Area; Secondary protection zone of the Environmentally Sensitive Area. Category A environmentally sensitive areas; No petroleum activities permitted.; Only low impact petroleum activities permitted.; Only essential petroleum activities permitted.. Category B environmentally sensitive areas that are other than ‘endangered' regional ecosystems; Only low impact petroleum activities permitted.; Only low impact petroleum activities permitted.; Only essential petroleum activities permitted.. Category B environmentally sensitive areas that are ‘endangered’ regional ecosystems; Only low impact petroleum activities permitted.; Only essential petroleum activities permitted.; Only essential petroleum activities permitted.. Category C environmentally sensitive areas that are ‘nature refuges’ or ‘koala habitat’; Only low impact petroleum activities permitted.; Only low impact petroleum activities permitted.. Category C environmentally sensitive areas that are ‘essential habitat’, ‘essential regrowth habitat’, or ‘of concern’ regional ecosystems; Only low impact petroleum activities permitted.; Only essential petroleum activities permitted.. Category C ESAs that are ‘protected wildlife habitat’; Only essential petroleum activities permitted.; Only essential petroleum activities permitted.. Category C environmentally sensitive areas that are ‘regional parks’ (previously known as ‘resources reserves’); Only essential petroleum activities permitted.; Only essential petroleum activities permitted.. Category C environmentally sensitive areas that are ‘state forests’ or ‘timber reserves’; Only essential petroleum activities permitted.; Petroleum activities permitted.. Areas of vegetation that are ‘critically limited’; Only low impact petroleum activities permitted.; Only essential petroleum activities permitted.. |
| Biodiversity 8B | Despite condition Biodiversity 8A, significant disturbance is authorised to be undertaken at the location and maximum extent of impact in Protecting Biodiversity Values, Table 2— Authorised petroleum activities in environmentally sensitive areas and their protection zones. Protecting Biodiversity Values, Table 2—Authorised petroleum activities in environmentally sensitive areas and their protection zones Environmentally Sensitive Areas; Location of impact; Maximum Disturbance (ha) (Stage 1); Maximum Disturbance (ha) (Stage 2). Category B environmentally sensitive areas that are 'endangered' regional ecosystems; PL253; 0.79; 0.41. Category C environmentally sensitive areas that are ‘essential habitat’, ‘essential regrowth habitat’, or ‘of concern’ regional ecosystems; PL253; 9.78; 0.78. Category C environmentally sensitive areas that are protected wildlife habitat*; PL253; 2.08; 3.41. Category B environmentally sensitive area primary protection zones; PL253; 0; To the extent they are required for authorised activities within the Hopeland Stage 2 maximum boundary, outlined in Condition Biodiversity 8C, Figure 1 – Stage 2 maximum boundary.. Category B environmentally sensitive area secondary protection zones; PL253; 0. Category C environmentally sensitive area primary protection zones; PL253; 0. *only applies to Stage 2 wells and infrastructure as Protected Wildlife Habitat was not an ESA at the time of authorisation of Stage 1. |
| Biodiversity 8C | Despite condition (Biodiversity 8A) and (Biodiversity 8B), petroleum activities are permitted in ESAs, as well as their primary protection zones (PPZs) and secondary protection zones (SPZs), if they satisfy the following: a) Do not exceed the maximum area for each environmentally sensitive area as shown in Biodiversity 8B, Table 2 – Authorised petroleum activities in environmentally sensitive areas and their protection zones; and b) are undertaken within the footprint prescribed in Biodiversity 8C, Figure 1 – Stage 2 Maximum Boundary. |

Figure 1 – Stage 2 Maximum Boundary

| Biodiversity 9 | A report must be prepared for each annual return period for all petroleum activities that involved clearing of any environmentally sensitive area or protection zone which includes: - records able to demonstrate compliance with conditions (Biodiversity 4), (Biodiversity 5), (Biodiversity 7) and (Biodiversity 8A and 8B); - a description of the work; - a description of the area and its pre-disturbance values (which may include maps or photographs, but must include GPS coordinates for the works); and - based on the extent of environmentally sensitive areas and primary protection zones on the relevant resource authority(ies), the proportion of native vegetation cleared per environmentally sensitive area and primary protection zone, including regional ecosystem type, over the annual return period. |
| --- | --- |
| Biodiversity 10 | Impacts to prescribed environmental matters are not authorised under this environmental authority or the Environmental Offsets Act 2014 unless the impact(s) is specified in Protecting biodiversity values, Table 3—Impacts to prescribed environmental matters. Protecting biodiversity values, Table 3—Impacts to prescribed environmental matters Prescribed Environmental Matter; Location of impact; Maximum extent of impact (ha) – Stage 1; SRI and Offsets required – Stage 1; Maximum extent of impact (ha) - Stage 2; SRI and Offsets required – Stage 2. REGULATED VEGETATION. Endangered regional ecosystem. RE 11.4.3; PL253; 0.75; Yes; 0; No. Of concern regional ecosystem. RE 11.3.4; PL253; 0.58; Yes; 0; No. Regional ecosystems (not within an urban area) within the defined distances from the defining banks of a relevant watercourse on the vegetation management watercourse map. RE 11.3.25 (BVG 16a); PL253; 0.41; Yes; 0.14; Yes. RE 11.5.1 (BVG 18b); PL253; 0.65; Yes; 0; No. RE 11.3.4 (BVG 16c); PL253; 0.02; Yes; 0; No. Essential habitat (not in an urban area) on the essential habitat map for endangered wildlife. Phascolarctos cinereus (Koala); PL253; 0; No; 0; No. Petaurus australis (Yellow-Bellied Glider - southern subspecies); PL253; 2.76; Yes; 0; No. Philotheca sporadica (Kogan Waxflower); PL253; 8.37; No; 0; No. Essential habitat (not in an urban area) on the essential habitat map for vulnerable wildlife. Jalmenus eubulus (Pale Imperial Hairstreak); PL253; 0.23; Yes; 0; No. CONNECTIVITY AREAS. Connectivity area that is a regional ecosystem (not in an urban area); PL253; 0; No; 0; No. PROTECTED WILDLIFE HABITAT. An area shown as a high risk area on the flora survey trigger map that contains plants that are endangered or vulnerable wildlife; PL253; In accordance with the protected plant clearing framework and relevant protected plant clearing permit under the Nature Conservation Act 1992. An area not shown as a high risk area on the flora survey trigger map that contains plants that are endangered or vulnerable wildlife; PL253; In accordance with the protected plant clearing framework and relevant protected plant clearing permit under the Nature Conservation Act 1992. Habitat for an animal that is endangered wildlife. Hemiaspis damelii (Grey snake); PL253; 17.65; Yes; 0.41; Yes. Phascolarctos cinereus (Koala); PL253; 66.84; Yes; 3.00; Yes. Petauroides volans (Greater Glider); PL253; 40.37; Yes; 0; No. Adclarkia cameroni (Brigalow Woodland Snail); PL253; N/A; No; 0.16; Yes. Habitat for an animal that is vulnerable wildlife. Acanthophis antarcticus (Common Death Adder); PL253; 40.65; Yes; 0; No. Calyptorhynchus lathami (Glossy Black Cockatoo); PL253; 3.92; Yes; 0; No. Jalmenus eubulus (Pale Imperial Hairstreak); PL253; 0.75; Yes; 0; No. Furina dunmalli (Dunmall’s Snake); PL253; 39.42; Yes; 0.41; Yes. Nyctophilus corbeni (South-Eastern Long-Eared Bat); PL253; 39.44; Yes; 0; No. Stagonpleura guttata (Diamond Firetail); PL253; N/A; No; 3.00; No. Aphelocephala leucopsis (Southern Whiteface); PL253; N/A; No; 3.00; No. Habitat for an animal that is special least concern wildlife. Tachyglossus aculeatus (Echidna); PL253; 3.75; Yes; 0; No. WATERWAY PROVIDING FOR FISH PASSAGE. Fish passage (not in an urban area); PL253 - Kogan Creek, Sixteen Mile Creek; 0.15; No; 0; No. Matter(s) of National Environmental Significance (MNES) have been prescribed and will be offset in accordance with the Environment Protection and Biodiversity Conservation Act 1999 (EPBC Act) Species Impact Management and Offset Plans, specifically the EPBC approval for the Surat Gas Project Environment Impact Statement (EPBC Approval 2010/5344, Tables 1 and 2). |
| Biodiversity 11 | Records demonstrating that each impact to a prescribed environmental matter not listed in Protecting biodiversity values, Table 3—Impacts to prescribed environmental matters did not, or is not likely to, result in a significant residual impact to that matter must be: - completed by an appropriately qualified person; and - kept for the life of the environmental authority. |
| Biodiversity 12 | An environmental offset made in accordance with the Environmental Offsets Act 2014 and Queensland Environmental Offsets Policy, as amended from time to time, must be undertaken for the maximum extent of impact to each prescribed environmental matter authorised in Protecting biodiversity values, Table 3—Impacts to prescribed environmental matters, unless a lesser extent of the impact has been approved in accordance with condition (Biodiversity 14). |
| Biodiversity 13 | The impacts to a prescribed environmental matter authorised in condition (Biodiversity 10) for which an environmental offset is required by condition (Biodiversity 12) may be carried out in stages. An environmental offset can be delivered for each stage of the impacts to prescribed environmental matters. |
| Biodiversity 16 | A notice of election for the staged environmental offset referred to in condition (Biodiversity 15), if applicable, must be provided to the administering authority no less than three months before the proposed commencement of that stage, unless a lesser timeframe has been agreed to by the administering authority. |
| Biodiversity 17 | Within six months from the completion of the final stage of the project, a report completed by an appropriately qualified person, that includes the following matters must be provided to the administering authority: - an analysis of the actual impacts on prescribed environmental matters resulting from the final stage; and - if applicable, a notice of election to address any outstanding offset debits for the authorised impacts. |
| Biodiversity Conditions |  |
| Condition Number | Condition |
| Water 1 | Prior to the commencement of drilling for Stage 1 Activities the holder must: - In accordance with condition Water 12, obtain 12 months’ worth of monitoring results for new groundwater monitoring wells required by condition Water 5; - Incorporate the results from condition Water 1(1) into the relevant groundwater model in condition Water 2; - An appropriately qualified hydrogeologist must prepare an interim review of the annual report in accordance with the requirements set out in condition Water 9 and submit it to the administering authority; and - Develop a Groundwater Monitoring Plan (GMP) in accordance with the requirements set out in condition Water 2 and submit it to the administering authority. |
| Water 2 | The GMP must be prepared by an appropriately qualified hydrogeologist and include, but not be limited to: - An assessment of the potential for the activity to impact on groundwater flow and pressure within the Springbok Sandstone and the Macalister coal seam (and its relevant aquifers) within the Walloon Coal Measures; - Site conceptual model of Lot 40 on DY85; - A groundwater model that targets aquifers on and in the vicinity of Lot 40 on DY85 including the Springbok Sandstone and the Macalister coal seam (and its relevant aquifers) of the Walloon Coal Measures that will measure and predict changes in contaminant movement, groundwater flow direction and drawdown; - A monitoring program for the early detection of any changes in groundwater pressure, groundwater level, bore gas pressure, groundwater flow direction, and groundwater quality in all aquifers monitored by groundwater monitoring bores included in Water Conditions, Table 1 – Groundwater Monitoring Locations and the section of the Walloon Coal Measure targeted for coal seam gas extraction; - Details (location, depth and aquifer) of all existing, replacement and proposed groundwater monitoring bores; and - Details of notification and investigation procedures in accordance with conditions Water 14 to Water 17. |
| Water 3 | The site conceptualisation model in condition Water 2(2) must: - Characterise the residual contaminant source of Lot 40 on DY85 using: - Information available on Lot 40 DY85 including existing wells both on and off Lot 40DY85; and - Information obtained under condition Water 5 and condition Water 7; - Identify mobilisation pathways and present the groundwater pressure and flow regime around Lot 40 on DY85; - Provide an analysis of how contaminants may migrate over time with and without Stage 1 Activities; - Provide uncertainties in conceptualisation resulting from formation heterogeneity, preferential pathways and structural influences; and - Assess the potential for long-term release of contaminants in the groundwater. |
| Water 4 | The relevant groundwater model in condition Water 2(3) must: - Be calibrated annually (at a minimum) and updated with the data collected as part of the GMP required under condition Water 2(4) and the site conceptualisation model under condition Water 3 to inform the Annual Report under condition Water 9; - Include a simulation period to at least the year 2200 that extends to beyond when groundwater levels are expected to have recovered in 2040 and contaminant transport off-site becomes possible; - Include a contaminant transport model that: - uses the most recent water quality data for validation; and - has dual porosity or permeability capability (fracture and matrix) to better represent transport into and out of dead pore volumes and fractures that might contain residual sources of contaminants; - Include a review and assessment of the density of particle tracking; - Include particle starting locations of: - multiple particles per cell, both horizontally and vertically; - multiple particles located on each edge of Lot40 DY85; - multiple particles on the northern edge of gasifier 5; - multiple particles at each groundwater monitoring well location including HSMBS1-HSMBS4; - Assess the effect of particle tracking starting in wells that have exhibited elevated Underground Coal Gasification contaminants; - Assess particle tracks (travel times, distances and trajectories) to inform where to best install new groundwater monitoring wells required by condition Water 5 and condition Water 11; - Include a sensitivity analysis and uncertainty analysis; - Analyse the prior and posterior parameter distributions to provide insight into which parameters are constrained and which are not, and to compare the calibrated parameter set with the ensemble; - Provide correlation coefficients, standard error of estimate, 95% upper and lower limit for the parameters; - Initial concentration of component species in water; - Longitudinal dispersivity; - Transverse dispersivity; - Bulk density; - Adsorption coefficient; - Freundlich adsorption isotherm; - First order decay coefficient in water and soil. - Describe impacts of the authorised activities relative to the Quantity of Interest; - Discuss the value of using non-reactive chemicals as tracers to improve solute transport calibration; - Discuss characterisation of the initial contamination source term; - Discuss how model parameter values near the gasifiers are influenced by observed potentiometric data and any bias in parameter estimates introduced by calibrating against recovery head observations without accounting for possible dual phase flow; - Resolve inconsistencies in the number of coal seam gas wells between the amended EA and groundwater modelling report; and - Resolve inconsistencies between modelled development scenarios and document assumptions and rationale to support the selected model simulation period. |
| Water 5 | Within 12 months, 4 new groundwater monitoring wells must be drilled in the Springbok Sandstone at locations informed by particle tracking (travel times, distances and trajectories), and in the vicinity of: - south of HSMB1S; - east of HSMB2S; - west of HSMB4S; and - north of HSMB3S2. |
| Water 6 | The wells under condition Water 5, are subject to the monitoring, reporting and investigations requirements in conditions Water 11 to Water 18 upon commissioning of each well. |
| Water 7 | The wells under condition Water 5 must have core samples taken and an analysis completed for: - residual contamination; and - presence of condensates and other compounds that might release benzene over time. |
| Water 8 | By 1 December of each year, an Annual Report prepared by an appropriately qualified hydrogeologist must be submitted to the administering authority. |
| Water 9 | The Annual Report in condition Water 8 must: - Summarise and analyse all results from field testing, sampling and analysis data collected in accordance with condition Water 2(4), as well as graphical (groundwater contour plots) and tabulated presentation of the data to demonstrate data trends; - Discuss the methodology for sampling and reporting groundwater methane data and any potential impact on data analysis; - Assess both long-term and short-term data trends including but not limited to contaminant movement, groundwater flow direction and drawdown; - Summarise and analyse predicted impacts, including but not limited to excessive drawdown and likelihood of wells becoming dry as a result of groundwater drawdown; - Assess whether the requirements in conditions Water 2, Water 3 and Water 4 are being complied with; - Recommend improvements or changes to the program to ensure the GMP complies with condition Water 2, including: - Submission of any amendments to the administering authority for review and comment; and - Incorporating comments from the administering authority. - Provide details of any recent impact assessments from the Arrow Full Field Development Plan scenarios showing the long-term impact on drawdown in the relevant region; and - Include the Peer Review Report in condition Water 10. |
| Water 10 | The Annual Report in condition Water 8 must be peer reviewed by an independent suitably qualified expert who must: - Assess whether the findings in the Annual Report are accurate; - Assess whether the groundwater modelling undertaken complies with condition Water 2(3) and condition Water 4; - Recommend improvements or changes to the program to ensure the GMP complies with condition Water 2; and - Produce a report (the Peer Review Report) detailing the outcomes of the peer review. |
| Water 11 | Any monitoring bores identified in Water conditions, Table 1 – Groundwater Monitoring Locations that are or predicted to be dewatered, plugged and abandoned or can no longer be accessed, must be either drilled/screened deeper or replaced within six (6) months of submission of the Annual Report each year. |
| Water 12 | Monitoring and reporting All groundwater monitoring points identified in condition Water 2(4), must be monitored quarterly (at a minimum) at the locations listed in Water conditions, Table 1 – Groundwater Monitoring Locations and for the parameters and chemical groups identified in Water Conditions, Table 2 – Groundwater Parameters and Water Conditions, Table 3 – Investigation Limits. |
| Water 13 | Prior to groundwater monitoring in accordance with condition Water 12, the well integrity of each bore must be monitored for the potential of fugitive gas emissions from the area immediately surrounding each monitoring bore. |

Water conditions, Table 1 – Groundwater Monitoring Locations

| Monitoring Bore | Coordinates |  | Target Formation | Depth (mbgl) |
| --- | --- | --- | --- | --- |
|  | Latitude | Longitude |  |  |
| On-site Lot 40 DY85 |  |  |  |  |
| HSMB1S (RN 172597) | -26.92481589 | 150.68201871 | Springbok Sandstone | 123.5 |
| HSMB1D (RN 172598) | -26.92516937 | 150.68211216 | Macalister coal seam | 162.1 |
| HSMB2S (RN 172596) | -26.93190354 | 150.68223621 | Springbok Sandstone | 127.54 |
| HSMB2D (RN 172604) | -26.9331186 | 150.68204019 | Macalister coal seam | 164.47 |
| HSMB3S1 (RN 172600) | -26.9220583 | 150.67457499 | Springbok Sandstone | 133.46 |
| HSMB3D1 (RN 172599) | -26.92219531 | 150.67577018 | Macalister coal seam | 148.25 |
| HSMB3S2 (RN 172605) | -26.92177379 | 150.6715506 | Springbok Sandstone | 139.54 |
| HSMB3D2 (RN 172601) | -26.9218484 | 150.6722437 | Macalister coal seam | 163.22 |
| HSMB4S (RN 172595) | -26.92776613 | 150.66828637 | Springbok Sandstone | 124.39 |
| HSMB4D (RN 172602) | -26.92659637 | 150.66849166 | Macalister coal seam | 156.34 |
| HSMB5D (RN 172603) | -26.9319377 | 150.67116057 | Macalister coal seam | 162.3 |
| NB01D | -26.9230946 | 150.6733920 | Macalister coal seam | 155.06 |
| NB01S | -26.9229506 | 150.6734110 | Springbok Sandstone | 120.1 |
| NB02D | -26.9238223 | 150.6710016 | Macalister coal seam | 139.4 |
| NB02S | -26.9239661 | 150.6709690 | Springbok Sandstone | 120.9 |
| HSMB6D2 (RN 180099) | -26.917717 | 150.671382 | Macalister Coal Seam | 187.0 |
| HSMB6S (RN 180101) | -26.917535 | 150.671403 | Springbok Sandstone | 126.55 |
| NB04D(S) | -26.92935598 | 150.676414437 | Springbok Sandstone | 167.4 |
| NB05S | -26.9320806 | 150.6745252 | Springbok Sandstone | 123.4 |
| Off-site Lot 40 DY85 |  |  |  |  |
| HSMB7D (RN 180100) | -26.931049 | 150.661133 | Macalister coal seam | 187.25 |
| HSMB7S (RN 180102) | -26.931019 | 150.660937 | Springbok Sandstone | 128.75 |
| Hopeland 20 (RN 180107) | -26.923571 | 150.699995 | Macalister coal seam | 115.3 |
| Hopeland 21 (RN 180108 | -26.923550 | 150.699838 | Wambo Seam Package | 148.36 |
| Hopeland 22 (RN 108109) | -26.932776 | 150.668254 | Springbok Sandstone Lower | 104.03 |
| Hopeland 23 (RN 180110) | -26.932756 | 150.668104 | Macalister coal seam | 148.71 |
| Hopeland 24 (RN 180111) | -26.932735 | 150.667954 | Wambo Seam Package | 176.2 |
| Hopeland 25 (RN 180112) | -26.925521 | 150.667447 | Springbok Sandstone Lower | 99.04 |
| Hopeland 26 (RN 180113) | -26.925507 | 150.667296 | Macalister coal seam | 138.93 |
| Hopeland 27 (RN 180114) | -26.925493 | 150.667145 | Wambo Seam Package | 162.83 |
| Existing Landholder Bores |  |  |  |  |
| RN 147004 | -26.930542 | 150.708260 | Springbok Sandstone | 80.5 |
| RN 160158 | -26.944361 | 150.718290 | Macalister coal seam | 154.0 |

Water Conditions, Table 2 - Groundwater Parameters

| Parameter | Measurement / Analyte |
| --- | --- |
| Groundwater level | Water level |
| Groundwater pressure | Water and gas pressure |
| Physicochemical field parameters | pH |
|  | Electrical Conductivity (EC) |
|  | Dissolved oxygen (DO) |
|  | Temperature |
|  | Oxidation Reduction Potential (ORP) |
| Inorganic parameters | Total Dissolved Solids (TDS) |
|  | Major ions (Ca, Mg, Na, K, F, Cl, SO4, alkalinity) and hardness (calculated) |
|  | Ammonia |
|  | Total cyanide, weak acid dissociable, cyanate and thiocyanate |
| Total and dissolved metals | Arsenic |
|  | Barium |
|  | Beryllium |
|  | Boron |
|  | Cadmium |
|  | Chromium VI |
|  | Chromium (total) |
|  | Cobalt |
|  | Copper |
|  | Mercury |
|  | Manganese |
|  | Nickel |
|  | Lead |
|  | Lithium |
|  | Selenium |
|  | Vanadium |
|  | Zinc |
| Dissolved gases | Methane |
|  | Unionised hydrogen sulfide |
| Organic compounds | Total organic carbon (TOC) |
|  | Total recoverable hydrocarbons (TRH) |

Water Conditions, Table 3 – Investigation Limits

| Chemical Group | Investigation Limits (mg/L) |
| --- | --- |
| Total Phenolic compounds | >Limit of Reporting (LOR) |
| Polynuclear aromatic hydrocarbons (PAHs) including Naphthalene | >LOR |
| Total recoverable hydrocarbons (TRH) (Silica Gel cleanup) | >LOR |
| Cyanide | 0.08 |
| Benzene | 0.001 |
| Toluene | 0.8 |
| Ethylbenzene | 0.3 |
| Xylene | 0.6 |

| Water 14 | Exceedance Investigation If groundwater quality monitoring results exceed any of the investigation limits specified in Water Conditions, Table 3 – Investigation Limits, the holder must notify the administering authority within 48 hours of receiving the results and complete an investigation in accordance with condition Water 15. |
| --- | --- |
| Water 15 | If required by conditions Water 14 or Water 18 of this environmental authority, the holder must: - Complete an investigation into the potential for environmental harm and provide a written report to the administering authority, outlining: - Results and interpretation of any samples taken and analysed; - An analysis of the trends in groundwater quality and groundwater contours, and the direction of groundwater flow; - An analysis of the holder’s contribution to changes in groundwater conditions; - Outcomes of actions taken at the time to prevent or minimise environmental harm; and - Proposed actions to prevent a recurrence of the exceedance. |
| Water 15A | The holder must provide the report required by condition Water 15 to the administering authority within 10 business days of either: - receiving the result, if the investigation is required by condition Water 14; or - receiving the request, if the investigation is required by condition Water 18. |
| Water 16 | If an investigation in accordance with condition Water 15 for any of the parameters specified in Water Conditions, Table 3 – Investigation Limits, has been completed, a repeat exceedance investigation report in accordance with condition Water 15 is not required to be prepared within a twelve month period unless the change in the groundwater quality monitoring result is more than twice the value reported in the initial investigation. |
| Water 17 | Production of water authorised by Stage 1 Activities must immediately cease if the report required in accordance with condition Water 15 fails to demonstrate that the activity is not impacting the migration of groundwater contaminants from Lot 40 DY85. |
| Water 17A | The holder must notify the administering authority within 24 hours of ceasing the production of water authorised by Stage 1 Activities as required by condition Water 17. |
| Water 18 | Other Investigations The holder must undertake an investigation in accordance with condition Water 15 at the request of the administering authority. |
| Water 19 | The extraction of groundwater as part of the petroleum activity(ies) from underground aquifers must not directly or indirectly cause environmental harm to a wetland. |
| Water 20 | Petroleum activities must not occur in or within 200m of a: - wetland of high ecological significance adjacent land use(s); - Great Artesian Basin Spring; - subterranean cave groundwater dependent ecosystem (GDE). |
| Water 21 | Only construction or maintenance of linear infrastructure is permitted in or within any wetland of other environmental value or in a watercourse. |
| Water 22 | The construction or maintenance of linear infrastructure in a wetland of other environmental value must not result in the: - clearing of riparian vegetation outside of the minimum area practicable to carry out the works; or - ingress of saline water into freshwater aquifers; or - draining or filling of the wetland beyond the minimum area practicable to carry out the works. |
| Water 23 | After the construction or maintenance works for linear infrastructure in a wetland of other environmental value are completed, the linear infrastructure must not: - drain or fill the wetland; - prohibit the flow of surface water in or out of the wetland; - lower or raise the water table and hydrostatic pressure outside the bounds of natural variability that existed before the activities commenced; - result in ongoing negative impacts to water quality; - result in bank instability; or - result in fauna ceasing to use adjacent areas for habitat, feeding, roosting or nesting. |
| Water 24 | The construction or maintenance of linear infrastructure activities in a watercourse must be conducted in the following preferential order: - firstly, in times where there is no water present; - secondly, in times of no flow; - thirdly, in times of flow, providing a bankfull situation is not expected and that flow is maintained. |
| Water 25 | The construction or maintenance of linear infrastructure authorised under condition (Water 21) must comply with the water quality limits as specified in Protecting water values, Table 1—Release limits for construction or maintenance of linear infrastructure. Protecting water values, Table 1—Release limits for construction or maintenance of linear infrastructure Water quality parameters; Units; Water quality limits. Turbidity; Nephelometric Turbidity Units (NTU); For a wetland of other environmental value, if background water turbidity is above 45 NTU, no greater than 25% above background water turbidity measured within a 50m radius of the construction or maintenance activity. For a watercourse, if background water turbidity is above 45 NTU, no greater than 25% above background water turbidity measured within 50m downstream of the construction or maintenance activity.. For a wetland of other environmental value, if background water turbidity is equal to, or below 45 NTU, a turbidity limit of no greater than 55 NTU applies, measured within a 50m radius of the construction or maintenance activity. For a watercourse, if background water turbidity is equal to, or below 45 NTU, a turbidity limit of no greater than 55 NTU applies, measured within 50m downstream of the construction or maintenance activity.. Hydrocarbons; -; For a wetland of other environmental value, or watercourse, no visible sheen or slick.. |
| Water 26 | Monitoring must be undertaken at a frequency that is appropriate to demonstrate compliance with condition Water 25. |
| Water 27 | Register of activities in wetlands and watercourses A register must be kept of all linear infrastructure construction and maintenance activities in a wetland of other environmental value and watercourses, which must include: - location of the activity (e.g. GPS coordinates (GDA94) and watercourse name); - estimated flow rate of surface water at the time of the activity; - duration of works, and - results of impact monitoring carried out under condition Water 26. |
| Water 28 | A seepage monitoring program must be developed by a suitably qualified person which is commensurate with the site-specific risks of contaminant seepage from containment facilities, and which requires and plans for detection of any seepage of contaminants to groundwater as a result of storing contaminants. |
| Water 29 | The seepage monitoring program required by condition Water 28 must include but not necessarily be limited to: - identification of the containment facilities for which seepage will be monitored; - identification of trigger parameters that are associated with the potential or actual contaminants held in the containment facilities; - identification of trigger concentration levels that are suitable for early detection of contaminant releases at the containment facilities; - installation of background seepage monitoring bores where groundwater quality will not have been affected by the petroleum activities authorised under this environmental authority to use as reference sites for determining impacts; - installation of seepage monitoring bores that: - are within formations potentially affected by the containment facilities authorised under this environmental authority (i.e. within the potential area of impact); - provide for the early detection of negative impacts prior to reaching groundwater dependent ecosystems, landholder’s active groundwater bores, or water supply bores; - provide for the early detection of negative impacts prior to reaching migration pathways to other formations (i.e. faults, areas of unconformities known to connect two or more formations); - monitoring of groundwater at each background and seepage monitoring bore at least annually for the trigger parameters identified in condition Water 29(b); - seepage trigger action response procedures for when trigger parameters and trigger levels identified in conditions Water 29(b) and Water 29(c) trigger the early detection of seepage, or upon becoming aware of any monitoring results that indicate potential groundwater contamination; - a rationale detailing the program conceptualisation including assumptions, determinations, monitoring equipment, sampling methods and data analysis; and - provides for annual updates to the program for new containment facilities constructed in each annual return period. |
| Water 30 | A bore drill log must be completed for each seepage monitoring bore in condition Water 29 which must include: - bore identification reference and geographical coordinate location; - specific construction information including but not limited to depth of bore, depth and length of casing, depth and length of screening and bore sealing details; - standing groundwater level and water quality parameters including physical parameter and results of laboratory analysis for the possible trigger parameters; - lithological data, preferably a stratigraphic interpretation to identify the important features including the identification of any aquifers; and - target formation of the bore. |
| Rehabilitation Conditions |  |
| Condition Number | Condition |
| Rehabilitation 1 | A Rehabilitation Plan must be developed by a suitably qualified person and must include the: - rehabilitation goals; and - procedures to be undertaken for rehabilitation that will: - achieve the requirements of conditions (Rehabilitation 2) to (Rehabilitation 8), inclusive; and - provide for appropriate monitoring and maintenance. |
| Rehabilitation 2 | Significantly disturbed areas that are no longer required for the on-going petroleum activities, must be rehabilitated within 12 months (unless an exceptional circumstance in the area to be rehabilitated (e.g. a flood event) prevents this timeframe being met) and be maintained to meet the following acceptance criteria: - contaminated land resulting from petroleum activities is remediated and rehabilitated; - the areas are: - non-polluting; - a stable landform; - re-profiled to contours consistent with the surrounding landform; - surface drainage lines are re-established consistent with natural flow patterns and self-sustaining; - top soil is reinstated; and - either: - groundcover, that is not a prohibited matter, restricted matter species or weed of national significance, is growing; or - an alternative soil stabilisation methodology that achieves effective stabilisation is implemented and maintained. |
| Rehabilitation 3 | All significantly disturbed areas caused by petroleum activities which are not being or intended to be utilised by the landholder or overlapping tenure holder, must be rehabilitated to meet the following final acceptance criteria measured either against the highest ecological value adjacent land use or the pre-disturbed land use: - greater than or equal to 70% of native ground cover species richness; - greater than or equal to the total per cent of ground cover; - less than or equal to the per cent species richness of prohibited matter, restricted matter species or weed of national significance; and - where the adjacent land use contains, or the pre-clearing land use contained, one or more regional ecosystem(s), then at least one regional ecosystem(s) from the same broad vegetation group, and with the equivalent biodiversity status or a biodiversity status with a higher conservation value as any of the regional ecosystem(s) in either the adjacent land or pre-disturbed land, must be present. |
| Rehabilitation 4 | Where significant disturbance to land has occurred in an environmentally sensitive area, the following final rehabilitation criteria as measured against the pre-disturbance biodiversity values assessment (required by conditions (Biodiversity 1) and (Biodiversity 2)) must be met: - greater than or equal to 70% of native ground cover species richness; - greater than or equal to the total per cent ground cover; - less than or equal to the per cent species richness of prohibited matter, restricted matter species or weed of national significance; - greater than or equal to 50% of organic litter cover; - greater than or equal to 50% of total density of coarse woody material; and - all predominant species in the ecologically dominant layer, that define the pre disturbance regional ecosystem(s) are present. |
| Rehabilitation 5 | Conditions (Rehabilitation 2) and (Rehabilitation 3) continue to apply after this environmental authority has ended or ceased to have effect. |
| Rehabilitation 6 | Where there is a dam (including a low consequence dam) that is being or intended to be utilised by the landholder or overlapping tenure holder, the dam must be decommissioned to no longer accept inflow from the petroleum activity(ies) and the contained water must be of a quality suitable for the intended on-going uses(s) by the landholder or overlapping tenure holder. |
| Well Construction, Maintenance and Stimulation Activities Conditions |  |
| Condition Number | Condition |
| Well activities 1 | Oil based or synthetic based drilling muds must not be used in the carrying out of the petroleum activity(ies). |
| Well activities 3 | Practices and procedures must be in place to detect, as soon as practicable, any fractures that have or may result in the connection of a target formation and another aquifer as a result of drilling activities. |
| Well activities 4 | Stimulation activities are not permitted. |
| Regulated Dam Conditions |  |
| Condition Number | Condition |
| Dams 1 | Assessment of Consequence Category The consequence category of any structure must be assessed by a suitably qualified and experienced person in accordance with the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933) at the following times: - prior to the design and construction of the structure, if it is not an existing structure; or - prior to any change in its purpose or the nature of its stored contents. |
| Dams 2 | A consequence assessment report and certification must be prepared for each structure assessed and the report may include a consequence assessment for more than one structure. |
| Dams 3 | Certification must be provided by the suitably qualified and experienced person who undertook the assessment, in the form set out in the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933). |
| Dams 4 | Operation of a regulated structure For existing structures that are regulated structures: - where the existing structure that is a regulated structure is to be managed as part of an integrated containment system for the purpose of sharing the DSA volume across the system, the holder must submit to the administering authority within 12 months of the commencement of this condition a copy of the certified system design plan including that structure; and - there must be a current operational plan for the existing structures. |
| Dams 5 | Each regulated structure must be maintained and operated, for the duration of its operational life until decommissioned and rehabilitated, in compliance with the current operational plan and, if applicable, the current design plan and associated certified ‘as constructed’ drawings. |
| Dams 6 | Mandatory reporting level Conditions Dams 7 to Dams 8 inclusive only apply to Regulated Structures which have not been certified as low consequence category for ‘failure to contain – overtopping’. |
| Dams 7 | The Mandatory Reporting Level (the MRL) must be marked on a regulated dam in such a way that during routine inspections of that dam, it is clearly observable. |
| Dams 8 | The holder must, as soon as practicable but within forty-eight (48) hours of becoming aware, notify the administering authority when the level of the contents of a regulated dam reaches the MRL. |
| Dams 9 | The holder must, immediately on becoming aware that the MRL has been reached, act to prevent the occurrence of any unauthorised discharge from the regulated dam. |
| Dams 10 | The holder must record any changes to the MRL in the Register of Regulated Structures. |
| Dams 11 | Design storage allowance The holder must assess the performance of each regulated dam or linked containment system over the preceding November to May period based on actual observations of the available storage in each regulated dam or linked containment system taken prior to 1 July of each year. |
| Dams 12 | By 1 November of each year, storage capacity must be available in each regulated dam (or network of linked containment systems with a shared DSA volume), to meet the Design Storage Allowance (DSA) volume for the dam (or network of linked containment systems). |
| Dams 13 | The holder must, as soon as practicable but within forty-eight (48) hours of becoming aware that the regulated dam (or network of linked containment systems) will not have the available storage to meet the DSA volume on 1 November of any year, notify the administering authority. |
| Dams 14 | The holder must, immediately on becoming aware that a regulated dam (or network of linked containment systems) will not have the available storage to meet the DSA volume on 1 November of any year, act to prevent the occurrence of any unauthorised discharge from the regulated dam or linked containment systems. |
| Dams 15 | Annual inspection report Each regulated structure must be inspected each calendar year by a suitably qualified and experienced person. |
| Dams 16 | At each annual inspection, the condition and adequacy of all components of the regulated structure must be assessed and a suitably qualified and experienced person must prepare an annual inspection report containing details of the assessment and include a recommendations section, with any recommended actions to ensure the integrity of the regulated structure or a positive statement that no recommendations are required. |
| Dams 17 | The suitably qualified and experienced person who prepared the annual inspection report must certify the report in accordance with the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933). |
| Dams 18 | The holder must within 20 business days of receipt of the annual inspection report, provide to the administering authority: - The recommendations section of the annual inspection report; and - If applicable, any actions being taken in response to those recommendations; and - If, following receipt of the recommendations and (if applicable) recommended actions, the administering authority requests a copy of the annual inspection report from the holder, provide this to the administering authority within 10 business days of receipt of the request. |
| Dams 19 | Transfer arrangements The holder must provide a copy of any reports, documentation and certifications prepared under this authority, including but not limited to any Register of Regulated Structures, consequence assessment, design plan and other supporting documentation, to a new holder on transfer of this authority. |
| Dams 20 | Register of Regulated Structures A Register of Regulated Structures must be established and maintained by the holder for each regulated structure. |
| Dams 21 | The holder must provisionally enter the required information in the Register of Regulated Structures when a design plan for a regulated dam is submitted to the administering authority. |
| Dams 22 | The holder must make a final entry of the required information in the Register of Regulated Structures once compliance with Dams 4 has been achieved. |
| Dams 23 | The holder must ensure that the information contained in the Register of Regulated Structures is current and complete on any given day. |
| Dams 24 | All entries in the Register of Regulated Structures must be approved by the chief executive officer for the holder of this authority, or their delegate, as being accurate and correct. |
| Dams 25 | The holder must, at the same time as providing the annual return, supply to the administering authority a copy of the records contained in the Register of Regulated Structures, in the electronic format required by the administering authority. |
| Dams 26 | Transitional arrangements All existing structures that have not been assessed in accordance with either the Manual or the former Manual for Assessing Hazard Categories and Hydraulic Performance of Dams must be assessed and certified in accordance with the Manual within 6 months of amendment of the authority adopting this schedule. |
| Dams 27 | All existing structures must subsequently comply with the timetable for any further assessments in accordance with the Manual specified in Table 1 (Transitional hydraulic performance requirements for existing structures), depending on the consequence category for each existing structure assessed in the most recent previous certification for that structure. |
| Dams 28 | Table 1 ceases to apply for a structure once any of the following events has occurred: - it has been brought into compliance with the hydraulic performance criteria applicable to the structure under the Manual; or - it has been decommissioned; or - it has been certified as no longer being assessed as a regulated structure. |
| Dams 29 | Certification of the transitional assessment required by Dams 26 and Dams 27 (as applicable) must be provided to the administering authority within 6 months of amendment of the authority adopting this schedule. Regulated Dam conditions, Table 1 – Transitional hydraulic performance requirements for existing structures Transition period required for existing structures to achieve the requirements of the Manual for Assessing Consequence Categories and Hydraulic Performance of Dams. Compliance with criteria; High consequence; Significant consequence; Low consequence. >90% and a history of good compliance performance in last 5 years; No transition required; No transition required; No transitional conditions apply. Review consequence assessment every 7 years.. >70%-≤90%; Within 7 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases.; Within 10 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases.; No transitional conditions apply. Review consequence assessment every 7 years.. ˃50-≤70%; Within 5 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases.; Within 7 years, unless otherwise agreed with the administering authority, based on no history of unauthorised releases.; Review consequence assessment every 7 years.. ≤50%; Within 5 years or as per compliance requirements (e.g. TEP timing).; Within 5 years or as per compliance requirements (e.g. TEP timing).; Review consequence assessment every 5 years.. Regulated levee designed to prevent the ingress of clean flood water <100% compliant1; Within 5 years, unless otherwise agreed with the administering authority.. 1 Levees designed for the diversion of contaminated waters or protection of the structural integrity of a dam are not to be considered as part of this provision. These levees are considered a key design element of the relevant dam and transitional periods should as such align to that relevant compliance criteria and consequence category. |

## Definitions

Key terms and/or phrases used in this document are defined in this section. Where a term is not defined, the definition in the Environmental Protection Act 1994, its regulations or environmental protection policies must be used. If a word remains undefined it has its ordinary meaning.

| Term | Definition |
| --- | --- |
| acid sulfate soil(s) | means a soil or soil horizon which contains sulfides or an acid soil horizon affected by oxidation of sulfides. |
| administering authority | means: - for a matter, the administration and enforcement of which has been devolved to a local government under section 514 of the Environmental Protection Act 1994—the local government; or - for all other matters—the Chief Executive of the Department of Environment and Science; or - another State Government Department, Authority, Storage Operator, Board or Trust, whose role is to administer provisions under other enacted legislation. |
| alternative arrangement | means a written agreement about the way in which a particular environmental nuisance impact will be dealt with at a sensitive place, and may include an agreed period of time for which the arrangement is in place. An alternative arrangement may include, but is not limited to, a range of nuisance abatement measures to be installed at the sensitive place, or provision of alternative accommodation for the duration of the relevant nuisance impact. |
| analogue site(s) | means an area of land which contains values and characteristics representative of an area to be rehabilitated prior to disturbance. Such values must encompass land use, topographic, soil, vegetation, vegetation community attributes and other ecological characteristics. Analogue sites can be the pre-disturbed site of interest where significant surveying effort has been undertaken to establish benchmark parameters. |
| annual inspection report | means an assessment prepared by a suitably qualified and experienced person containing details of the assessment against the most recent consequence assessment report and design plan (or system design plan); - against recommendations contained in previous annual inspections reports; - against recognised dam safety deficiency indicators; - for changes in circumstances potentially leading to a change in consequence category; - for conformance with the conditions of this authority; - for conformance with the ‘as constructed’ drawings; - for the adequacy of the available storage in each regulated dam, based on an actual observation or observations taken after 31 May each year but prior to 1 November of that year, of accumulated sediment, state of the containment barrier and the level of liquids in the dam (or network of linked containment systems); - for evidence of conformance with the current operational plan. |
| annual return period | means the 12 month period from 1 January to 31 December in a calendar year. |
| appraisal well | means a petroleum well to test the potential of one (1) or more natural underground reservoirs for producing or storing petroleum. For clarity, an appraisal well does not include an exploration well. |
| appropriately qualified person / suitably qualified person | means a person who has professional qualifications, training or skills or experience relevant to the nominated subject matters and can give authoritative assessment, advice and analysis about performance relevant to the subject matters using relevant protocols, standards, methods or literature. |
| approved quality criteria | for the purposes of residual drilling materials, means the residual drilling material meets the following quality standards: Part A In all cases: Parameter; Maximum concentration. pH; 6–10.5 (range). Electrical Conductivity; 20dS/m (20,000μS/cm). Chloride*; 8000mg/L. *Chloride analysis is only required if an additive containing chloride was used in the drilling process The limits in Part A must be measured in the clarified filtrate of oversaturated solids prior to mixing. Part B If any of the following metals are a component of the drilling fluids, then for that metal: Parameter; Maximum concentration. Arsenic; 20mg/kg. Selenium; 5mg/kg. Boron; 100mg/kg. Cadmium; 3mg/kg. Chromium (total); 400mg/kg. Copper; 100mg/kg. Lead; 600mg/kg. The limits in Part B and Part C refer to the post soil/by-product mix. Part C If a hydrocarbon sheen is visible, the following hydrocarbon fractions: TPH; Maximum concentration. C6-C10; 170mg/kg. C10-C16; 150mg/kg. C16-C34; 1300mg/kg. C34-C40; 5600mg/kg. Total Polycyclic Aromatic Hydrocarbons (PAHs); 20mg/kg. Phenols (halogenated); 1mg/kg. Phenols (non-halogenated); 60mg/kg. Monocyclic aromatic hydrocarbons (Total sum of benzene, toluene, ethyl benzene, xylenes (includes ortho, para and meta xylenes) and styrene); 7mg/kg. Benzene; 1mg/kg. |
| areas of pre-existing disturbance | means areas where environmental values have been negatively impacted as a result of anthropogenic activity and these impacts are still evident. Areas of pre-disturbance may include areas where legal clearing, logging, timber harvesting, or grazing activities have previously occurred, where high densities of weed or pest species are present which have inhibited re-colonisation of native regrowth, or where there is existing infrastructure (regardless of whether the infrastructure is associated with the authorised petroleum activities). The term ‘areas of pre-disturbance’ does not include areas that have been impacted by wildfire/s, controlled burning, flood or natural vegetation die-back. |
| assessed or assessment | by a suitably qualified and experienced person in relation to a consequence assessment of a dam, means that a statutory declaration has been made by that person and, when taken together with any attached or appended documents referenced in that declaration, all of the following aspects are addressed and are sufficient to allow an independent audit of the assessment: - exactly what has been assessed and the precise nature of that determination; - the relevant legislative, regulatory and technical criteria on which the assessment has been based; - the relevant data and facts on which the assessment has been based, the source of that material, and the efforts made to obtain all relevant data and facts; and - the reasoning on which the assessment has been based using the relevant data and facts, and the relevant criteria. |
| Australian Standard 3580 | means any of the following publications or their subsequent replacements: - AS3580.10.1 Methods for sampling and analysis of ambient air—Determination of particulate matter—Deposited matter—Gravimetric method. - AS3580.9.6 Methods for sampling and analysis of ambient air—Determination of suspended particulate matter—PM10 high volume sampler with size-selective inlet—Gravimetric method - AS3580.9.9 Methods for sampling and analysis of ambient air—Determination of suspended particulate matter— PM10 low volume sampler—Gravimetric sampler. |
| background noise level | means the sound pressure level, measured in the absence of the noise under investigation, as the LA90,T being the A-weighted sound pressure level exceeded for 90% of the measurement time period T of not less than 15 minutes (or LA 90, adj, 15 mins), using Fast response. |
| being or intended to be utilised by the landholder or overlapping tenure holder | for significantly disturbed land, means there is a written agreement (e.g. land and compensation agreement) between the landholder or the overlapping tenure holder and the holder of the environmental authority identifying that the landholder or the overlapping tenure holder has a preferred use of the land such that rehabilitation standards for revegetation by the holder of the environmental authority are not required. For dams, means there is a written agreement (e.g. land and compensation agreement) between the landholder or the overlapping tenure holder and the holder of the environmental authority identifying that the landholder or the overlapping tenure holder has a preferred use for the dam such that rehabilitation standards for revegetation by the holder of the environmental authority are not required. |
| BTEX | means benzene, toluene, ethylbenzene, ortho-xylene, para-xylene, meta-xylene and total xylene. |
| Category A Environmentally Sensitive Area | has the meaning in the Environmental Protection Regulation 2019. |
| Category B Environmentally Sensitive Area | has the meaning in the Environmental Protection Regulation 2019. |
| Category C Environmentally Sensitive Area | means any of the following areas: - nature refuges as defined in the conservation agreement for that refuge under the Nature Conservation Act 1992 - koala habitat areas as defined under the Nature Conservation (Koala) Conservation Plan 2006 - state forests or timber reserves as defined under the Forestry Act 1959 - regional parks (previously known as resource reserves) under the Nature Conservation Act 1992 - an area validated as ‘essential habitat’ from ground-truthing surveys in accordance with the Vegetation Management Act 1999 for a species of wildlife listed as endangered or vulnerable under the Nature Conservation Act 1992 - an area validated from ground-truthing surveys as ‘protected wildlife habitat’ that is category A, B or C on the remnant vegetation management map, in accordance with section 20A of the Vegetation Management Act 1992, for a species of wildlife listed as critically endangered, endangered or vulnerable under the Nature Conservation Act 1992 - ‘of concern regional ecosystems’ that are remnant vegetation and identified in the database called ‘RE description database’ containing regional ecosystem numbers and descriptions. |
| certified or certification | in relation to any matter other than a design plan, ‘as constructed’ drawings or an annual report regarding dams means, a Statutory Declaration by a suitably qualified person or suitably qualified third party accompanying the written document stating: - the person’s qualifications and experience relevant to the function - that the person has not knowingly included false, misleading or incomplete information in the document - that the person has not knowingly failed to reveal any relevant information or document to the administering authority - that the document addresses the relevant matters for the function and is factually correct; and - that the opinions expressed in the document are honestly and reasonably held. |
| certification | in relation to regulated dam conditions means assessment and approval must be undertaken by a suitably qualified and experienced person in relation to any assessment or documentation required by this Manual, including design plans, ‘as constructed’ drawings and specifications, construction, operation or an annual report regarding regulated structures, undertaken in accordance with the Board of Professional Engineers of Queensland Policy Certification by RPEQs (ID: 1.4 (2A)). |
| clearing | has the meaning in the dictionary of the Vegetation Management Act 1999 and for vegetation— - means remove, cut down, ringbark, push over, poison or destroy in any way including by burning, flooding or draining; but - does not include destroying standing vegetation by stock, or lopping a tree. |
| closed-loop systems | means using waste on site in a way that does not release waste or contaminants in the waste to the environment. |
| consequence | in relation to a structure as defined, means the potential for environmental harm resulting from the collapse or failure of the structure to perform its primary purpose of containing, diverting or controlling flowable substances. |
| consequence category | means a category, either low, significant or high, into which a dam is assessed as a result of the application of tables and other criteria in the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933). |
| control measure | has the meaning in section 32 of the Environmental Protection Regulation 2019 and means a device, equipment, structure, or management strategy used to prevent or control the release of a contaminant or waste to the environment. |
| coal seam gas water | means underground water brought to the surface of the earth, or moved underground in connection with exploring for, or producing coal seam gas. |
| dam(s) | means a land-based structure or a void that contains, diverts or controls flowable substances, and includes any substances that are thereby contained, diverted or controlled by that land-based structure or void and associated works. |
| daily peak design capacity | for sewage treatment works, has the meaning in Schedule 2, section 63(4) of the Environmental Protection Regulation 2019 as the higher equivalent person (EP) for the works calculated using each of the formulae found in the definition for EP. |
| design storage allowance or DSA | means an available volume, estimated in accordance with the Manual for Assessing Consequence Categories and Hydraulic Performance of Structures (ESR/2016/1933), published by the administering authority, as amended from time to time, that must be provided in a dam to an annual exceedance probability specified in that Manual. |
| development wells | means a petroleum well which produces or stores petroleum. For clarity, a development well does not include an appraisal well. |
| document | has the meaning in the Acts Interpretation Act 1954 and means: - any paper or other material on which there is writing; and - any paper or other material on which there are marks; and - figures, symbols or perforations having a meaning for a person qualified to interpret them; and - any disc, tape or other article or any material from which sounds, images, writings or messages are capable of being produced or reproduced (with or without the aid of another article or device). |
| enclosed flare | means a device where the residual gas is burned in a cylindrical or rectilinear enclosure that includes a burning system and a damper where air for the combustion reaction is admitted. |
| environmental harm | has the meaning in section 14 of the Environmental Protection Act 1994 and means any adverse effect, or potential adverse effect (whether temporary or permanent and of whatever magnitude, duration or frequency) on an environmental value, and includes environmental nuisance. Environmental harm may be caused by an activity— - whether the harm is a direct or indirect result of the activity; or - whether the harm results from the activity alone or from the combined effects of the activity and other activities or factors. |
| environmental nuisance | has the meaning in section 15 of the Environmental Protection Act 1994 and means unreasonable interference or likely interference with an environmental value caused by— - aerosols, fumes, light, noise, odour, particles or smoke; or - an unhealthy, offensive or unsightly condition because of contamination; or - another way prescribed by regulation. |
| environmentally sensitive area | means Category A, B or C environmentally sensitive areas (ESAs) |
| equivalent person or EP | has the meaning under section 3 of the Planning Guidelines for Water Supply and Sewerage, 2005, published by the Queensland Government. It is calculated in accordance with Schedule 2, Section 63 of the Environmental Protection Regulation 2019 where: - EP = V/200 where V is the volume, in litres, of the average dry weather flow of sewage that can be treated at the works in a day; or - EP = M/2.5 where M is the mass, in grams, of phosphorus in the influent that the works are designed to treat as the inlet load in a day. |
| essential petroleum activities | means activities that are essential to bringing the resource to the surface and are only the following: - low impact petroleum activities - geophysical, geotechnical, geological, topographic and cadastral surveys (including seismic, sample /test / geotechnical pits, core holes) - single well sites not exceeding 1 hectare disturbance and multi-well sites not exceeding 1.5 hectare disturbance - well sites with monitoring equipment (including monitoring bores): - for single well sites, not exceeding 1.25 hectares disturbance - for multi-well sites, not exceeding 1.75 hectares disturbance - well sites with monitoring equipment (including monitoring bores) and tanks (minimum 1 ML) for above ground fluid storage: - for single well sites, not exceeding 1.5 hectares disturbance - for multi-well sites, not exceeding 2.0 hectares disturbance - well sites with slope considerations (>2% slope) for cut and fill earthworks and drainage: - for single well sites, not exceeding 1.5 hectares disturbance - for multi-well sites, not exceeding 2.5 hectares disturbance - well sites including a communications tower: - for single well sites, not exceeding 1.5 hectares disturbance - for multi-well sites, not exceeding 3.0 hectares disturbance - on site disposal of residual drilling material as per condition (Waste 16) - communications towers, not exceeding 1.0 hectares disturbance. - associated infrastructure located on a well site necessary for the construction and operations of wells: - water pumps and generators - flares - flare pits - chemical / fuel storages - sumps for residual drilling material and drilling fluids - tanks, or dams which are not significant or high consequence dams to contain wastewater (e.g. stimulation flow back waters, produced water) - pipe laydown areas - soil and vegetation stockpile areas - a temporary camp associated with a drilling rig that may involve sewage treatment works that are no release works - temporary administration sites and warehouses - dust suppression activities using water that meets the quality and operational standards approved under the environmental authority - communication and power lines that are necessary for the undertaking of petroleum activities and that are located within well sites, well pads and pipeline right of ways without increasing the disturbance area of petroleum activities - supporting access tracks - gathering / flow pipelines from a well head to, or between, any one or more of the following: - initial compression facility - water storage facility - the initial compression facility and/or to, or between, - water storage facilities - activities necessary to achieve compliance with the conditions of the environmental authority in relation to another essential petroleum activity (e.g. sediment and erosion control measures, and rehabilitation). |
| exploration well | means a petroleum well that is drilled to: - explore for the presence of petroleum or natural underground reservoirs suitable for storing petroleum; or - obtain stratigraphic information for the purpose of exploring for petroleum. For clarity, an exploration well does not include an appraisal or development well. |
| flare pit | has the meaning in the Manual for Assessing Consequence Categories and Hydraulic Performance of Structures (EM635) and means containment area where any hydrocarbon that is discovered in an over-pressured reservoir during a drilling operation is diverted to, and combusted, The flare pit is only used during the drilling and work over process on a petroleum well. |
| flare precipitant | means waste fluids which result from the operation of a flare. |
| flowable substance | means matter or a mixture of materials which can flow under any conditions potentially affecting that substance. Constituents of a flowable substance can include water, other liquids fluids or solids, or a mixture that includes water and any other liquids fluids or solids either in solution or suspension. |
| GDA | means Geocentric Datum of Australia. |
| Great Artesian Basin (GAB) spring | means an area protected under the Environment Protection and Biodiversity Conservation Act 1999 because it is considered to be a Matter of National Environmental Significance and identified as a: - community of native species dependent on natural discharge of groundwater from the Great Artesian Basin; or - Great Artesian Basin spring; or - Great Artesian Basin discharge spring wetland. A GAB spring includes a spring vent, spring complex or watercourse spring and includes the land to which water rises naturally from below the ground and the land over which the water then flows. Note: The Australian Government’s Protected Matters Search Tool should be used to get an indication of whether the area of interest may contain an MNES spring. Note: The GAB springs dataset can be requested from the Queensland Government Herbarium |
| green waste | means waste that is grass cuttings, trees, bushes, shrubs, material lopped from trees, untreated timber or other waste that is similar in nature but does not include prohibited matter, restricted matter species or weeds of national significance. |
| Groundwater dependent ecosystem (GDE) | means ecosystems which require access to groundwater on a permanent or intermittent basis to meet all or some of their water requirements so as to maintain their communities of plants and animals, ecological processes and ecosystem services. For the purposes of the environmental authority, groundwater dependent ecosystems do not include those mapped as “unknown”. |
| growing | means to increase by natural development, as any living organism or part thereof by assimilation of nutriment; increase in size or substance. |
| holder | means: - where this document is an environmental authority, any person who is the holder of, or is acting under, that environmental authority; or - where this document is a development approval, any person who is the registered operator for that development approval. |
| hydraulic integrity | refers to the capacity of a dam to contain or safely pass flowable substances based on its design. |
| impulsive (for noise) | means sound characterised by brief excursions of sound pressure (acoustic impulses) that significantly exceed the background sound pressure. The duration of a single impulsive sound is usually less than one second. |
| incidental activity | for this environmental authority means an activity that is not a specified relevant activity and is necessary to carry out the activities listed in General, Table 1 – Authorised Petroleum Activities. |
| Independent suitably qualified expert | means a person with: - at least a post graduate degree (or equivalent) in a suitable are such as hydrology or hydrogeology; - a minimum of 10 years relevant experience in water resource assessment; and - who is independent of the appropriately qualified hydrogeologist in condition Water 2 and the holder. |
| LA 90, adj, 15 mins | means the A-weighted sound pressure level, adjusted for tonal character that is equal to or exceeded for 90% of any 15 minutes sample period equal, using Fast response. |
| LAeq, adj, 15 mins | means the A-weighted sound pressure level of a continuous steady sound, adjusted for tonal character, that within any 15 minute period has the same square sound pressure as a sound level that varies with time. |
| linear infrastructure | means powerlines, pipelines, flowlines, roads and access tracks. |
| liquid | means a substance which is flowing and offers no permanent resistance to changes of shape. |
| long term noise event | means a noise exposure, when perceived at a sensitive receptor, persists for a period of greater than five (5) days, even when there are respite periods when the noise is inaudible within those five (5) days. |
| low consequence dam | means any dam that is not classified as high or significant as assessed using the Manual for Assessing Consequence Categories and Hydraulic Performance of Structures, published by the administering authority, as amended from time to time. |
| low impact petroleum activities | means petroleum activities which do not result in the clearing of native vegetation, cause disruption to soil profiles through earthworks or excavation or result in significant disturbance to land which cannot be rehabilitated immediately using hand tools after the activity is completed. Examples of such activities include but are not necessarily limited to soil surveys (excluding test pits), topographic surveys, cadastral surveys and ecological surveys, may include installation of monitoring equipment provided that it is within the meaning of low impact and traversing land by car or foot via existing access tracks or routes or in such a way that does not result in permanent damage to vegetation. |
| Mandatory reporting level or MRL | means a warning and reporting level determined in accordance with the criteria in the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933) published by the administering authority. |
| Max LpA, 15 min | means the absolute maximum instantaneous A-weighted sound pressure level, measured over 15 minutes. |
| Max LpZ, 15 min | means the maximum value of the Z-weighted sound pressure level measured over 15 minutes. |
| maximum extent of impact | means the total, cumulative, residual extent and duration of impact to a prescribed environmental matter that will occur over a project’s life after all reasonable avoidance and reasonable on-site mitigation measures have been, or will be, undertaken. |
| medium term noise event | is a noise exposure, when perceived at a sensitive receptor, persists for an aggregate period not greater than five (5) days and does not re-occur for a period of at least four (4) weeks. Re-occurrence is deemed to apply where a noise of comparable level is observed at the same receptor location for a period of one hour or more, even if it originates from a difference source or source location. |
| methodology | means the science of method, especially dealing with the logical principles underlying the organisation of the various special sciences, and the conduct of scientific inquiry. |
| month | has the meaning in the Acts Interpretation Act 1954 and means a calendar month and is a period starting at the beginning of any day of one (1) of the 12 named months and ending— - immediately before the beginning of the corresponding day of the next named month; or - if there is no such corresponding day—at the end of the next named month. |
| NATA accreditation | means accreditation by the National Association of Testing Authorities Australia. |
| prescribed environmental matters | has the meaning in section 10 of the Environmental Offsets Act 2014, limited to the matters of State environmental significant listed in schedule 2 of the Environmental Offsets Regulation 2014. |
| pre-disturbed land use | means the function or use of the land as documented prior to significant disturbance occurring at that location. |
| prescribed water contaminants | has the meaning in section 440ZD of the Environmental Protection Act 1994 and means: - earth (meaning sand, soil, silt or mud); or - a contaminant prescribed under section 440ZF of the Environmental Protection Act 1994. |
| primary protection zone | means an area within 200m from the boundary of any Category A, B or C ESA. |
| produced water | has the meaning in Section 15A of the Petroleum and Gas (Production and Safety) Act 2004 and means CSG water or associated water for a petroleum tenure. |
| prohibited matter | has the meaning in section 19 of the Biosecurity Act 2014. |
| protection zone | means the primary protection zone of any Category A, B or C ESA or the secondary protection zone of any Category A or B ESA. |
| Quantity of Interest | The Quantity of Interest is the Arrow Energy incremental impact in the simulated particle track output of the groundwater model required by condition Water 2. |
| restricted matter | has the meaning in section 21 of the Biosecurity Act 2014. |
| regional ecosystem | has the meaning in the Methodology for Surveying and Mapping of Regional Ecosystems and Vegetation Communities in Queensland (Version 7, December 2023) and means a vegetation community in a bioregion that is consistently associated with a particular combination of geology, landform and soil. Regional ecosystems of Queensland were originally described in Sattler and Williams (1999). The Regional Ecosystem Description Database (Queensland Herbarium 2013) is maintained by Queensland Herbarium and contains the current descriptions of regional ecosystems. |
| Register of regulated structures | includes: - Date of entry in the register; - Name of the structure, its purpose and intended/actual contents; - The consequence category of the dam as assessed using the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933); - Dates, names, and reference for the design plan plus dates, names, and reference numbers of all document(s) lodged as part of a design plan for the dam; - Name and qualifications of the suitably qualified and experienced person who certified the design plan and 'as constructed' drawings; - For the regulated dam, other than in relation to any levees – - The dimensions (metres) and surface area (hectares) of the dam measured at the footprint of the dam; - Coordinates (latitude and longitude in GDA94) within five metres at any point from the outside of the dam including its storage area - Dam crest volume (megalitres); - Spillway crest level (metres AHD). - Maximum operating level (metres AHD); - Storage rating table of stored volume versus level (metres AHD); - Design storage allowance (megalitres) and associated level of the dam (metres AHD); - Mandatory reporting level (metres AHD); - The design plan title and reference relevant to the dam; - The date construction was certified as compliant with the design plan; - The name and details of the suitably qualified and experienced person who certified that the constructed dam was compliant with the design plan; - Details of the composition and construction of any liner; - The system for the detection of any leakage through the floor and sides of the dam; - Dates when the regulated dam underwent an annual inspection for structural and operational adequacy, and to ascertain the available storage volume for 1 November of any year; - Dates when recommendations and actions arising from the annual inspection were provided to the administering authority; - Dam water quality as obtained from any monitoring required under this authority as at 1 November of each year. |
| regulated dam | means any dam in the significant or high consequence category as assessed using the Manual for Assessing Consequence Categories and Hydraulic Performance of Structures (EM635), published by the administering authority, as amended from time to time. |
| regulated structure | means any structure in the significant or high consequence category as assessed using the Manual for assessing consequence categories and hydraulic performance of structures (ESR/2016/1933) published by the administering authority. A regulated structure does not include: - a fabricated or manufactured tank or container, designed and constructed to an Australian Standard that deals with strength and structural integrity of that tank or container; - a sump or earthen pit used to store residual drilling material and drilling fluid only for the duration of drilling and well completion activities - a flare pit. |
| rehabilitation or rehabilitated | means the process of reshaping and revegetating land to restore it to a stable landform and in accordance with acceptance criteria and, where relevant, includes remediation of contaminated land. For the purposes of pipeline rehabilitation, rehabilitation includes reinstatement, revegetation and restoration. |
| reinstate or reinstatement | for steel pipelines, means the process of bulk earth works and structural replacement of pre-existing conditions of a site (i.e. soil surface typography, watercourses, culverts, fences and gates and other landscape(d) features) and is detailed in the Australian Pipeline Industry Association (APIA) Code of Environmental Practice: Onshore Pipelines (2013). |
| reporting limit | means the lowest concentration that can be reliably measured within specified limits of precision and accuracy during routine laboratory operating conditions. For many analytes, the reporting limit is selected as the lowest non-zero standard in the calibration curve. Results that fall below the reporting limit will be reported as “less than” the value of the reporting limit. The reporting limit is also referred to as the practical quantitation limit or the limit of quantitation. For polycyclic aromatic hydrocarbons, the reporting limit must be based on super-ultra trace methods and, depending on the specific polycyclic aromatic hydrocarbon, will range between 0.005 ug/L–0.02 ug/L. |
| residual drilling material | means waste drilling materials including muds and cuttings or cement returns from well holes and which have been left behind after the drilling fluids are pumped out. |
| restoration | means the replacement of structural habitat complexity, ecosystem processes, services and function from a disturbed or degraded site to that of a pre-determined or analogue site. For the purposes of pipelines, restoration applies to final rehabilitation after pipeline decommissioning. |
| revegetation or revegetating or revegetate | means to actively re-establish vegetation through seeding or planting techniques in accordance with site specific management plans. |
| sensitive place | means: - a dwelling (including residential allotment, mobile home or caravan park, residential marina or other residential premises, motel, hotel or hostel) - a library, childcare centre, kindergarten, school, university or other educational institution - a medical centre, surgery or hospital - a protected area - a public park or garden that is open to the public (whether or not on payment of money) for use other than for sport or organised entertainment - a work place used as an office or for business or commercial purposes, which is not part of the petroleum activity(ies) and does not include employees accommodation or public roads - for noise, a place defined as a sensitive receptor for the purposes of the Environmental Protection (Noise) Policy 2019. |
| sensitive receptor | is defined in Schedule 2 of the Environmental Protection (Noise) Policy 2019, and means an area or place where noise is measured. |
| short term noise event | is a noise exposure, when perceived at a sensitive receptor, persists for an aggregate period not greater than eight hours and does not re-occur for a period of at least seven (7) days. Re-occurrence is deemed to apply where a noise of comparable level is observed at the same receptor location for a period of one hour or more, even if it originates from a different source or source location. |
| significant residual impact | has the meaning in section 8 Environmental Offsets Act 2014. |
| significantly disturbed or significant disturbance or significant disturbance to land or areas | means Land is significantly disturbed if– - it is contaminated land; or - it has been disturbed and human intervention is needed to rehabilitate it– - to a condition required under the relevant environmental authority; or - if the environmental authority does not require the land to be rehabilitated to a particular condition–to the condition it was in immediately before the disturbance. Without limiting subsection (1)(b), land requires human intervention to rehabilitate it if– - the disturbance has made the land more susceptible to erosion; or - the land use capability or suitability of the land is diminished; or - the quality of water in a watercourse downstream of the land has been significantly reduced. |
| species richness | means the number of different species in a given area. |
| stable | means the rehabilitation and restoration of the site is enduring or permanent so that the site is unlikely to collapse, erode or subside. |
| Stage 1 activities | includes up to an additional 55 coal seam gas wells approved and with the required statutory authorisations to address the actions required for this environmental authority to take effect. |
| statement of compliance | for a condition in an environmental authority has the meaning in section 208 of the Environmental Protection Act 1994 and is a condition that requires the holder to give the administering authority a statement of compliance about a document or work relating to a relevant activity. The condition must also state— - the criteria (the compliance criteria) the document or work must comply with; and - that the statement of compliance must state whether the document or work complies with the compliance criteria; and - the information (the supporting information) that must be provided to the administering authority to demonstrate compliance with the compliance criteria; and - when the statement of compliance and supporting information must be given to the administering authority. |
| stimulation | means a technique used to increase the permeability of natural underground reservoir that is undertaken above the formation pressure and involves the addition of chemicals. It includes hydraulic fracturing / hydrofraccing, fracture acidizing and the use of proppant treatments. Explanatory note: This definition is restricted from that in the Petroleum and Gas (Production and Safety) Act 2004 in order to only capture the types of stimulation activities that pose a risk to environmental values of water quality in aquifers. |
| structure | means a dam or levee. |
| subterranean cave GDE | - means an area identified as a subterranean cave in the mapping produced by the Queensland Government and identified in the Queensland Government Information System, as amended from time to time; and - means a cave ecosystem which requires access to groundwater on a permanent or intermittent basis to meet all or some of their water requirements so as to maintain its communities of plants and animals, ecological processes and ecosystem services. Subterranean cave GDEs are caves dependent on the subterranean presence of groundwater. Subterranean cave GDEs have some degree of groundwater connectivity and are indicated by either high moisture levels or the presence of stygofauna, or both, referred to in the Queensland Government WetlandsInfo mapping program, as amended from time to time. Note: the Subterranean GDE (caves) dataset can be displayed through the Queensland Government WetlandInfo mapping program. Note: the Subterranean GDE (caves) dataset can be obtained from the Queensland Government Information System. |
| suitably qualified and experienced person | in relation to regulated structures means a person who is a Registered Professional Engineer of Queensland (RPEQ) under the provisions of the Professional Engineers Act 2002, and has demonstrated competency and relevant experience: - for regulated dams, an RPEQ who is a civil engineer with the required qualifications in dam safety and dam design - for regulated levees, an RPEQ who is a civil engineer with the required qualifications in the design of flood protection embankments. Note: It is permissible that a suitably qualified and experienced person obtain subsidiary certification from an RPEQ who has demonstrated competence and relevant experience in either geomechanics, hydraulic design or engineering hydrology. |
| suitably qualified person | means a person who has professional qualifications, training or skills or experience relevant to the nominated subject matters and can give authoritative assessment, advice and analysis about performance relevant to the subject matters using relevant protocols, standards, methods or literature. |
| suitably qualified third party | means a person who: - has qualifications and experience relevant to performing the function including but not limited to: - a bachelor’s degree in science or engineering; and - 3 years’ experience in undertaking soil contamination assessments; and - is a member of at least one organisation prescribed in Schedule 14 of the Environmental Protection Regulation 2019; and - not be an employee of, nor have a financial interest or any involvement which would lead to a conflict of interest with the holder(s) of the environmental authority. |
| sump | means a pit in which waste residual drilling material or drilling fluids are stored only for the duration of drilling activities. |
| top soil | means the surface (top) layer of a soil profile, which is more fertile, darker in colour, better structured and supports greater biological activity than underlying layers. The surface layer may vary in depth depending on soil forming factors, including parent material, location and slope, but generally is not greater than about 300mm in depth from the natural surface. |
| valid complaint | means all complaints unless considered by the administering authority to be frivolous, vexatious or based on mistaken belief. |
| waste and resource management hierarchy | has the meaning provided in section 9 of the Waste Reduction and Recycling Act 2011 and is the following precepts, listed in the preferred order in which waste and resource management options should be considered— - AVOID unnecessary resource consumption - REDUCE waste generation and disposal - RE-USE waste resources without further manufacturing - RECYCLE waste resources to make the same or different products - RECOVER waste resources, including the recovery of energy - TREAT waste before disposal, including reducing the hazardous nature of waste - DISPOSE of waste only if there is no viable alternative. |
| waste and resource management principles | has the meaning provided in section 4(2)(b) of the Waste Reduction and Recycling Act 2011 and means the: - polluter pays principle - user pays principle - proximity principle - product stewardship principle. |
| waste fluids | has the meaning in section 13 of the Environmental Protection Act 1994 in conjunction with the common meaning of “fluid” which is “a substance which is capable of flowing and offers no permanent resistance to changes of shape”. Accordingly, to be a waste fluid, the waste must be a substance which is capable of flowing and offers no permanent resistance to changes of shape. |
| watercourse | has the meaning in Schedule 4 of the Environmental Protection Act 1994 and means: - a river, creek or stream in which water flows permanently or intermittently— - in a natural channel, whether artificially improved or not; or - in an artificial channel that has changed the course of the watercourse. - Watercourse includes the bed and banks and any other element of a river, creek or stream confining or containing water. |
| waters | includes all or any part of a creek, river, stream, lake, lagoon, swamp, wetland, spring, unconfined surface water, unconfined water in natural or artificial watercourses, bed and bank of any waters, non-tidal or tidal waters (including the sea), stormwater channel, stormwater drain, roadside gutter, stormwater run-off, and underground water. |
| weeds of national significance | includes the weeds declared in Appendix B of the Australian Weeds Strategy 2017–2027. |
| well integrity | the ability of a well to contain the substances flowing through it. |
| wetland | for the purpose of this environmental authority, wetland means an area shown as a wetland on the map of Queensland Wetland Environmental Values. Note: The Environmental Protection (Water and Wetland Biodiversity) Policy 2019 Schedule 2, Map of Queensland Wetland Environmental Values means the document ‘Map of Queensland Wetland Environmental Values’ made by the Chief Executive and published on the website. Environmental values in section 8 of the Environmental Protection (Water and Wetland Biodiversity) Policy 2019 apply to wetland areas on the map, which are categorised as wetlands of high or general ecological significance. |
| wetland of high ecological significance | means a wetland that meets the definition of a wetland and that is shown as a wetland of ‘high ecological significance’ or wetland of ‘high ecological value’ on the Map of referable wetlands. |
| wetland of other environmental value | means a wetland that meets the definition of a wetland and that is shown as a wetland of ‘general environmental significance’ or wetland of ‘other environmental value’ on the Map of referable wetlands. |

END OF ENVIRONMENTAL AUTHORITY